Code No.321052

OR

III B.Tech. II-Semester Supplementary Examinations, November-2003

COMPUTER ORGANISATION

(Electronics and Instrumentation Engineering)

Time: 3 hours							Max.Marks:70

Answer any FIVE questions

All questions carry equal marks

---

1.	Describe any Four interconnection structures for multi processors?

2.a)	A two-way set associative Cache memory uses blocks of 4 words. The Cache can 	accomdiate 2 K words of main memory. The main memory size is 128 k  32. 	Give the size of the Cache memory.

b)	Memory access time is 1000 ns and access time of Cache memory is 100 ns. What 	should be the hit rotion to maintain an average access time of 350 ns?

c)	Give the organization of one cell of associative memory.

3.a)	Represent the following decimal numbers  (i) 145	(ii) 65	(iii) 92, into grey 	codes and  BCD-2421  code format.

b)	Represent the following decimal numbers into SMF and 1C form. Where the 	representation takes 10 bits out of which MSbit is used to represent the sign?

(i) -15		(ii) 249		(iii) -120

c)	Represent the following decimal numbers into Floating point binary format with 	48 bits where 12 bits represents the exponent?

(i) -289.5	(ii) 352.75

4.a)	Describe the architecture of a Floating Point pipe line adder.

b)	Describe various types of parallel processors organizations.

5.a)	Describe programmed I/O organization.

b)	Describe the DMA organization.

6.a)	Describe any Four addressing modes.

b)	A computer with main memory of 512 k  64 bits with 129 CPU registers and 	with 64 addressing modes. The computer has 500 instructions. In one of the 	instruction format the First Field represents operator field, Second Field is Mode, 	the Third Field represents an indirect address and Fourth Field represents one of 	the CPU registers. Give the instruction format indicating Number of bits for each 	Field.

7.	Describe various instruction Formats and represent the following statement using 	Three, Two, One and Zero address instruction formats.

K = (l+m)  *  n-d/k

8.	Write short notes on the following:   	(a) Nano programming

(b) Software development aids (c) Compare Assembly and high level language.

***     ***     ***