Code No.321702

Set No.

1

III B.Tech. II-Semester Supplementary Examinations, November-2003

ELECTRONICS MEASUREMENTS AND INSTRUMENTATION

(Electronics and Telematics)

Time: 3 hours							Max.Marks: 80

Answer any FIVE questions

All questions carry equal marks

---

1.a)	Explain the circuit diagram of following type of electronic voltmeter (i) voltmeter 	using a series connected diode  (ii) voltmeter using a full bridge rectifier

(iii) peak reading voltmeter using shunt connected diode.

Explain the causes of error if the input voltage is non-sinusoidal.

b)	An analog indicating instrument with a scale range of 0-2.5V shows a voltage of 	1.46 V where as the true value is 1.50V. What are the values of absolute error and 	correction. Express the error as fraction of the true value and the full scale 	deflection.

2.	Explain the functioning of successive approximation type and potentiometric type 	of digital voltmeters.

3.a)	Derive the equations of balance for an Anderson’s bridge. Draw the phasor 	diagram for condition under balance. Discuss the advantages and disadvantages of 	the bridge.

b)	Derive the general equations for balance of an a.c. bridge. What are the different 	sources of errors in a.c. bridges? Explain the precaution taken and the techniques 	used for eliminating or minimizing the errors.

4.a)	Draw the block diagram of a general purpose CRO and explain the function of the 	following control  (i) intensity	(ii) focus	(iii) horizontal and vertical 	positioning 	(iv) synchronization.

b)	Explain the function of a time base generator in a CRO.

5.	Describe with a neat sketch the principle and construction details of resistance 	thermometer and thermistor. Draw its characteristics curve and state its merits 	and demerits.

6.a)	Describe the measurement of the following using a Q meter  (i) Q factor            (ii) inductance (iii) effective resistance	(iv) self capacitance	(v) bandwidth.

b)	A circuit consisting of a coil, a resistance and a variable capacitor connected in 	series is tunned to resonance using a Q meter. If the frequency is 500 KHz, the 	resistance 0.5 Ω and the variable capacitor set to 350 PF. Calculate the effective 	inductance and resistance of the coil, if the Q meter indicates 90.

(Contd…2)

Code No.321702				-2-			Set No.1

7.a)	Derive an expression for vertical deflection of an electron beam in a CRT.

b)	Explain different type of graticules used in a CRO. Describe their advantage and 	disadvantage.

8.	Explain in detail the construction and working of a spectrum analyzer. State its 	application.

***     ***     ***

Code No.321702

Set No.

2

III B.Tech. II-Semester Supplementary Examinations, November-2003

ELECTRONICS MEASUREMENTS AND INSTRUMENTATION

(Electronics and Telematics)

Time: 3 hours							Max.Marks: 80

Answer any FIVE questions

All questions carry equal marks

---

1.a)	Explain the working of electronic voltmeter which use rectifiers. Explain the d.c 	and a.c. modes of operation and describe why negative feed back is used in them.

b)	A 10,000 Ω variable resistance has a linearity of 0.1% and the movement of 	constant arm is 320o  (i) determine the maximum position deviation in degrees and 	the resistance deviation in ohm.   (ii) if this instrument is to be used as a 	potentiometer with a linear scale of 0 to 1.6 V determine the maximum voltage 	error.

2.a)	Explain in detail about continuous balance type digital voltmeter.

b)	Describe with the help of suitable block diagram how the following type of 	measurements are carried out time interval and low frequency measurement.

3.a)	Describe the circuit and working of a Q meter. Describe its application.

b)	Describe how corrections for shunt resistance and distributed capacitance are 	applied when measuring Q factor of a coil with a Q meter.

4.	Describe the construction and working of the following types of accelerometers 	and state its advantage and disadvantage (a) LVDT type   (b) Potentiometer type.

5.a)	How much voltage is required across two deflection plates separated by 1 cm, to 	deflect an electron beam 1o if the effective length of the deflection plate is 2 cm 	and the accelerating potential is 1000 V.

b)	What is oscilloscope probe compensation? How is this adjusted? What effect are 	noted when the compensation is not correctly adjusted? What are the advantage of 	using an active voltage probe?

c)	State the kind of application where an x-y recorder is more suitable than the 	common form of single pen recorder.

6.a)	With suitable diagram, explain a method for force measurement.

b)	Explain the principle of capacitive transducer and show how it can be used for 	displacement measurement.

c)	Under what condition a dummy strain gauge used and what is the function of that 	gauge.

(Contd…2)

Code No.321702				-2-			Set No.2

7.a)	Describe the basic components of a magnetic tape recorder. State its advantage 	and disadvantage over other type of recording system.

b)	What is x-y recorder? How do you distinguish it from x-t or y-t recorder? 	Describe the construction and working of x-y recorder.

8.a)	The four arms of a Maxwell’s capacitance bridge at balance are: arm ab, an unknown inductance L1, having an inherent resistance R1; arm bc, a                 non-inductive resistance of 1000 Ω; arm cd, a capacitor of 0.5F in parallel with a resistance of 1000 Ω; arm da, a resistance of 1000 Ω. Derive the equations of balance for the bridge and determine the value of R1 and L1.

b)	A four arm a.c bridge a, b, c, d has the following impedance:

Arm  ab: Z1 = 200∠60oΩ  inductive impedence

Arm  ad: Z2= 400∠−60oΩ  capacitive

Arm  bc: Z3= 300∠0oΩ     resistive

Arm  cd: Z4= 600∠30oΩ   inductive

Determine whether it is possible to balance the bridge under above condition.

***     ***     ***

Code No.321702

Set No.

3

III B.Tech. II-Semester Supplementary Examinations, November-2003

ELECTRONICS MEASUREMENTS AND INSTRUMENTATION

(Electronics and Telematics)

Time: 3 hours							Max.Marks: 80

Answer any FIVE questions

All questions carry equal marks

---

1.a)	Describe the circuit diagram and operation of a D.C. voltmeter using a direct 	coupled amplifier. Explain its advantage and major disadvantage of a d.c. 	amplifier. Describe with the help of a circuit diagram how small values of voltage 	and current can be measured with it.

b)	Brief the advantages of electronic voltmeter over conventional type voltmeter.

2.	Describe with the help of a suitable block diagram, how the following type of 	measurements are carried out.

a)	Single and multiple period measurements.

b)	Time interval measurements.

3.	In an Anderson bridge for the measurement of inductance the arm AB consists of 	an unknown impedence with inductance L and R, a known variable resistance in 	arm BC, fixed resistance of 600 Ω each in arms CD and DA, a known variable 	resistance in arm DE and a capacitor with fixed capacitance of 1 micro farad in 	the arm CE. The a.c. supply of 100 Hz is connected across A and C, and the 	detector is connected between B and E. If the balance is obtained with a resistance 	of 400 Ω in the arm of DE and a resistance of 800 Ω in the arm BC calculate the 	value of unknown R and L. Derive the conditions for balance and draw the phasor 	diagram under balanced conditions. Discuss the advantage and disadvantages of 	the bridge.

4.a)	Describe the function of attenuators in CRO.

b)	Describe the phenomen of synchronization of vertical input signal to its sweep 	generator.

c)	Explain in detail the vertical and horizontal amplifiers used in CRO.

5.a)	Describe the methods of measurement of pressure using capacitive transducer and 	photo-electric transducer.

b)	Explain in detail the measurement of differential pressure using an inductive 	transducer.

6.	Describe the construction and working of any two vacuum gauge used for the 	measurement of low pressure.

(Contd…2)

Code No.321702				-2-			Set No.3

7.	Describe the construction and working of current transformer and potential 	transformer.

8.a)	Explain in detail the construction and working of sampling oscilloscope.

b)	Explain the principle of secondary emission.

***     ***     ***

Code No.321702

Set No.

4

III B.Tech. II-Semester Supplementary Examinations, November-2003

ELECTRONICS MEASUREMENTS AND INSTRUMENTATION

(Electronics and Telematics)

Time: 3 hours							Max.Marks: 80

Answer any FIVE questions

All questions carry equal marks

---

1.a)	Describe the circuit diagram and operation of a true rms reading voltmeter using 	thermocouple. Explain how these voltmeters are free from waveform errors.

b)	Define accuracy and precision with suitable example.

c)	Draw a difference-amplifier type of electronic voltmeter and show the placement 	of zero adjustment and calibration resistance in the circuit.

2.	Explain with help of suitable diagrams the functioning of ramp type and 	integrating type digital voltmeter.

3.a)	Describe the working of a low voltage Schering bridge. Derive the equation for 	capacitance and dissipation factor. Draw the phasor diagram of the bridge under 	condition of balance.

b)	Derive the equations for balance in the case of Maxwell’s inductance capacitance 	bridge. Draw the phasor diagram for balance condition.

4.a)	Describe in detail the construction and working of an analog type storage 	oscilloscope.

b)	How does the sampling oscilloscope increase apparent frequency response of an 	oscilloscope?

c)	What is the relationship between the period of a waveform and its frequency. 	How is an oscilloscope used to determine frequency?

5.	Explain the principle of operation of constant temperature and constant current 	type hotwire anemometer with neat sketch.

6.a)	Describe the construction and working principle of LVDT.

b)	Explain the construction of thermocouple with an application. Describe the 	thermo-electric laws.

(Contd…2)

Code No.321702				-2-			Set No.4

7.a)	Derive the equation of balance for a Kelvin double bridge.

b)	The ratio arms of the Kelvin bridge are 100 Ω each. The galvanometer has an 	internal resistance of 500 Ω and a current sensitivity of 200 mm/A. The 	unknown resistance R = 0.1002 Ω and the standard resistance is set at 0.1000 Ω. 	A DC current of 10 A is passed through the standard and the unknown from a 	2.2 U battery in series with a rheostat. The resistance of the yoke may be 	neglected. Calculate

(i)  the deflection of the galvanometer.

(ii) the resistance unbalance required to produce a galvanometer deflection of 		     1 mm.

8.a)	Describe the working and theory of an ultrasonic flow meter. List its advantage.

b)	Explain with suitable diagram a method by which velocity measurement can be 	done.

***     ***     ***