Code No.320453

OR

III B.Tech. II-Semester Supplementary Examinations, November-2003

MICROPROCESSOR AND INTERFACING

(Common to Electronics and Communication Engineering and Electronics and Control Engineering)

Time: 3 hours							Max.Marks:70

Answer any FIVE questions

All questions carry equal marks

---

1.a)	Explain the functions of the following pins of 8085.

(i)   ALE 	(ii) IOM 	(iii) HLDA 	(iv) INTA	(v) S0 and S1 and

(vi) SID and SOD.

b)	Draw the machine cycle timing diagram and explain the following intractions.

(i) RET	(ii) MOV  A, M.

2.a)	Write a subroutine for 8085 to obtain 1 milli sec. delay. The crystal frequency is 	6.144 MHz. Use this subroutine write a programme for 50 milli seconds delay 	using LOOP instruction.

b)	Write an 8085 ALP to check a block of data and count how many times a given 	value occurs in the data block.

3.a)	Design a memory that has 16k x 8 of ROM and 8k x 8 of RAM. The memory 	interfaces with an 8085 processor system bus. The ROM is to be constructed 	using 8k x 8 EPROMs. These devices have an active law chip enable and active 	law output enable. The RAM is to be constructed from 4k x 8 SRAMs that has 	two control lines. Each of the SRAM control lines is active law. The SRAM has 	common I/O. Use 1 out-of-8 	decoder plus other logic circuit to implement 	address decoding. Draw the memory 	mapping and logic circuit and explain its 	function.

b)	Distinguished between memory mapped I/O and isolated I/O. Compare their 	merits and demerits.

4.a)	With a neat block schematic diagram explain the internal architecture of 8255PPI.

b)	What is meant by double Hand-Shake data transfer? With an example explain the 	use of Hand-Shake signals in 8255PPI.

c)	Give an interfacing scheme of 8255 to 8085 in I/O mapped I/O mode draw the 	address 804 onwards and explain.

5.a)	What is a DMA data transfer? Explain how it is implemented using 	programmable DMA controller 8257 in a 8085 system.

b)	Explain the necessity of providing interrupts in a CPU. Explain the mechanism of 	interrupt system of 8085.

(Contd…2)

Code No.320453				-2-				OR

6.a)	Distinguish between synchronous and asynchronous data transfer in serial mode.

b)	Give a neat schematic of 8251 USART and describe its operation.

c)	How do you interface a CRT data terminal to an 8085 CPU bus in a 8251? 	Explain the hardware and software aspects of interface.

7.a)	Give the characteristics of a stepper motor.

b)	Show the schematic for interfacing a stepper motor to 8085 processor. Write ALP 	for 8085 to control the speed and direction of the stepper motor and explain its 	operation.

8.	Write short notes on the following:

a)	Interrupt controller 8259

b)	8212 I/O

c)	Interfacing ADC to microprocessor.

***     ***     ***