Code No: 320454

OR

III B.Tech. II-Semester Supplementary Examinations, November-2003.

DIGITAL SIGNAL PROCESSING

(Electronics and Communication Engineering)

Time: 3 hours							Max. Marks: 70

Answer any Five questions

All question carry equal marks

---

1.a)	Consider a discrete linear time invariant system described by the difference             equation:

Y(n)-(3/4)y(n-1)+(1/8)y(n-2)=x(n))+(1/3)x(n-1)

Where y(n) is the output and x(n) is the input. Assuming that the system is relaxed initially obtain the unit sample response of the system.

b)	Find the:

i)  impulse response

ii)  output response for a step input applied at n=0 of a discrete time linear time  invariant system whose difference equation is given by

y(n) = y(n-1)+0.5 y(n-2)+x(n)+x(n-1)

2.a)	Explain magnitude and phase spectrum with reference to frequency response of 	system.

b)	Find the impulse response ,frequency , magnitude response and phase response y(n)-y(n-1)+316y(n-2)=x(n)−12 x(n-1)	of the given second order system

3.a)	Prove the following properties.

i)	x*(n)		               X*((-K))N RN(K)

ii)	x*((-n))N RN(n)              Xep(k) = ½ [X((K))N +X*((-K))N]RN(K)

b)	Let X(K) denotes the N-point DFT of the N-point sequence x(n) show that if x(n) satisfies the relation x(n) = -x(N-1-n) then X(0) = 0.

4.	An 8 point sequence is given by x(n) = {2,2,2,2,1,1,1,1}. Compute 8 point DFT of x(n) by   (i)  radix – 2 D I T  F F T  (ii)  radix – 2 D I F  FF T

Also sketch magnitude and phase spectrum

5.a)	An LSI system is characterized by:

y(n)=56y(n−1)−16y(n−2)+x(n). Find the system function and determine its response to the input signal x(n)=δ(n)−13δ(n−1)

b)	Write about Jury’s criterion.

(Contd…2)

]

Code No:320454			             :: 2 ::					OR

6.	A digital low pass filter is required to meet the following specifications

Pass band attenuation  1db

Pass band edge 4 KHz

Stop band attenuation  40 db, Stop band edge = 8KHz

Sample rate 24 KHz

The filter is to be designed by performing a bilinear transformation on an analog system function. Design a butter worth filter and realize it.

7.a)	Design a low pass filter using rectangular window by taking samples of ω (n) and with a cut-off frequency of 1.2 radians/sec.

b)	Compare the various window functions.

8.a)	Explain the different structures for realisation of IIR system. and explain how

conversion can be made from direct form I structure to direct form II structure.

b)	H(Z)=1+12Z−11−Z−1+14Z−21−Z−1+12Z−2Realize the given system in cascade and parallel form

-*-*-*-