Code No.410355

OR

IV-B.Tech. I-Semester Supplementary Examination November 2003

MECHANICAL VIBRATIONS

(Common to Mechanical Engineering, Production Engineering and Mechanical Manufacturing Engineering)

Time: 3 hours							Max.Marks:70

Answer any FIVE questions

All questions carry equal marks

---

1.a)	Define (i) free vibration and (ii) forced vibration.

b)	A spring of a valve mechanism has its wire diameter 2mm, mean coil diameter 	20mm. The number of coils is 7. Assuming the material to be steel determine the 	natural frequency of the spring alone. If the maximum speed of the engine is 3000 	rpm, determine the engine order frequency which is nearest to the spring natural 	frequency. Assume G = 75 x 109pa.

2.a)	Discuss about structural damping and viscous damping.

b)	An automobile can be modeled as a mass placed on four shock absorbers (shock 	absorber consists of a spring and a damper), such that each spring is equally 	loaded. Determine siffness and the damping constant of each shock absorber so 	that the natural frequency is 1.5 Hz and the system is critically damped. The mass 	of the vehicle is 1500 kg.

3.	A turbine disk of mass 30 kg with operating speed of 8000 rpm, is mounted at the 	centre of a shaft of span 0.5m and diameter 4cm. The disk has an unbalance of

10-3kgm. Assume the bearings to be rigid and the end conditions to be simply 	supported. Damping is represented by an equivalent viscous damping ratio of 	0.08. Determine

a)	Critical speed

b)	Whishing amplitude at (i) operating speed (ii) critical speed

c)	Power required to drive the rotor at  (i) operating speed (ii) critical speed.

4.a)	Explain the different vibration measuring instruments.

b)	Specify the lowest frequency of a vibrometer that can be measured with 1 percent 	error, if its natural frequency is 4 Hz and damping ratio is 0.2.

5.	Figure given below shows a locomotive hanling two freight cars. Determine the 	two non-zero natural frequencies. Assume following data:

Engine mass, mE = 200 tonne

Mass of first car, m1 = 60 tonne

Mass of second car, m2 = 80 tonne

Stiffness of coupling between the engine and first car K1=10MN/m

Stiffness of coupling between the first car and the second car, K2=15 MN/m.

(Contd…2)

Code No.410355				-2-					OR

6.	Determine natural frequencies in transverse vibration and the corresponding mode 	shapes of the mass coupled system shown in figure given below.

7.	A steam turbine blade of length l, can be considered as a uniform cantilever beam, 	m per unit length, with a tip mass M as shown in figure given below. The flexural 	rigidity of the blade is EI. Determine the fundamental frequency by Rayleigh’s 	method.

8.	Write short notes on the following:

a)	Logarithmic decrement

b)	Vibration isolation

c)	Matrix iteration method.

***     ***     ***