Code No: 411056

OR

IV-B.Tech. I-Semester Supplementary Examinations November, 2003

DIGITAL CONTROL SYSTEMS

(Electronics and Instrumentation Engineering)

Time: 3 Hours							Max. Marks: 70

Answer any Five questions

All questions carry equal marks

---

1.	The signal f(t)=5sin(20πt)+2Πt−0.140.16

where Π(x) is the unit pulse signal defined byΠ(x)=1 ifx≤12

is sampled at 25 samples per second. Determine the Z-transform of f(kT) for 	0≤k≤8.

2.  	Find the state variable models for the following system represented by the difference equation

(i)   y(k+3)+5y(k+2)+7y(k+1)+3y(k)=0

(ii)  y(k+2)+3y(k+1)+2y(k)=5r(k+1)+3r(k)

3.  	Consider the system defined by,

x1(k+1)

x2(k+1)

a         b

c         d

x1(k)

x2(k)

1

1

=				          +	   u(k)

1      0

x1(k)

x2(k)

y(k)=

Determine the condition on a, b, c and d for complete state controllability and complete observability.

4.	Explain the Liapunov stability analysis of linear time invariant continuous time system and linear time invariant discrete time system.

Contd…2.

Code No.411005			  -2-					Set No.4

- The given system shows a unity feedback system with a forward path transfer function G(s)  =          10

s(s+1)

Determine phase margin & bandwidth of compressed and uncompressed schemes. In each, adjust KA to a value of which gives error constant Kv = ω

6.	A discrete time regulator system has the plant equation

X(k+1) = 2−1−11 X(k) + 43 u(k)

y(k) = 11 X(k) + 7u(k)

Design a state feedback control algorithm with u(k) = -Kx(k) which places the closed loop characteristic roots at +j1/2,  -j1/2.

7.a) 	Derive the relations of prediction-type Kalman filter.

b) 	Design the reduced order observer (first-order-observer), for the plant model given by

X(k+1)=1.0  0.0952

we are measuring position, x1(k) and will estimate velocity, x2(k).

The closed-loop characteristic equation was chosen to be

e(z) = z2 – 1.776 z + 0.819 = 0.

8.	Write notes on linear regulator with a prescribed degree of stability that can be solved by using state regulator results.

-x-