Code No.410252

OR

IV-B.Tech. I-Semester Supplementary Examination November 2003

COMPUTER METHODS IN POWER SYSTEMS

(Electrical and Electronics Engineering)

Time: 3 hours							Max.Marks:70

Answer any FIVE questions

All questions carry equal marks

---

1.a)	Develop an equivalent network showing the interconnection of sequence 	networks to simulate a double line to ground fault.

b)	A star connected three phase synchronous generator has a solidly grounded 	neutral and feeds a line upon which there is a fault. The total impedances upto the 	fault are as follows

To positive sequence currents Z1=j2.2

To negative sequence currents Z2 = j1.9

To zero sequence currents Z0 = j1.4.

If the generator induced voltage is 11kv, calculate the fault currents for (i) LG 	fault  (ii) a double line to ground fault  (iii) a LL fault. Also calculate the voltages 	of the lines at the fault.

2.a)	A 60HZ generator is rated 625 KVA, 480 V with Xd''=0.08pu. It supplies a 	purely resistive load of 500kw at 480V. The load is connected directly across the 	terminals of the generator. If all the three phases of the load are short circuited 	simultaneously. Find the initial symmetrical RMS current in the generator in pv 	on a base of 625KVA, 480V.

b)	A 10MVA, 132/6.6KV transformer having a reactance of 0.15pu is fed from an 	infinite bus. Transformer feeds two motors each 5MVA, 6.6kv. Each motor has a 	transient reactance of 0.3 pu and a subtransient reactance of 0.2pu based on its 	own rating.

A 3 phase fault occurs at the terminals of a motor as shown in the figure. Find the 	subtransient fault current and also subtransient current in breaker D.

3.a)	Derive swing equation of a single machine connected to an infinite bus.

b)	Explain the equal area criterion of stability applicable to a power system.

(Contd…2)

Code No.410252				-2-					OR

4.a)	What are the factors affecting the steady state stability and transient stability of a 	power system.

b)	A typical power system is shown below

The system is operating in equilibrium with pi = 1.5pu. When one of the lines is 	suddenly switched out, predict whether the system will be stable or not.

5.	Formulate the load flow problem in rectangular coordinates. Explain the Newton 	Raphson method of solution with the algorithm and flow chart. What 	simplifications are possible in NR method. Explain clearly how voltage and 	reactive power limits are considered.

6.	The load flow data for a 3 bus system is given in Tables below. Taking bus 1 as 	slack bus, determine the voltages at the end of Ist iteration by Gauss. Seidal 	method.

| Bus Code | Impedance | Admittance |
| --- | --- | --- |
| 1-2 | 0.06+j0.18 | J0.05 |
| 1-3 | 0.02+j0.06 | J0.05 |
| 2-3 | 0.04+j0.12 | J0.05 |

| Bus Code | Voltage | Generation P Q | Load P Q |
| --- | --- | --- | --- |
| 1 | 1.06+j0.0 | 0.0 0.0 | 0.0 0.0 |
| 2 | 1.0+j0.0 | 0.2 0.0 | 0.0 0.0 |
| 3 | 1.0+j0.0 | 0.0 0.0 | 0.6 0.25 |

7.a)	What are the different methods for solution of the differential equations in the 	transient stability problem. Discuss the numerical stability, convergence and 	computational aspects of the various schemes.

b)	Explain Newton-Raphson technique for the solution of simultaneous algebraic 	equations.

8.	Write short notes on:

a)	Alternate cycle solution method

b)	DC load flow studies

c)	Methods of improving stability

d)	State estimation.

***     ***     ***