Code No.410257

OR

IV-B.Tech. I-Semester Supplementary Examination November 2003

POWER SEMICONDUCTOR DRIVES

(Electrical and Electronics Engineering)

Time: 3 hours							Max.Marks:70

Answer any FIVE questions

All questions carry equal marks

---

1.a)	Explain with circuit diagram, the speed control of separately excited D.C.motor 	using fully controlled single-phase bridge converter. Show how 2-quadrant of 	drive is made possible.

b)	A 220 Volts, 960rpm, 13 Amps separately excited d.c.motor has armature 	resistance of 2 ohms. It is fed from a single-phase half controlled rectifier with an 	a.c.source of 230 volts, 50HZ. Assuming continuous conduction, calculate motor 	torque for  = 60o and speed 600 rpm.

2.a)	A fuly controlled rectifier is feeding a separately excited d.c.motor driving a load. 	Motor is operating in steady-state with a rectifier firing angle of 30o. Firing angle 	is now changed from 30o to 60o. Explain how the motor current and speed will 	change with fire.

b)	A 220 Volts, 1500 rpm, 10 Amps separately excited d.c.motor is fed from a 1-	phase fully-controlled rectifier with an a.c.source voltage of 1-phase fully-	controlled rectifier with an a.c. source voltage of 230 Volts, 50 HZ, Ra=2 ohms. 	Conduction can be assumed to be continuous. Calculate the firing angles for

(i)	Half the rated motor torque and 500 rpm.

(ii)	Rated motor torque and (-1000) rpm.

3.a)	Discuss in detail with suitable circuit diagrams and waveforms, the speed control 	operations of a separately excited d.c.motor using a circulating current dual 	converter.

b)	A 230 Volts, 870 rpm, 100 A separately excited d.c.motor has an armature 	resistance of 0.05 ohm. It is coupled to an over hauling load with a torque of 400 	N-m. Determine the speed at which motor can hold the load by regenerative 	braking.

4.a)	In a variable frequency control of a synchronous motor, why is (v/f) ratio 	maintained constant upto base speed and V constant above the base speed? 	Explain.

b)	Why is the load commutated inverter fed synchronous motor drive frond suitable 	for high speed and high power applications? Explain.

(Contd…2)

Code No.410257				-2-					OR

5.a)	With a block schematic, explain the closed-loop torque control scheme of a 	d.c.motor.

b)	A 230 Volts, 1200 rpm, 15 Amps separately excited motor has an armature 	resistance of 1.2 ohm. Motor is operated under dynamic braking with chopper 	control Braking resistance has a value of 20 ohms.

(i)	Calculate the duty ratio of chopper for motor speed of 1000 rpm and 			braking torque equal to 1.5 times the rated motor torque and

(ii)	What will be the motor speed for a duty ratio of 0.5 and motor torque 			equal to its rated torque?

6.a)	Discuss the speed control of induction motor by stator voltage control. With 	suitable speed-torque curves, explain how this is achieved.

b)	A 3-phase, delta connected, 6 pole, 50 HZ, 400 Volts, 925 rpm, squirrel-cage 	induction motor has the following parameters:-

RS= 0.2 ohm; Rr1=0.3 ohm; XS=0.5 ohm; Xr1=1 ohm. The motor is fed from a 	voltage source inverter with a constant v/f ratio from 0 to 50 HZ and constant 	voltage of 400 volts above 50 HZ frequency.

Determine the breakdown torque for a frequency of 100 HZ as a ratio of its value 	at 50 HZ.

Also, obtain the torque at the rated motor current and 75 HZ as the ratio of rated q	full-load torque of the motor.

7.a)	Explain the theoretical principle of slip-energy recovery scheme in induction 	motor speed control. Draw a suitable circuits and explain.

b)	A 3-phase, 400 V, 50 HZ, 6 pole, 960 rpm, -connected wound rotor induction 	motor has the following constants referred to the stator:

RS= 0.5 ohm; Rr1=0.7 ohm; XS=1.5 ohm; Xr1=1.6 ohm.

The speed of the motor is reduced to 800 rpm at half full load torque by injecting 	a voltage in phase with the source voltage in to the rotor. Calculate the magnitude 	and the frequency of the injected voltage. Stator to rotor turns ratio is 2.2.

8.	Write short notes on the following:-

a)	A.C. voltage controllers

b)	Four quadrant operation of D.C motor by dual converters

c)	CSI fed synchronous motor operation.

***     ***     ***