Code No: 210552

OR

II B.Tech I-Semester Supplementary Examinations, November-2004

PRINCIPLES OF ELECTRICAL ENGINEERING AND NETWORKS

(Computer Science Engineering)

Time: 3 hours							Max. Marks: 70

Answer any Five questions

All questions carry equal marks

---

1.a)	State and explain Kirchoff’s laws.

b)	Two power sources supply a common load of 10Ω as shown. Calculate the 	voltage across the 10Ω resistor.

2.a)	Derive the expression for emf generated in D.C generator.

b)	A 4-pole long shunt compound generator supplies a load of 100 A at a 	terminal voltage of 400V. Calculate the generated emf of the machine if the 	resistance of the armature is 0.02Ω, the series field resistance is 0.04Ω and 	neglect the armature reaction.

3.a)	Explain the construction and principle of operation of single phase 	transformer.

b)	A 50 KVA transformer has an efficiency of 98% at full load and 0.8 PF. And 	an efficiency of 96.9% at 1/4th full load and 0.8 PF. Determine the iron losses 	and full load copper losses.

4.a)	Explain the slip-torque characteristics of an induction motor with a neat 	sketch.

b)	A 12 pole. 3-phase induction motor runs at 485 rpm on a 50 Hz supply 	calculate i) the slip and 	ii) the frequency of rotor emf.

5.a)	Derive the expression for emf generated in an alternator.

b)	Explain the constructional details of an alternator with neat sketch.

(Contd….2)

Code No: 210552				-2-					OR

6.a)	Explain i)	Max-power transfer theorem.

ii)	Reciprocity theorem.

b)	In the circuit shown below, calculate the current through 25Ω resistance 	using Thevinin’s theorem.

7.a)	Explain the No-load, and load characteristics of a D.C. shunt generator.

b)	Obtain the relation between line and phase values of emfs and currents in star 	connection.

8.	Write short notes on:

a)	DC motor starters.

b)	OC and SC tests on transformer.

c)	Super position theorem.

@&@&@