Code No: 210251

OROR

# II B.Tech. I-Semester Supplementary  Examinations, November-2004

# ELECTROMAGNETIC FIELDS

(Electrical and Electronics Engineering)

Time: 3 hours							    Max Marks: 70

# Answer any FIVE Questions

# All Questions carry equal marks

---

1.a)	State and explain Coulombo’s law.

b)	State the electrostatic boundary conditions and explain.

2.a)	Explain electric dipole moment and derive expression for potential at large distances due to a dipole.

b)	A current I flows down a wire of radius R. (i) If it is uniformly distributed over the surface, what is the surface current density K?

(ii) If it is distributed such that volume current density J is inversely proportional to the distance from the axis, what is J?

3.a)	Find magnetic field at point P in the following figure

b)	A thick slab with thickness extending from z = - a to z = + a carries a uniform volume current J = J↑A/m2.  Find magnetic field inside and outside the slab.  The slab extends to infinity along the xy plane.

4.a)	Define emf. Explain how static and dynamic emfs are produced.  Show that a  changing magnetic field produces an electric field.

b)	A copper wire carries a conduction current of 1 A.  Determine the displacement current at 100 MHZ.  = o, 5.8x107 ℧/m.

5.a)	Prove that electric field is uniform inside a large parallel plate capacitor.

b)	Prove that magnetic field is uniform inside a long solenoid.

6.a)	Derive expression for vector magnetic potential due to a long wire of circular cross-section carrying a current I.

b)	Two narrow circular coils A and B have a common axis and are 10 cm apart. Coil A has 10 turns of radius 5 cm and coil B has 1 turn of radius 7.5 cm. If the magnetic field at the center of Coil A must be zero, what current should flow in coil B?

(Conted…2)

Code No: 210251        			-2-					OR

7.a)	Derive expression for intrinsic impedance and find its value for free space.

b)	Currents I1 = 10, I2 = -10 are flowing in two long parallel wires 20 cm apart. Find magnitude and direction of magnetic flux density at a point 20 cm distant from each wire.

8.	Write notes on

a) Polarisation

b) Poynting.

*$*$*$*