Code No: 310453

OR

III B.Tech. I-Semester Supplementary Examinations, November-2004

# ANALOG COMMUNICATION

(Electronics and Communication Engineering)

Time: 3 hours							    Max Marks: 70

## Answer any FIVE Questions

All Questions carry equal marks

- - -

1.a) 	Give a neat sketch of a telephone relay and explain its working.

- A telephone relay having a resistance of 1500 ohms and an inductance of

3 Henries has an operating current of 20mA and release current of 8 ma. Calculate 	time lag in operating and in releasing when the relay is working across a 50v 	supply. Assume the relay short circuited under release conditions.

2.a) 	What are the applications of balanced modulator? Prove that the balanced modulator produces an output consisting of sidebands, with the carrier removed.

b) 	An AM broadcast station has a modulation index of 0.75 on the average. What would be its average power saving if it could go over to SSB-SC transmission while having to maintain the same signal strength in its reception area.

3.a) 	Explain clearly the difference between Amplitude, Frequency, and Phase modulations, beginning with the definition of each type and the meaning of the modulation index in each case.

b) 	Explain with the block diagram the Armstrong method of FM generation.

4.a)	Distinguish between negative peak clipping and diagonal peak clipping in an envelope detector.  The output of a diode envelope detector is fed through a DC blocking capacitor to an amplifying stage, which has an input resistance of 10 kilo-ohms. If the diode load resister is 5k-ohm, determine the maximum depth of sinusoidal modulation the detector can handle with out negative peak clipping.

b)	Consider a composite wave obtained by adding a non-coherent carrier Ac cos 2 π fc t + φ to DSB-SC wave Xt cos2 π fc t where Xt is the message waveform.  This composite waveform is applied to ideal envelope detector.  Find the resulting detector out put.

Evaluate this for (1) φ = 0 and(2)φ ≠ 0 and Xt << Ac .

5.a)	Explain how frequency   stability is achieved in modern transmitter.

b)	Describe with aid of suitable diagram, the principal method of SSB power  	generation.

c)	Describe the advantages of a SSB system for high frequency point to point 	communication & explain why it is unsuitable for broadcasting.

(Contd…2)

### Code No: 310453					-2-				OR

6.a)	What are the advantages of Super heterodyne receiver as compared to a TRF          	receiver?

b)	Explain clearly what is meant by image frequency in a superhet receiver and     	how it can be eliminated.

c)	Define conversion transconductance of a mixer.  With the help of a typical     	circuit diagram, explain the working of a separately excited mixer.

7.a) 	Differentiate between simple, delayed and amplified AGC and explain their     	action with the help of simple circuit blocks.

b) 	Discuss briefly similarities and differences between FM and AM receivers.

c) 	Write in detail about the limiter used in FM receiver.

8.a) 	How does Johnson noise arise in a resistor?

b) 	What are the different types of man made noise, which may arise in a communication system?

c)	Define the equivalent noise temperature of a radio receiver.

d) 	Define noise figure of a radio receiver.

= + = + =