Code No: 310452

OR

III B.Tech. I-Semester Supplementary Examinations, November-2004

CONTROL SYSTEMS

(Common to Electronics and Communication Engineering, Electronics and Instrumentation Engineering and Electronics and Control Engineering)

Time: 3 hours							    Max Marks: 70

Answer any FIVE Questions

All Questions carry equal marks

- - -

1. a)	In Chemical processes, it is desirable to control the chemical composition of the product.  A measurement of the composition may be obtained by using an infra red stream analyzer.  The value on the additive stream may be controlled.  Obtain a block diagram of the complete feedback control loop and briefly describe the operation.

b)	Obtain a control system model of the human arm picking and placing objects.  Briefly explain each of the blocks used in this model.

2. a)	A system has the following transfer function

G(s) = 5(s + 100)s2 + 60s + 500

Find the output for an unit step input.  What is the final value of the output?

b)	Obtain the transfer function Vo(s)V1(s) of the op-Amp circuit shown below

Assume ideal op-Amp.

3.	A control system has the open-loop plant transfer function.  G(s) =Ks(s+2) (s2 +4s +5) with unity negative feedback.  Sketch the root locus for K ≥ 0with all pertinent geometrical properties.  For what value of K do you get dominant poles?

4.	Give the transfer function G(s) = 300(s +100)s(s +10) (s2 +6s +20) obtain the Bode plot and hence the gain and phase margins.

Contd…2

Code No: 310452				.2.					OR

5. a)	A feedback system is shown below.  The objective is to reduce the effects of noise and disturbance.

Assuming R(s)= 0

- Determine the effect of d(s) on y(s).

- Determine the effect of n(s) on y(s).

- Select the best value of K, so the effect of steady-state error due to the disturbance and noise is minimized.  Assume step disturbance As and step noise Bs.

b)	A control system with unity negative feedback has the plant transfer function. G(s) = K(s +2)s(1 +τs) (1 +2s).  Select the parameters K and  so that the system is stable and the steady-state error to a ramp input is less than or equal to 0.25.

6.	Determine the stability of the system whose loop transfer function is G(s) =3(s+2)s2 (s+4) by applying Nyquist criterion.  Measure the gain and phase margins.

7.	A control system with negative unity feedback has the plant Gp(s) =400s(s+40) we wish to use a PI controller Gc(s) =K1 +1s.  Find a suitable value for K1 so that the overshoot for an unit step input is approximately 20%.  Also, find the setting time of the closed-loop system.

8.	Obtain the state variable differential matrix equation for the following circuit.

How do you choose the state variables?  Obtain the state transition matrix assuming the product RC = 1.

@&@&@