Code No: NR-310102

Set No.

1

III B.Tech. I-Semester Supplementary Examinations, November-2004

STRUCTURAL ANALYSIS-I

(Civil Engineering)

Time: 3 hours							    Max Marks: 80

Answer any FIVE Questions

All Questions carry equal marks

- - -

1.	Determine the slope and deflection at the point C of the propped cantilever beam shown in figure.

2.	A beam of total length 7 m is fixed at its left end and propped at a distance of 2 m 	from its right end. If a UDL is placed on the beam with 10kN/m intensity at the 	right end and 4 kN/m intensity at the left end, find the positions and magnitudes 	of the maximum positive and negative bending moments.

3.	Using the method of tension coefficients, find the load in each member of the 	space frame shown in Fig. 11, when a load of 1kN is applied at joint E acting 	towards joint D. ABEF is a rectangle in a horizontal plane and A, B, C and D are 	pinned supports forming a square on a vertical wall.

4.	A continuous beam ABC consists of spans AB and BC of lengths 4m and 6m       respectively, the ends A and B being fixed. C is a free end. The span AB carries a uniformly distributed load of 24 kN/m while the span BC carries a point load of 108 kN at a distance of 2m from C. Find the support moments and support reactions.

5.	Define Strain energy. Derive an expression for strain energy for a linear elastic system under axial load.

6.	Calculate the focal length of a girder of 18m span carrying a dead load of 3kN/m and a uniform live load of 2kN/m, 4m long travelling from left to right.

Contd….2

## Code No: NR-310102			.2.				Set No: 1

7.		An over hanging beam DABC , 14m long is supported at A and B . DA=BC=2m;

AB=10m. Draw the influence lines for the reactions at A and B, shear and bending     moment at section 3m from A. Hence obtain their values for a uniformly     distributed load of 10kN/m, 5m long acting from A.

8.	Draw the SFD and BMD for the beam of 10m, which is fixed at the left end. It is 	subjected to udl of 2kN/m from the free end to middle of beam and 3kN/m from 	the fixed end to the middle of beam. It is also subjected to point load of 6kN 	at the center of beam. 	Use graphical procedure.

###

Code No: NR-310102

Set No.

2

III B.Tech. I-Semester Supplementary Examinations, November-2004

STRUCTURAL ANALYSIS-I

(Civil Engineering)

Time: 3 hours							    Max Marks: 80

Answer any FIVE Questions

All Questions carry equal marks

- - -

1.	Determine the slope and deflection at the point C of the propped cantilever beam shown in figure.

2.	A fixed beam of span 6 m has two concentrated loads 25 kN and 20 kN at 2 m 	and 4 m distances from the left support besides a 4kN/m UDL in between the two 	loads. Analyze the beam and draw the bending moment and shear force diagrams.

3.	The space frame shown in Figure is pinned to supports A, B, C and D in a 	horizontal plane. The member EF is horizontal and 2m above the level of the 	supports. The loads at the joints E and F act in a horizontal plane. Find the forces 	in the members of the frame using the method of tension coefficients.

4.	A continuous beam ABC consists of spans AB and BC of lengths 8m and 6m respectively. A is fixed. The point B is simply supported and end C is free. The span AB is subjected to a clockwise couple of 80 kN-m at its middle point and the span BC carries a point load of 40 kN at its middle point. Calculate the support reactions and support moments.

5.	Define Strain energy. Derive an expression for strain energy due to bending moment.

Contd….2

## Code No: NR-310102			.2.				Set No: 2

6.	A uniformly distributed load of 3kN/m run 5m long crosses a girder of 15m span.      Construct the maximum B.M diagrams and calculate its values at 5m, 7.5m and 10m from the left hand support.

7.	Four equal loads of 60kN each, equally spaced at 2m apart followed by a uniformly distributed load of 40kN/m run at a distance of 2m from the last 60kN, cross a girder of 20m span from right to left. Using influence lines, calculate the S.F and B.M at a section 8m from the left support, when the leading 60kN load is 5m from this support.

8.	Draw the SFD and BMD for the beam that is resting at both the ends, and has a span of 8m. The beam is loaded with UDL of 4kN/m for the middle 1/3 portion and it is subjected to point loads of 5kN at both end of the UDL loads. Use graphical procedure.

###

Code No: NR-310102

Set No.

3

III B.Tech. I-Semester Supplementary Examinations, November-2004

STRUCTURAL ANALYSIS-I

(Civil Engineering)

Time: 3 hours							    Max Marks: 80

Answer any FIVE Questions

All Questions carry equal marks

- - -

1.	Determine the slope and deflection at the point C for the propped cantilever beam shown in figure.

2.	Find the fixed end moments for a fixed beam of span 5 m when the beam is 	subjected to a UDL whose intensity varies from 5kN/m at the left end and 2 kN/m 	at the right end of the span.

3.	Four straight wires AO, BO, CO and OD are made of the same material and have 	the same cross section. Their upper ends are connected to a horizontal ceiling, the 	points A, B, C and D forming a rectangle in which AB = 2m and BC = 2.5m. The 	junction O is 3m vertically below a point Q on the ceiling, 1m from AB and 	0.75m 	from AD. The connections are adjusted so that initially there is no 	slackness in 	any wire and a load of W is then suspended from O. Denoting the 	pull in the wire AO as P, find the pull in each of the remaining wires in terms of P 	and W. Use the method of tension coefficients.

4.	A beam ABC 5.80 m long is fixed at A and simply supported at B(4 m from A) and free at C. It carries a point load of 5 kN at C. Analyze the beam for support reactions and draw the B.M.D and S.F.D.

5.	Define Strain energy. Derive an expression for Strain energy for a linear elastic system under shear force.

6.	A uniformly distributed load of 2kN/m, 6m long crosses a girder of 20m span.      Construct the maximum S.F diagrams and calculate its values at 4m, 6m and 10m from the left support.

7.	A beam ABC is supported at A,B and C and has an internal hinge at D at a            distance of 3m from A. AB=6m and BC=9m .Draw the influence lines for the            reactions at   supports and S.F and B.M at a point 1m from B in the span BC.

Contd….2

## Code No: NR-310102			.2.				Set No: 3

8.	A beam AB, 20 m long supported on two intermediate props 12 m apart carries a 	U.D.L 6kN/m together with concentrated loads of 30kN at the left end A and 	50kN at the right end B. The props are so located that the reaction is the same at 	each support. Determine the position of the props and draw S.F.D and B.M.D. 	Mark the values of the maximum B.M and S.F.

###

Code No: NR-310102

Set No.

4

III B.Tech. I-Semester Supplementary Examinations, November-2004

STRUCTURAL ANALYSIS-I

(Civil Engineering)

Time: 3 hours							    Max Marks: 80

Answer any FIVE Questions

All Questions carry equal marks

- - -

1.	Determine the deflections at the points C and D for the propped cantilever beam shown in figure.

2.	Find the fixed end moments of a fixed beam when the left support rotates in the 	clockwise direction by . The span of the beam is 'I' and the flexural rigidity of 	the beam is EI.

3.	Three flexible wires AO, BO and CO are attached at their upper ends to a rigid 	horizontal plane and are just tight in the positions shown in Figure, when no load 	is applied at O. Determine the tension in each wire when a load of 5kN is applied 	at O. Use the method of tension coefficients.

4.	A continuous beam ABC is fixed at A and simply supported at B and C. AB and BC are 4 m and 6 m long. The spans AB and BC carry point loads 2kN and       20 kN at the respective mid-spans. Find the bending moments at A and B and draw the bending moment diagram.

5.	State and explain Castigliano’s first theorem taking any example.

6.	Two point loads of 6kN and 8kN spaced 6m apart cross a girder of span 16m, the 6kN load leading from left to right. Construct the maximum S.F and B.M diagrams, stating the absolute maximum values.

Contd….2

## Code No: NR-310102			.2.				Set No: 4

7.	A beam ABCD is simply supported at A,B,C and D and contains two internal hinges in the span BC at E and F. AB=12m, BC=16m, CD=12m BF=4m and FC=4m. Draw the influence lines for reactions at supports B and D and B.M at G where G is 5m from A.

8.	A beam of 10m length is simply supported at its end beam and carries a u.d.l of 	20kN/m over the left half its span together with point loads of 20kN, 40kN 	and 20kN at 1.5m, 2.5m and 5m respectively from right hand support. Draw the 	shear force and bending moment diagram. Also find the position and magnitude 	of the maximum bending moment. Use graphical procedure.

###