Code No: 310253

OR

III B.Tech. I-Semester Supplementary Examinations, November-2004

ELECTRONICS CIRCUITS

(Electrical and Electronics Engineering)

Time: 3 hours							    Max Marks: 70

Answer any FIVE Questions

All Questions carry equal marks

- - -

1. a)	Show that for the self-bias circuit the stability factor is given by

S=1+β1+βRERE+RB

- In a self-bias circuit, R1=Rc=500Ω, R2=5k, RE=100±10Ω, μFE=75,  ICB0=0.2μA and Vcc=20v.  Determine the Q-point.

2. a)	Derive the h-Para model for CE configuration and how do the h-parameters are

determined from the i/p and o/p characteristics.

b)	What are the effects of bypass and coupling capacitors on the LF response of the amplifier?

3. a)	A single-stage CE amplifier has RS=1kΩ, R1=50k, R2=2k, RC=1k, RL=1.2k, hfe=50 and hie=1.1k.  Find the current gain, Ri, Ro and AV.

b)	Compare different biasing circuits in terms of stability factor S.

4. a)	Derive the expressions for hybrid-π parameters in terms of LF h-parameters.

b)	Given the following transistor measurements made at Ic=5mA, Vce=10v, and at room temperature:

hfe=100,   hie=600Ω,   |Aie|=10 at 10 MHZ, Ce=3PF.

Find fβ, fτ, fα, and Ce.

5. a)	Explain the biasing circuit for an enhancement type MOSFET.

b)	Determine the output voltage VO and the o/p resistance of self-bias FET amplifier for the given circuit parameters

Rd=1kΩ, RS=1kΩ, R1=900kΩ, R2=100kΩ,

rs (Signal generator resitance)=10kΩ, gm=5mA/v, and  rd=10kΩ, CS=50μFand Cg=μF.

6.a)	Discuss the advantages and disadvantages of negative feedback.

b)	Device the expressions for Ri and Ro of voltage-series feedback amplifier.

7.a)	Discuss the conditions for sustaining oscillations of oscillators.

b)	Explain with a neat circuit, the principle of operation of RC-phase shift oscillator.

8.	Write short notes on any two of the following:

a)	Thermal runaway & thermal stability.

b)	Various stability factors.

c)	Crystal oscillator.

x-x-x