Code No: 310454

### OR

## III B.Tech. I-Semester Supplementary Examinations, November-2004

## POWER AND INDUSTRICAL ELECTRONICS

(Common to Electronics and Instrumentation Engineering and Electronics and Communication Engineering)

Time: 3 hours							    Max Marks: 70

## Answer any FIVE Questions

## All Questions carry equal marks

---

1. a)	In what respects does a class C amplifier differ from class A and class B amplifiers?

- An induction furnace derives 10  kw of h.f  power at 20 MHZ from a class C amplifier.  D.C. Voltage of 10 K.V. is available.  The loaded Q of the tank circuit is 8.  Calculate the d.c current to be supplied by the source and the L and C component values.  Assume Idc/I1=0.62.  Justify any other assumptions made.

2. a)	Draw the circuit of a typical turned amplifier with capacitive coupling and tapped tank circuit and explain.

- How it works

- The role of the tank circuit and

- The purpose of the tap

b)	Calculate the approximate surface area of a heat sink for an SCR whose ratings are given below, if actual Pj(max)=75w.

Tjmax=1250C			TAmax=550C

θJA=1250C/w

(Assume θ jc=0.A0C/W)

3. a)	Explain various Turn-On methods of an SCR and the advantages/disadvantages of each.

- With a simple circuit, explain

  - Crow-bar protection for power converters and

  - Static circuit Breaker.

4. a)	Draw the basic circuit of a push-pull inverter and explain its working.

- A phase controlled half wave rectifier uses a thyristor with RL load and a free wheeling diode.  Shown the voltage and current wave forms and explain the operation of the circuit.

5. a)	Explain the gate characteristics of a thyristor and indicate the preffered gate area.

- Draw block schematic of a sequence timer to control welding process and explain its role.

(Contd….2)

# Code No: 310454					-2-			  	OR

6. a)	A UJT pulse Trigger circuit is to be used for feeding pulses to two thyristors in a single phase half controlled converter.  Draw the circuit diagram for the pulse generator and explain its working.

- A chopper with a 200V D.C input has ON time of 30ms and OFF time of 10 ms.  Determine the value of the average D.C. output voltage.

7. a)	Explain the principles of:

- Dielectric heating

- Induction heating.

b)	Indicate any two applications of each.

8.	Write brief notes on:

a)	Forced Turn-Off for SCRs.

b)	DIAC

c)	Cross-over distortion in amplifiers.

d)	Complementary push-pull amplifier and

e)	Commutation of a TRIAC with inductive load

x-x-x