Code No: 410402

Set No.

1

IV B.Tech. I-Semester Regular Examinations, November-2004

COMPUTER NETWORKS

(Common to Electronics and Communications Engineering, Electronics and Instrumentation Engineering, Electronics and Telematics, Bio-Medical Engineering and Computer Science and Systems Engineering)

Time: 3 hours							    Max Marks: 80

Answer any FIVE Questions

All Questions carry equal marks

- - -

1.a)	With a neat diagram, explain the functionality of layers, protocols and interfaces.

b)	What is the difference between connection-oriented and connectionless switching.

2.a)	Differentiate B-ISDN   ATM reference model with OSI  reference model

b)	Explain the ISDN  interfaces.

3.a)	Explain in detail the operation of slotted Aloha?

b) 	A Ten thousand air line reservation station is competing for the line of

single slotted Aloha channel.  The slot is 125  sec , What  is approximate

total channel load.

4.a)	Explain the operation of SLIP	.

b) 	Write different types of LCP packets that are defined in RFC 1661.

5.a)	What is count-to-infinity problem ? Discuss how it can be overcome.

b) 	Compare leaky bucket and token bucket algorithm used for congestion control.

6.a) 	Explain and compare the  different adaptive routing algorithms.

b) 	Discuss the concept of connectionless internetworking.

7.a) 	In order to overcome the problem of sequence numbers wrapping around while old 	packets still exist, one could use 64-bit sequence numbers. However, theoretically, an 	optical fibre can run at 75 Tbps. What maximum packet lifetime is required to make sure 	that future 75 Tbps networks do not have wraparound problems even with 64-bit sequence numbers? Assume that each byte has its own sequence number, as TCP does.

b) 	The maximum payload of a TCP segment is 65,515 bytes. Why such a strange number was chosen?

8.a)	Explain Breaking DES in secret key algorithms.

b)	What is IDEA (International Data Encryption Algorithms). Explain it with a neat diagram.

-*-*-*-

Code No: 410402

Set No.

2

IV B.Tech. I-Semester Regular Examinations, November-2004

COMPUTER NETWORKS

(Common to Electronics and Communications Engineering, Electronics and Instrumentation Engineering, Electronics and Telematics, Bio-Medical Engineering and Computer Science and Systems Engineering)

Time: 3 hours							    Max Marks: 80

Answer any FIVE Questions

All Questions carry equal marks

- - -

1.a)	Write short notes on interfaces and services of ISO layers.

b)	Two networks each provide a reliable connection oriented service.  One of them 	offers a reliable byte stream and other offers a reliable message stream. Are they 	identical? If so, why is distinction made? If not give an example of how they 	differ?

2.a) 	Why does ATM use small fixed-length cells.

b) 	List the advantages and disadvantages of having international standards for 	network protocols.

c) 	A system has an N-layer protocol hierarchy. Application generate messages of length M bytes. At each of the layers, an h-byte header is added. What fraction of the network bandwidth is filled with the header?

3.a)	Explain the need of framing in, details in data link layer.

b) 	The following data fragment occurs in the middle of data stream for which the character stuffing algorithm is used “ DLE STX,A, DLE  C, DLE, ETX”. What is transmitted.

c) 	If the bit string 011011111011101111110 is bit stuffed. What is the out-put string?

4.a)	Explain the role of Discovery Frames in source Routing bridges in detail.

b) 	Explain the algorithm used in Transparent bridges.

5.a)	Compare distance vector routing and link state routing algorithms.

b) 	Explain the use of hop-to-hop choke packets and load shedding for congestion 	control.

6.a) 	Discuss the functions of the communication subnet to provide datagram 	services.

b)  	Explain selective flooding and discuss some of its practical usage.

c)  	Give the format of IPv6 header.

Contd……..2

Code No: 410402				:: 2 ::				Set No: 2

7.a) 		A group of N users located in the same building are all using  the same remote 	computer via an ATM network. The average user generates L lines of traffic 	(input+ output) per hour, on the average, with the mean line length being P 	bytes, excluding the ATM headers. The packet carrier charges C paise per 	byte of user data transported plus X paise per hour for each ATM virtual circuit open. Under what conditions it is cost effective to multiplex all N 	transport connections onto the same ATM virtual circuit, if such multiplexing adds 2 bytes of data to each packet? Assume that even one ATM virtual circuit has sufficient bandwidth for all the users.

b) 	What are blocking calls, primitives and non-blocking primitives?

8.a)	Explain clearly the RSA algorithm with an example.

- Write brief notes on Knapsack public key algorithm.

-*-*-*-

Code No: 410402

Set No.

3

IV B.Tech. I-Semester Regular Examinations, November-2004

COMPUTER NETWORKS

(Common to Electronics and Communications Engineering, Electronics and Instrumentation Engineering, Electronics and Telematics, Bio-Medical Engineering and Computer Science and Systems Engineering)

Time: 3 hours							    Max Marks: 80

Answer any FIVE Questions

All Questions carry equal marks

- - -

1.a) 	What is the purpose of source and destination address in the IPX packet.

b) 	Compare the various networking services.

2.a)   	Briefly explain ATM layers..                                           			                                                            b)  	Draw ISDN system architecture for large business applications and explain

c)   	Explain input queueing at an ATM switch.

3.a)	Explain the operation of simplex stop and wait protocol.

b) 	What is piggy backing? Explain the merits of piggy backing.

4.a)	Explain the schemes of channel allocation? Explain static channel  allocation in LANS.

b) 	Explain assumption used in designing the dynamic channel allocation in LAN.

5.	Design algorithm using Dijkstra’s algorithm, to list all nodes on the cheapest path to a given destination.

6.a)  	What is the drawback in running TCP/IP over ATM.

b)  	Give the structure of concatenated virtual circuits for Internetworking.

c)  	Discuss the goals of IPv6.

7.a)  	Consider a situation when you are measuring the time to receive a           		TPDU. When an interrupt occurs, you read out the system clock in 	milliseconds. When the TPDU is fully processed, you read out the clock 	again. You measure 0 msec 270,000 times and 1 msec 730,000 times. How 	long does it take to receive a TPDU?

b) 	Hazard a guess at what the field sizes for AAL 2 were, before they were 	pulled from the standard.

8. a)	Explain how authentication-using Kerberos operates?

- Explain the working of Mutual authentication using Public-Key Cryptography.

-*-*-*-

Code No: 410402

Set No.

4

IV B.Tech. I-Semester Regular Examinations, November-2004

COMPUTER NETWORKS

(Common to Electronics and Communications Engineering, Electronics and Instrumentation Engineering, Electronics and Telematics, Bio-Medical Engineering and Computer Science and Systems Engineering)

Time: 3 hours							    Max Marks: 80

Answer any FIVE Questions

All Questions carry equal marks

- - -

1.a)	What is the difference between a service –point  address, a logical address and  a

physical address?.

b)     	What are the responsibilities of the network layer? Explain them.

2.a)  	What are the advantages and disadvantages of twisted pair?

b)  	What are the different categories of twisted pairs and explain each.

c)   	Write about broadband coaxial-cable.

3.a)	What are the various methods of accessing a broadcast channel?

b) 	Describe the expression for throughput S, for an infinite population pure

ALOHA system

4.a)	What are the frame control bytes used in 802.4 and explain their functions.

b)   	What are collision free protocol ? Explain any one method

5.	What are some of the possible services that a link-layer protocol can offer to 	network layer? Which of these link-layer services have corresponding services in 	IP and in TCP?

6.a) 	Explain the concept of virtual circuit and datagram.

b) 	Explain interior gateway protocol and exterior gateway protocol with an   	example.

7.a)	Derive Jacobson’s relation for dynamic estimation of RTT in TCP timer 	management.

b)	Discuss the problem of dynamic estimation of RTT and a solution proposed by 	Phil Karn.

c)	Mention and define the different timers used by TCP.

8.a)	Discuss the techniques of Cryptographic transformation.

b)	Define P and S-Boxes and indicate how they are used in Data Encryption Standards (DES).

-*-*-*-