Code No: 411701

Set No.

1

IV B.Tech I–Semester Regular Examinations, November-2004

CELLULAR AND MOBILE COMMUNICATION

(Electronics and Telematics)

Time: 3 hours							Max. Marks: 80

Answer any Five questions

All questions carry equal marks

---

1.a)	Explain the limitations of conventional mobile system? How are they overcome by cellular system?

b)	Briefly explain the cell shape and handoff?

2.a)	With help of neat diagrams, explain the N cell reuse pattern for four and seven cell reuse?

b)	Derive the co- channel interference factor for 7 cell reuse pattern?

3.a)	Compare the co-channel interference performance of a directional antenna system for K=7 and K = 4?

b)	What is tilting antenna? How can these antenna patterns reduce the co-channel 	interference?

4.a)	Derive the equation for the Feher’s delay spread in mobile environment?

b)	What is the delay spread bound Tmax of a 220 MHz public land line mobile 	radio system.

If Pt = 1W ( +30dBm) and Prmin = - 90 dBm, How much Tmax if  the sensitivity of the receiver is improved to Prmin =  -  100dBm Why does increased sensitivity of increased system gain Gs = Pt Prmin lead to a higher delay spread bound?

5.a)	Classify the mobile unit antennas?

b)	Explain about the sectorised cells?

6.	Present the frequency management of cellular systems. for efficient spectrum 	utilization and increases the number of channels?

7.a)	Explain clearly how to calculate  and    for single cell?

b)	Why hand off is necessary for cellular systems. Determine the two types of 	handoffs based on signal strength and C/I ratio?

8.	Present the concept of the narrow beam sector. How does it increase the traffic 	capacity? Consider K = 7.

@&@&@

Code No: 411701

Set No.

2

IV B. Tech I – Semester Regular Examinations, November/2004

CELLULAR AND MOBILE COMMUNICATION

(Electronics and Telematics)

Time: 3 hours							Max. Marks: 80

Answer any Five questions

All questions carry equal marks

---

1.a)	Explain the direct wave  path, line of sight path, out of sight path, and obstructive path?

b)	What are the different types of noises in cellular frequency ranges. Explain in detail?

c)	Explain about the marketing of hexagonal cells?

2.a)	Derive the maximum number calls per hour per cell and the maximum number frequency channels per cell . How are they related?

b)	Find the desire co-channel interference phenomena in a reuse cellular system 	that employs omni directional antenna system?

3.a)	Explain about the co-channel and adjacent channel interference?

b)	How many users can be supported for 0.3% blocking probability for the 	following number of trunked channels in a blocked calls clear system?

i) 2  ii) 	5  iii) 25.  Assume each user generates a 0.2% E of traffic

4.a)	Explain about the small scale multi path propagation?

b)	Consider a transmitter which radiates a sinusoidal carrier frequency of 	1850MHz for a vehicle moving 60mph. Compute the received carrier 	frequency if the mobile is moving a) Direction towards the transmitter b) 	directly away from the transmitter c) in a direction which is perpendicular to 	the direction of the arrival of the transmitting signal?

5.a)	Classify the cell site antennas and describe these in detail?

b) 	What do you understand by an engineering antenna pattern? Explain the 	corresponding pattern?

6.a)	Describe the grouping of the voice, setup and paging channels?

b)	Present the reuse partition scheme in overlaid cell system. Mention the 	advantages associated with it?

7.a)	Explain the queuing handoff mechanism?

b)	If the number channels at the cell site N=70, the call holding time is 101S.  	The number of originated calls per hour expressed as  is 2270. The number 	of handoff calls attempted per hour is expressed as 2 is 80.  Find the 	probability of the queuing handoff calls but not the originated calls . Also find 	the probability of the queuing the originated calls but not the handoff calls?

8.	What do you mean by operational techniques? Why are these needed in 	cellular systems? Explain briefly different operational techniques.

@&@&@

Code No: 411701

Set No.

3

IV B. Tech I – Semester Regular Examinations, November/2004

CELLULAR AND MOBILE COMMUNICATION

(Electronics and Telematics)

Time: 3 hours							Max. Marks: 80

Answer any Five questions

All questions carry equal marks

---

1.a)	Distinguish between the landline telephone network and cellular telephone network?

b)	Explain the phenomena of severe fading?

2.a)	Explain the general description of the in mobile environment?

b)	If the maximum calls per hour Qi in one cell be 3000 and an average calling 	time T be 1.76min. The blocking probability is 2%.   Find out the offered  	load A. For the above Qi is 28000 what is the offered load and find the 	required channels to the above. Finally compare the two systems (use Erlang 	B model).

3.a)	Explain the concept of the delay time t concern to the real time co-channel interference?

b)	Give the idea about the directional antenna system?

4.a)	Determine the maximum and minimum spectral frequency received from a stationary transmitter has a central frequency of exactly 1950MHz.  Assume that the receiver is traveling at speeds of a)1Km  b) 5Km  c) 10Km  d)100Km?

b)	Describe all physical circumstances that relate to a stationary transmitter and a moving receiver such that the Doppler shift at the receiver is equal to

a) 0Hz b) fdmax c)  -fdmax  d) fdmax/2.

5.a)	Write short notes on roof mounted antennas?

b)	Explain about the necessity of diversity reception in mobile antennas?

6.a)	What do you understand by non fixed channel assignment? Describe the corresponding algorithms?

b)	Explain in detail access channels and operational techniques?

7.a)	How do you find the values of  and  related to the cell?

b)	How do you reduce the dropped call rate and explain?

8.a)	As a mobile unit in a communication with base station moves, what factors determine the need for power control and the amount of power adjustment?

b)	Explain the difference between the open loop and closed loop power control.

@&@&@

Code No: 411701

Set No.

4

IV B. Tech I – Semester Regular Examinations, November/2004

CELLULAR AND MOBILE COMMUNICATION

(Electronics and Telematics)

Time: 3 hours							Max. Marks: 80

Answer any Five questions

All questions carry equal marks

---

1.a)	What do you mean by Mean Opinion Score and explain in detail?

b)	Explain the history of 800MHz spectrum allocation to cellular systems?

2.a)	What do you mean by desired C/I  and Explain?

b)	Compare interference from first tier of six interferes with that from twelve 	interferes of second tier (including first and second tiers).

3.a)	Explain the importance of the notch in the tilted antenna pattern to reduce the co-channel interference?

b)	What are the conditions on tilting antennas?

4.a)	From the free space propagation model derive the equation for received 	power?

b)	If a transmitter produces 50W of power express the transmitter power in units of (i) dBm  (ii) dBW, if 50W is applied to a unity gain antenna with a 900MHz carrier frequency find the received power in dBm at a free space distance of 100m from the antenna. What is Pr (10Km)? Assume unity for the receiver antenna?

5.a)	Explain the important effects of cell site antenna height?

b)	Explain top mounted antennas in mobile systems?

6.a)	Distinguish clearly the channel assignment to cell sites and the mobile units?

b)	Explain the concept of the setup channels?

7.a)	How hand offs are prioritized? Explain.

b)	Discuss the practical hand off considerations. What is meant by cell dragging?

8.	Explain the following special features in detail.

i) SMS		ii) MMS	   iii) EMS	iv) Call waiting

v) Call forwarding   	vi) Call diverting    	vii) Voice storage box

viii) Call re routing.

@&@&@