Code No: 412103

Set No.

1

IV B.Tech. I–Semester Regular Examinations, November-2004.

THEORY OF VIBRATIONS

(Aeronautical Engineering)

Time: 3 hours							Max. Marks: 80

Answer any FIVE questions

All questions carry equal marks

---

1.a)	Define simple harmonic motion and various terms associated with it.

b)	Find the equation of motion of a mass attached to a spring and oscillating in 	vertical direction.

2.	Find the expression for the displacement of a disc attached to the end of a 	circular shaft whose other end is fixed.  The disc is oscillating in horizontal 	plane and is coaxial with the shaft.

3.	A gun barrel weighing 600 kg has a recoil spring of 350 kg/cm.  If the barrel 	recoils one meter on firing find

a) The initial recoil velocity of the gun.

b) The critical damping coefficient of a dash pot which is engaged at the end

of recoil stroke.

c) The time required for the barrel to return to a position 5 cm from its initial

position.

4.	Explain the working of a vibrometer.  A vibrometer having a natural 	frequency of 31.4 rad/sec is employed to measure the vibration of a machine 	part.  If it gives a reading of 0.15 cm., what is the amplitude of vibration of the 	part?  Operating frequency is 200 rad/sec.  Assume no damping.

5.	What are the generalized coordinates write the equations of motion of two 	degree of freedom system by using Lagrange’s equations.

6.	Figure shows an electrical motor-generator set.  Find the natural frequencies 	and amplitude ratios of the principal modes.

Contd….2

Code No: 412103				-2-				Set No:1

7.	Three bogies are connected by two springs of stiffness 40  105 N/m each.  	The mass of each bogey is 20  103 kg.  Determine the frequencies of 	vibration.  Neglect friction between the wheels and rails.

8.a)	Explain the phenomenon of galloping of the transmission lines.

b)	What is control reversal discuss the factors affecting it.

- - -

Code No: 412103

Set No.

2

IV B.Tech. I–Semester Regular Examinations, November-2004.

THEORY OF VIBRATIONS

(Aeronautical Engineering)

Time: 3 hours							Max. Marks: 80

Answer any FIVE questions

All questions carry equal marks

---

1.	Discuss the energy method to find the equation of motion of a system. Use this 	method to find the equation of motion of a simple pendulum.

2.	What is Trifilar suspension, derive the expression to find out the moment of 	inertia of an object by this method.

3.	A shaft of negligible weight 6 cm diameter and 5 meter long is simply 	supported at the ends and carries four weights 50 kg each at equal distances 	over the length of the shaft.  Find the frequency of vibration by Dunkerley’s 	method .  Take E = 2  106 kg/cm2.

4.	With the help of a neat sketch explain the working of ring torsional vibration 	absorber.

5.	Using Matrix iteration method find the natural frequencies of the system 	shown in the figure:

m1 = 2m	k1 = 2k

m2 = 2m	k2 = k

m3 = m	k3 = k

6.	Determine the natural frequency of the above shown system shown in Q.5 by 	Stodola’s method.

m1 = m2 = m3 = m

k1 = k2 = k3 = k

Contd…..2

Code No: 412103				-2-				Set No:2

7.a)	Define aeroelasticity and discuss the phenomenon of wing divergence.

b)	Draw and explain Coller’s triangle.

8.	An electric motor of mass 250 kg with an unbalance of 10-2 kg is mounted on 	four rubber isolators which have a material loss factor of 0.5.  The rubber 	isolator deflect by 2.5 mm due to self load of the motor.

a) What will be the amplitude of the motor at the operating speed of 990 rpm.

b) What will be the force transmitted to the ground at the operating speed.

- - -

Code No: 412103

Set No.

3

IV B.Tech. I–Semester Regular Examinations, November-2004.

THEORY OF VIBRATIONS

(Aeronautical Engineering)

Time: 3 hours							Max. Marks: 80

Answer any FIVE questions

All questions carry equal marks

---

1.	Describe the various methods for the analysis of an oscillating system and 	find the equations of motion.

2.	What is damping, explain the importance of the critical damping and find the 	expression for the logarithmic decrement.

3.	An electric motor of mass 250 kg with an unbalance of 10-2 kg is mounted on 	four rubber isolators which have a material loss factor of 0.5.  The rubber 	isolator deflect by 2.5 mm due to self load of the motor.

(a) What will be the amplitude of the motor at the operating speed of 990 rpm.

(b) What will be the force transmitted to the ground at the operating speed.

4.	Calculate the natural frequency of a shaft of diameter 10 cm and length       	300 cm carrying two discs of diameters 125 cm and 200 cm respectively at its 	ends weighing 480 kg and 900 kg respectively.  Modulus of rigidity of the 	shaft may be taken as 2  106 kgf/cm2.

5.	With the help of a neat sketch explain the working of centrifugal pendulum 	vibration absorber.

Contd…..2

Code No: 412103				-2-				Set No:3

6.	Using Matrix iteration method find the natural frequencies of the system 	shown in the figure:

m1 = 2m	k1 = 2k

m2 = 2m	k2 = k

m3 = m	k3 = k

7.	Determine the natural frequency of the above system shown by Stodola’s 	method.

m1 = m2 = m3 = m

k1 = k2 = k3 = k

8.a)	Define aeroelasticity, draw and explain Coller’s triangle.

b)	Explain the phenomenon of galloping of transmission lines.

- - -

Code No: 412103

Set No.

4

IV B.Tech. I–Semester Regular Examinations, November-2004.

THEORY OF VIBRATIONS

(Aeronautical Engineering)

Time: 3 hours							Max. Marks: 80

Answer any FIVE questions

All questions carry equal marks

---

1.a)	What is D’Alembret’s Principal how it is used for the analysis of an 	oscillating system.

b)	Use the above principal to analyse any single degree of freedom system.

2.	What is Bifilar suspension. Derive the expression to find out the moment of 	inertia of an object by this method.

3.	A weight of one kilograms attached to spring having a stiffness 4 kg/cm.  The 	weight slides on a horizontal surface, the coefficient of friction between the 	weight and surface being 0.1.  Determine the frequency of vibration of the 	system and amplitude after one cycle if the initial amplitude is 0.25 cm.  	Determine the final rest position.

4.	A vibratory system performs the motions as expressed by the following 	equations:

X⋅+800X+90θ=0θ⋅+800θ+90X=0

If the system is turned through 1.5 radians and released, find the frequencies 	and mode shapes.

5.	State orthogonality principal and derive the required equations.

6.	Three bogies are connected by two springs of stiffness 40  105 N/m each.  	The mass of each bogey is 20  103 kg.  Determine the frequencies of 	vibration.  Neglect friction between the wheels and rails.

7.	A shaft of negligible weight 6 cm diameter and 5 meter long is simply 	supported at the ends and carries four weights 50 kg each at equal distances 	over the length of the shaft.  Find the frequency of vibration by Dunkerley’s 	method.  Take E = 2  106 kg/cm2.

8.	What is aeroelasticity, discuss the various phenomenon related to it which 	affect the performance of a flying aircraft.

- - -