Code No 410303

Set No.

1

IV B.Tech I–Semester Regular Examinations, November-2004

PRODUCTION TECHNOLOGY-III

(Mechanical Engineering)

Time: 3 hours							Max. Marks: 80

Answer any Five questions

All questions carry equal marks

---

1.a)	Discuss the geometrical considerations in rolling process.

b)	Discuss the features of flat rolling practice.

2.a)	Describe the difference between a compound die and progressive die.

b) 	What is stretch forming? Explain its necessities with a neat sketch.

3.a)	Assume that you are reducing the diameter of two rods, one by simple tension 	and the other by indirect extrusion. Which method will require more force? 	Why?

b)  	Calculate the extrusion force for a round billet of 150 mm diameter, made of 	beryllium and extruded at 1000 oC  to a diameter of 40 mm (assume suitable 	data, if necessary)

c)  	How does extrusion differ from forging?

4.a) 	State the advantages of both mechanical and hydraulic presses for press 	forging  applications.

b)	Compare hammer and press forging operations

5.a) 	Explain  the  concept  of  Limit  gauging, taking the example of gauging of

holes and shafts. Bring out the advantages and disadvantages of using limit

gauges.

b)	Discuss about the tolerance and wear allowances on limit gauges.

6.a) 	What do you understand by the term ‘Airy  points’  State  the  condition to 	achieve it.

b)	Distinguish between Primary, Secondary, Tertiary and working standards of

length.

c)	Discuss the procedure for comparison of an end gauge with a line  standard by 	displacement method.

7.a)	Describe a method to find out the flatness of a surface plate.

b)	Discuss the method for testing for straightness by using spirit level and auto-collimator.

8.a)	Explain how the various elements of internal threads are measured using a 	pitch measuring machine

b)	How is Taylor’s principle of limit gauging applicable to gauging of screw

threads?

@&@&@

Code No 410303

Set No.

2

IV B. Tech I – Semester Regular Examinations, November/2004

PRODUCTION TECHNOLOGY-III

(Mechanical Engineering)

Time: 3 hours							Max. Marks: 80

Answer any Five questions

All questions carry equal marks

---

1.a)	What are the defects in rolled products? Explain the causes and remedies.

b)	What is tandem rolling operations? Explain.

2.a) 	A circular disc with a square hole is to be made of a sheet metal strip. Describe

with sketch a die for making this component.

b)	With reference to presses, discuss the following:

i)	Shut height      ii) 	Bolster plate    iii) 	Stripper     iv)	Knock out

3.a)  	Explain the forces in extrusion process.

b)  	Determine the maximum force required for extruding a cylindrical aluminium 	billet of 25mm diameter and 50 mm length to a final diameter of 5 mm. 	Tensile yield stress for aluminium is 170 N/mm2. also calculate the power loss 	in friction.

4.a)	Name the common materials used for forging.  Give the correct forging 	temperature of some of the common forgeable metals.

b) 	Describe in detail the process making a cold chisel using smith forging	with rough sketches

5.a) 	Explain Taylor’s Principle of limit gauging, with reference to gauging of,

say, rectangular holes. Discuss clearly the effect of violating the Taylor’s

Principle.

b) 	Sketch and explain any four types of plug gauges.

6.a)	With the help of sketches explain the constructional details and working of a

dial  gauge.   Clearly  indicate  in  your  answer,  how  the  magnification  is

achieved.

b) 	What characteristics you expect of a good dial indicator.

c) 	Outline the uses of a dial gauge.

7.a)	Discuss the main advantages of using optical instruments in Linear measurements.

b)	Explain with the help of neat sketch the working principle of Tool maker’s microscope.

8.a)	Compare two-wire and three-wire methods of measuring the effective diameter of a screw

b) 	Explain the method of checking the ‘thread form’ and ‘angle’ by optical projection of a thread

@&@&@

Code No 410303

Set No.

3

IV B. Tech I – Semester Regular Examinations, November/2004

PRODUCTION TECHNOLOGY-III

(Mechanical Engineering)

Time: 3 hours							Max. Marks: 80

Answer any Five questions

All questions carry equal marks

---

1.a)	Discuss ring rolling operation in detail.

b)	Write a note on thread rolling process.

2.a)	With a neat sketch explain blanking die and bending die.

b) 	What are the different operations done in presswork?

3.a)	Extrusion ratio, speed and temperature all affect the extrusion force. Explain 	why do they?

b) 	Discuss the advantages of extrusion process.

4.	Explain the following with suitable sketches.

a) 	Rotary forging		b) Wobble forging

5.a) 	What are snap gauges ? What are its advantages over ring gauges?

Explain.

b) 	With the help of neat sketches, explain why a GO gauge should be a full

form gauge, whereas a NO GO gauge should incorporate minimum metal

limit of a single dimension at a time.

c) 	With neat sketches explain:

(i) 	A Progressive type of solid plug gauge.

(ii) 	A taper plug gauge.

6.a) 	With the help of sketches explain the working of an external micrometer.

b) 	Discuss the function of ratchet drive in case of a micrometer.

c) 	Describe a differential screw micrometer.

7.a)	What are important uses of tool makers microscope.

b)	Explain the procedure for testing the dimension of a slip gauge with the help of a reference gange and a pair of optical flats.

8.a) 	With a neat sketch illustrate how the effective diameter of a screw thread may 	be checked using the three-wire method

b) 	What is the ‘best size’ wire for the above case?

@&@&@

Code No 410303

Set No.

4

IV B. Tech I – Semester Regular Examinations, November/2004

PRODUCTION TECHNOLOGY-III

(Mechanical Engineering)

Time: 3 hours							Max. Marks: 80

Answer any Five questions

All questions carry equal marks

---

1.a)	What are various cross sections that can be produced by rolling process?

b)	Discuss the importance of cold rolling?

2.a)	List the characteristics of sheet metals that are important in sheet forming

operation .

b)	Classify the sheet metal operations based on stresses induced.

3.a)  	Describe the consequences of extruding a metal above and below its specified 	temperature range of extrusion

b) 	How the die angle affects the extrusion process? Explain.

4.a) 	What are the advantages and limitations of using die inserts in forging.

b)	How would the temperature of a hot work piece change as it is forged, using 	room temperature dies, in (i) slow acting hydraulic press and (ii) drop 	hammer?  Explain

5.a) 	Explain in detail the two line standards namely:

(i)  Imperial standard yard

(ii) International Prototype Meter

b) 	Discuss the need for wave length of light as a standard for length.

c) 	Indicate clearly the various wave length standards.

6.a) 	Discuss  in  detail  two  methods  of   testing   parallelism   of   micrometer    	measuring faces.

b) 	With the help of sketches describe a vernier type micrometer.  How do you

calculate its least count.

7.a)	Discuss the following terms in connection with surface finish measurement.

i)  Wariness	(ii)  Lay	(iii)  Roughness      	iv)  Centre line profile

b)	In the measurement of roughness, the heights of 16 successive peaks and 	troughs were measured from a datum and were 18, 42, 25, 35, 22, 36, 18, 42, 	22, 32, 24, 36,16, 38, 23, 40 microns.  If these measurements were obtained 	over a length of 30 mm, determine the following values

i)  Ra		(ii)  Rz		(iii)  RMS

8.	What is projection comparator? Show a sketch to illustrate the principle and 	discuss the types of work for which this instrument is specially suitable

@&@&@