OR

Code No:  411052

IV B.Tech. I-Semester Supplementary Examinations, November-2004

## DIGITAL INSTRUMENTATION

(Electrical and Instrumentation Engineering)

Time: 3 hours							    Max Marks: 70

## Answer any Five questions

All questions carry equal marks

- - -

1.a)	Give the differences between analog DAS and digital DAS.

- Explain the block diagram approach of Data Acquisition System with neat sketch.

2.a)	Discuss the significance of DAS.

- Give the operation and advantages of S/H device with neat sketch.

- Explain briefly about line drivers and analog switches.

3.a)	Compare S/H DAC with Shannon-Rock DAC.

- Explain specification & errors in D/A converters.

4.a)	Give various types of Analog to Digital Conversion techniques and explain any one of these with a neat diagram.

- A  4½ digital multimeter is used for voltage measurement.

  - Find its resolution.

  - How would 12.98 V be displayed on 10 V range.

  - How would 0.75 V be displayed on 1 V range.

  - How would 0.75 V be displayed on 10 V range.

5.a)	Explain the operation of logarithmic A/D converters with neat sketch.

- What are the differences between digital multimeter and digital panel meter?

6.a)      Explain the serial interfacing using Rs 232C- signaling levels.

- Write an ALP to perform successive Approximation 8 bit A/D conversion for

8085 cpu.

7.a)	What is the significance of digital signals over analog signals and explain the conversion of non-linear systems to linear systems by using discretization.

- Explain the basic components of magnetic tape recorder.

8.a)	Explain the advantages and disadvantages of ink and inkless recorders.

b)	Explain serial digital recoding using randomizing technique.

^*^*^