Code No.: 420352

OR

IV B-tech II-Semester Examination April, 2003

HYDRAULIC MACHINERY AND SYSTEMS

(common to Mechanical Engg., Production Engg. and Mechatronics)

Time: 3 hours							Max. Marks: 70

Answer any five questions

All questions carry equal marks

- - -

1.a)	What do you mean by jet propulsion of ship? Derive expression for efficiency of 	propulsive of ship when the inlet orifices are normal to the direction of motion of 	the ship.

b)	A jet propelled boat is to cruise with a maximum velocity of 4 m/sec. Area of the 	jet propelled is 50 cm2 and absolute velocity of jet at maximum speed is 5m/sec. 	Determine the 	power of the engine required to run the pump and propulsion 	efficiency of the maximum 	speed of the boat (a) when the boat takes water 	amidships, (b) the boat takes water in the direction opposite to the boat.

2.a)	Describe with neat sketches the different types of draft tubes. Why draft tube is 	necessarily be used in reaction turbine?

b)	A propeller turbine runner has an out let diameter of 5m and inner diameter 2m 	and develops 	25000 H.P. when running at 120 rpm. Under a head of 20m. The 	hydraulic efficiency is 95% 	and overall efficiency is 85% find the discharge 	through the turbine and guide blade angle at 	in let.

3.a)	What do you mean by characteristics curves of turbines? Draw with neat diagrams 	the characteristic curve for Pelton wheel.

b)	A hydraulic turbine, under a head of 25m develops 9500 H.P. running at 110 rpm. 	What is the specific speed of a turbine? What type of turbine is this? Find also the 	normal speed and output if the head on the turbine is reduced to 20m

4.a)	Define indicator diagram. How will you prove that area of indicator diagram is 	proportional to work done by the reciprocating pump?

b)	A single acting reciprocating pump has cylinder diameter 15cm and stroke length 	30cm. The centre of the pump is 4m above the water surface in the sump. The 	atmospheric pressure 	head is 10.3 m of water and the pump running at 40 rpm. If 	the length and diameter of a 	suction pipe are 5m and l0cm respectively. 	Determine the pressure head due to acceleration 	in the cylinder (i) At the 	beginning of suction stroke (ii) in the middle of suction stroke.

Contd…2.

Code No:420352				:: 2 ::					OR

5.a)	Draw a neat diagram of inlet and out let triangle velocities and derive expression 	for work done with the usual notation.

b)	A centrifugal pump is to discharge 0.120 m3/sec at a speed of 1500 rpm against a 	head of 25cm diameter and its width at out let is 5cm and manometric efficiency 	is 75%. Determine the vane angle at outer periphery of the impeller.

6.a)	What is specific speed of an centrifugal pump? Derive expression for specific 	speed in terms of power.

b)	Two geometrically similar pumps are running at the same speed 1000 rpm. One 	pump has an 	impeller diameter 0.3 m and lifts water at the rate of 20 lit/see 	against head of 15m. 	Determine head and impeller diameter of the other pump to 	deliver half the discharge.

7.a)	Define the term hydraulic accumulator. Obtain the expression for the capacity of 	hydraulic accumulator.

b)	The water is supplied at a pressure of 1.5 kg/cm2 to an accumulator having a ram 	of diameter 2m, if the total lift of ram is 10m. Determine (i) capacity of 	accumulator and (ii) total weight placed on the ram (including weight of ram)

8.	Write notes on the following:

a) Transmission of power through pipe.

b) Gear and bane pump.

c) Hydraulic circuits.

~ ~ ~ ~