For 2002 Batch

Set No.

1

Code No:RR-10205

I.B.Tech. (Regular) Examinations April/May 2003

ELECTRONIC DEVICES AND CIRCUITS

(common to Electrical and Electronics Engineering, Electronics and Communications Engineering, Computer Science and Engineering, Electronics and Instrumentation Engineering, Bio-Medical Engineering, Computer Science and Information Technology, Electronics and Control Engineering, Computer Science and Systems Engineering, Electronics and Telematics, Electronics and Computer Engineering, Instrumentation and Control  Engineering)

Time: 3 hours							       Max.Marks: 80

Answer any FIVE questions

All questions carry equal marks

- - -

1.a)	An electron is moving perpendicular to magnetic field ‘B’. Derive the expression 	for radius ‘R’ of the trajectory and period of rotation T

b)	Derive the expression for the electro magnetic deflection sensitivity in the 	case of 	the CRT.

2.a)	With the help of necessary graphs and sketches explain the potential distribution 	in an open circuited p-n junction.

b)	Explain about Forward bias and Reverse bias in the case of a p-n junction diode.

3.a)	Define the following terms of a half wave rectifier with resistive load.

i) Ripple factor.	ii) Peak inverse voltage.	iii) Rectification Efficiency.

b)	A 230 V, 60Hz voltage is applied to the primary of a 5:1 step down, center tapped                        	transformer used in a full wave rectifier having a load of 900 Ω.If the diode 	resistance and the secondary coil resistance together has a resistance of 100 Ω, 	determine

i) dc voltage across the load.

ii) dc current flowing through the load.

iii) dc power delivered to the load.

iv) PIV across each diode

v) Ripple voltage and its frequency.

4.a)	Explain the input and output characteristics of a transistor in CB configuration.

b)	Draw the circuit diagram of an NPN   junction transistor in CE configuration and      	describe   its characteristics.

c)	Define active, saturation and cut-off regions and saturation resistance of a CE   	transistor.

5.a)	From the static characteristics how to obtain quiescent voltage and current using 	load line for JFET.

b)	Explain how FET can be used as a switch.

Contd…2

Code No.: RR-10205				-2-				    Set No.1

6.a)	Define the stability factors, S', S'', and what is the need of this in BJT circuits.

b)	Draw the circuit diagram of a self bias BJT circuit and how to determine the 	values of R1 and R2.

7.a)	Explain the concept of feedback as applied to electronic amplifier circuits. What 	are the advantages and disadvantages of positive and Negative feedback.

b)	With the help of general block diagram explain the term feedback.

c)	Define the following terms in connection with feedback.

i)	Return difference feedback

ii)	Closed loop voltage gain

iii)	Open loop voltage gain.

8.a)	What type of feedback is employed in oscillators? And what are its advantages. Discuss the conditions for sustaining oscillations.

b)	Find the capacitor C and hfe for the transistor to provide a resonating frequency of 10kHz of a transistorized phase-shift oscillator. Assume R1=25k, R2=60k, Rc=40k, R=7.1k and hie=1.8k.

^^^

For 2002 Batch

Set No.

2

Code No:RR-10205

I.B.Tech. (Regular) Examinations April/May 2003

ELECTRONIC DEVICES AND CIRCUITS

(common to Electrical and Electronics Engineering, Electronics and Communications Engineering, Computer Science and Engineering, Electronics and Instrumentation Engineering, Bio-Medical Engineering, Computer Science and Information Technology, Electronics and Control Engineering, Computer Science and Systems Engineering, Electronics and Telematics, Electronics and Computer Engineering, Instrumentation and Control  Engineering)

Time: 3 hours							       Max.Marks: 80

Answer any FIVE questions

All questions carry equal marks

- - -

1.a)	Derive the expression for trajectory of an electron placed in combined electric(E) and magnetic fields(B). Both the fields are perpendicular to each other and the initial velocity is zero.

b)	The magnetic flux density B=0.02wb/m2 and electric field strength E=105 v/m are 	uniform fields, perpendicular to each other.A pure source of an electron is placed 	in a field. Determine the minimum distance from the source at which an electron 	with 0v will again have 0v in its trajectory under the influence of combined  	Electric and magnetic fields.

2.a)	Explain the term Transition Capacitance CT of a  p-n junction diode.		              b)	Derive the expression for transition capacitance CT of a diode.

3.a)	Define the following terms

i) Transformer utilization factor.

ii) Form factor and ripple factor of a HWR with resistive load and derive the 	expression	for the same.

b)	 A HWR has a load of 3.5 KΩ .If the diode resistance and secondary coil 	resistance together have a resistance of 800 Ω and the input voltage has a signal 	voltage of peak value 240 V, calculate

i) Peak, average and rms value of current flowing.

ii) dc power output.

iii) ac power input .

iv) Efficiency of the rectifier.

4.a)	With the help of a neat diagram show different current components in a transistor.

b)	Why  is CE configuration is  most widely used ?

c)	Calculate the values of Ic and IB for a transistor with  αdc = 0.99 and ICBO = 5μA , 	IB is measured as 20 μA.

Contd…2

Code No.: RR-10205				-2-				    Set No.2

5.a)	For a MOSFET if VGS varies from –ve to +ve voltage draw the transfer 	characteristics and mention modes of operation.

b)	Sketch the drain characteristics of MOSFET for different values of VGS and mark 	different regions of operation.

6.a)	What is meant by thermal runway briefly explain.

b)	What is the condition for thermal stability.

c)	An npn transistor if  = 50 is used in common emitter circuit with Vcc = 10V and 	Rc = 2k. The bias is obtained by connecting 100k resistance from collector to 	base. Find the quiescent point and stability factors.

7.a)	Draw the circuit diagram of a voltage shunt feedback using BJT and derive 	expression for voltage gain with feedback.

b)	What are the advantages and disadvantages of negative feedback.

c)	Calculate the gain input impedance, output impedance of voltage series feedback 	amplifier having A=-300 RI=1.5K RO=50K and =-1/20.

8.a)	Classify various oscillators based on O/P waveforms circuit components, operating frequencies and feedback used.

b)	A phase shift oscillator is to be designed with FET having gm=5000s, rd=4k while the resistance in the feedback circuit is 9.7k. Select the proper value of C and RD to have the frequency of oscillations as 5kHz.

^^^

For 2002 Batch

Set No.

3

Code No:RR-10205

I.B.Tech. (Regular) Examinations April/May 2003

ELECTRONIC DEVICES AND CIRCUITS

(common to Electrical and Electronics Engineering, Electronics and Communications Engineering, Computer Science and Engineering, Electronics and Instrumentation Engineering, Bio-Medical Engineering, Computer Science and Information Technology, Electronics and Control Engineering, Computer Science and Systems Engineering, Electronics and Telematics, Electronics and Computer Engineering, Instrumentation and Control  Engineering)

Time: 3 hours							       Max.Marks: 80

Answer any FIVE questions

All questions carry equal marks

- - -

1.a)	Give the block diagram of CRO and explain about each block in detail?

b)	In a electrostatic deflecting CRT the length of the deflection plats is 2 cm, and spacing between deflecting is 0.5cm, The distance from the centre of the deflecting plate to the screen is 20 cm, the deflecting voltage is 25V Find the deflection sensitivity, the angle of dection and velocity of the beam. Assume Final Anode potential is 1000V.

2.a)	Explain about grown and alloy junctions in the case of a p-n junction diode.	         b)	Determine the forward resistance of a p-n junction diode, when the forward 	current is 5mA at T=300o  K. Assume silicon diode.

3.a)	Describe the action of a full wave bridge rectifier. With the aid of input-output 	wave forms.

b)	What are the advantages of bridge rectifier over center trapped transformers?

c)	In a Bridge rectifier, the transformer is connected to 220 V, 60 Hz mains and the 	turns ratio of the step down transformer is 11:1. Assuming the diode is ideal, find

i) Idc.

ii) Voltage across the load.

iii) PIV.

4.a)	Define the following terms and explain .

i)	Emitter efficiency.

ii)	Transport factor.

iii)	 Large signal current gain.

b)	The reverse leakage current of the transistor when connected in CB configuration 	is 0.2 μA while it is 18 μA when the same transistor is connected in CE 	configuration . Calculate  αdc and  βdc of the transistor.

c)	If  αdc = 0.99 and  ICBO = 50 μA , find Emitter current.

Contd…2

Code No.: RR-10205				-2-				    Set No.3

5.a)	Give symbol of UJT and mark required polarities for operation..

b)	Give the equivalent circuit of UJT.

c)	Explain how the UJT can be used as a –ve resistance device with the aid of static 	characteristics.

6.a)	Draw the circuit diagram of a self bias circuit and derive expression for S. Why it 	is widely used.

b)	How to obtain quiescent point graphically for a given transistor amplifier of CE 	configuration- explain.

7.a)	For the given circuit find Av, Rif and also the resistance seen by Vs, Assume 	Rc = 4K, R' = 40K, RS = 10K and the transistor parameters are hie=1.1K hfe=50 	hre=0 1hoe=40K.

(Assume AV>>1)

b)	The gain of an amplifier is decreased to 1000 with –ve feedback from its gain of 	5000. Calculate the feedback factor and the amount of negative feedback in dBs.

8.a)	Show that the gain of Wien bridge oscillator using BJT amplifier must be at least 3 for the oscillations to occur.

- In a transistorized Hartley oscillator the two inductances are 2mH and 20H 	while the frequency is to be changed from 950kHz to 2050kHz. Calculate the 	range over which the capacitor is to be varied.

^^^

For 2002 Batch

Set No.

4

Code No:RR-10205

I.B.Tech. (Regular) Examinations April/May 2003

ELECTRONIC DEVICES AND CIRCUITS

(common to Electrical and Electronics Engineering, Electronics and Communications Engineering, Computer Science and Engineering, Electronics and Instrumentation Engineering, Bio-Medical Engineering, Computer Science and Information Technology, Electronics and Control Engineering, Computer Science and Systems Engineering, Electronics and Telematics, Electronics and Computer Engineering, Instrumentation and Control  Engineering)

Time: 3 hours							       Max.Marks: 80

Answer any FIVE questions

All questions carry equal marks

- - -

1.a)	With the help of necessary equations show the   trajectory of an electron is cycloid when it is placed in perpendicular electric and magnetic fields.

b)	Determine the velocity and kinetic energy of an electron accelerated through 	potential of 3kV.

2.a)	What are three approximations of forward biased semiconductor diode. Explain 	with neat sketches and equivalent circuits.

b)	The expression for the breakdown voltage of a Zener diode is given as  	Vz=Ez2/2eNA with usual notation. Assuming that NA  <<ND, prove that the 	breakdown Voltage for a Germanium Diode is 51\σp  if EZ = 2* 107 V/m.

3.a)	Explain  the following terms.

i) Ripple factor	ii) Peak inverse voltage	iii) Efficiency

iv) Transformer utilization factor	  v) Form factor	 vi) Peak factor.

b) 	A half wave rectifier has a load of 3.5 KΩ. If the diode resistance and the    	secondary coil resistance together have a resistance of 800Ω and the input voltage 	has a signal voltage of  240 V, calculate

i) Peak, average and rms value of current flowing.

ii) dc power output.

iii) ac power input .

iv) Efficiency of the rectifier.

4.a)	Draw the input and output  characteristics of transistor in CE configuration  with 	regions of operation and explain .Show how h-parameters can be determined 	graphically

b)	Derive the relation between α and β .

Contd…2

Code No.: RR-10205				-2-				    Set No.4

5.a)	Draw the firing characteristics of SCR and briefly explain

b)	Define the following with respect to SCR

i)	Forward break over Voltage

ii)	Reverse break over voltage

iii)	Holding current

iv)	Gate trigger current.

6.a)	Draw the circuit diagram of small signal CE amplifier circuit and give its 	equivalent hybrid model. What is the role of Cc and Ce.

b)	Obtain frequency response of CE amplifier circuit and find out its band width. 	What is the impact of Cc and Cs on the band width?

7.a)	Draw the circuit diagram of voltage series feedback and derive expressions for 	input resistance and output resistance.

b)	For the circuit shown below hfe = 100, hie=1K and other two parameters are 	negligible if Re = 1K. Find the value of Av = V0Vi

AVS=V0VS , Rif = Rof

8.a)	Derive an expression for frequency oscillation of transistorized Colpilts oscillators.

- A quartz crystal has the following constants. L=50mH, C1=0.02PF, R=500 and 	C2=12PF. Find the values of series and parallel resonant frequencies. If the 	external capacitance across the crystal changes from 5PF to 6PF, find the change 	in frequency of oscillations.

^^^