Code No: 420251

OR

IV- B.Tech. II Semester Examination April, 2003

UTILIZATION OF ELECTRICAL ENERGY

(Electrical and Electronics Engineering)

Time: 3 Hours							Max. Marks: 70

Answer any Five questions

All questions carry equal marks

- - -

1.a)	Explain the laws of Illumination.

b)	A hall 30m long and 12m wide is to be illuminated and illumination required is 50 	metre 	candles (lux). Compute the number, wattage, location and mounting height 	of lamps. Depreciation factor =1.3, utilization coefficient=0.5, Light outputs are 	as follows:

watts: 		100 	200 	300 	500 	1000

lumens:	1615 	3650 	4700 	9950 	21500

2.a)	Explain, with the help of a neat sketch, the Ajax-Wyatt furnace.

b)	Determine the amount of Energy required to melt the brass at the rate of one 	ton per hour in a single phase Ajax Wyatt Furnace.

Specific heat of brass = 0.094

Latent heat of fusion = 40 K cal/kg

Initial temperature = 24°C

Melting point of brass = 920°c.

Assume the efficiency to be 65%

3.a)	What type of electric motors you will select for the following and why

(i) Textile mills  (ii) Printing press.

b)	Explain and describe the various characteristics which should be considered for 	selecting an Electric motor for a given industrial load.

4.a)	Compare and contrast DC and AC supply system used for traction applications.

b)	Write a short note on recent trends in Electric traction.

5.a)	Explain the three methods of electric braking employed in traction, bringing their 	merits and demerits.

b)	An electric train is required to be driven up an inclined plane with a gradient of 	0.5 percent at a speed of 40 kmph. The train resistance is 40 Newtons/ton. If the 	power taken up by the motor from the grid is 200 kW. Compute maximum 	permissible weight of train. The combined efficiency of motor and gearing 	systems is 75%,

Contd	..2

Code No:420251				-2-					OR

6.a)	Define the following:

(i)   Tractive effort

(ii)  Coefficient of adhesion

(iii) Specific energy consumption.

b)	An electric train has a quadrilateral speed time curve as follows:

(i)   Uniform acceleration from rest at 2.5 kmphps for 25 sec.

(ii)  Coasting for 50 sec.

(iii) Duration of braking 25 sec.

If the train is moving along a uniform up gradient of 1 in 100 with a tractive resistance of 45 N/ T. Rotational inertia 10% of dead weight, duration of stops at stations 20 sec. and over all efficiency of transmission gear and motor 80%. Calculate the schedule speed and specific energy consumption of run.

7.a) 	Discuss the various types of electric welding with the help of neat diagrams.

b) 	Calculate the maximum load that can be carried by a 15 kW output motor if the 	temperature rise is not to exceed 45°C after one hour on over load. The 	temperature rise after one hour is 30°C on full-load and 40°C after 2 hours. 	Assume that losses are proportional to square of the load.

8.	Write short notes on:

a) Flood lighting

b) AC and DC welding

c) Speed control of Electric drives.

~ ~ ~ ~