Code No.:10205

Set No.

1

I-B.Tech. Supplementary Examinations, December 2002/January 2003

SEMICONDUCTOR DEVICES AND CIRCUITS

(Common to Electrical and Electronics Engineering, Electronics and Communication Engineering, Computer Science and Engineering, Computer Science and Information Technology, Electronics and Instrumentation Engineering, Bio-medical Engineering, Electronics and Control Engineering, Electronics and Telematics, Electronics and Computer Engineering and Computer Science and Systems Engineering)

Time: 3 hours  							        Max. Marks: 80

Answer any FIVE questions

All questions carry equal marks

- - -

1.a)	Discuss in detail the applications of C.R.O.

b)	An infinitely large parallel plane plates are spaced 1cm apart.  The voltage at one of the plates is raised from 0-6.5U in one nanosecond at a uniform rate w.r.t. the other.  After this duration, the p.d. between the plates is suddenly dropped to 0 volts and remains the same thereafter.  Find the total time of the transit of electron from the cathode to the anode; and the position of the electron which started with zero initial velocity from the negative plate when the p.d. drops to zero volts.

2.a)	With neat diagram and wave forms explain the operation of a full-wave rectifier.  Derive for its average value, R.M.S value, and ripple factor.

b)	Explain the U-I characteristic of varactor diode and zener diode.

3.a)	Explain the four distinct regions of the output characteristics of JFET.

b)	Explain the U-I characteristics of a U.J.T. What are its applications?

4.a)	Explain the following terms related to amplifiers: (i) Biasing (ii) Thermal Runaway (iii) Thermal Stability.

b)	Determine the values of R1, R2 and Re of  a germanium transistor in the self-biasing arrangement shown in fig.4(b).  Take Vcc=15V, Rc=1.5Ω.  Quiescent point:- VcE=8V and Ic=4mA.  S=12 and β=45.

Contd(2)…….

Code No.10205				    -2-			                    Set No.1

5.a)	Explain how band width of single stage and multistage amplifiers calculated.

b)	Draw the equivalent circuit of a CE amplifier using hybrid-Π model at high frequencies and derive the expression for voltage gain with and without considering source resistance.

6.a)	Draw the circuit diagram of a voltage shunt feed back amplifier.  Explain how voltage gain, input impedance and output impedance are calculated for it.

b)	State and explain Bark Hausen criterion for oscillation.

7.a)	Derive the equation for the 3-dB bandwidth of double tuned amplifier.

b)	Analyze single tuned amplifier.

8.	Write short notes on any THREE of the following:

(a) Tunnel diode

(b) B.J.T, characteristics

(c) configurations of FET amplifiers

(d) L.C.Oscillators

(e) Push-pull amplifiers.

- - -

Code No.:10205

Set No.

2

I-B.Tech. Supplementary Examinations, December 2002/January 2003

SEMICONDUCTOR DEVICES AND CIRCUITS

(Common to Electrical and Electronics Engineering, Electronics and Communication Engineering, Computer Science and Engineering, Computer Science and Information Technology, Electronics and Instrumentation Engineering, Bio-medical Engineering, Electronics and Control Engineering, Electronics and Telematics, Electronics and Computer Engineering and Computer Science and Systems Engineering)

Time: 3 hours  							        Max. Marks: 80

Answer any FIVE questions

All questions carry equal marks

- - -

1.a)	Compare electric and magnetic deflection systems.  Also mention their applications.

b)	Electrons are projected into the region of constant electric field intensity(E), 5500V/m.  The electron gun makes an angle of 370 with the horizontal.  It ejects the electrons with an energy of 120 eV.  Find the time it takes an electron leaving the gun to pass through A hole H at a horizontal distance of 3.5 cms from the position of the gun; and also find the distance ‘d’ in order that the particles emerge through the hole H?

Fig.1(b)

2.a)	Derive a general expression for the diode curaent and then high light the significance of the reverse saturation current.

b)	Derive the expression for diffusion capacitance of a P-N junction diode.

3.a)	Explain the U-I characteristics of B.J.T.  Also give its specification.

b)	Explain the enhancement MOSFET operation.

4.a)	Explain the following terms related to amplifiers. (i) Voltage gain  (ii) Current gain (iii) Input impedance  and (iv) Output impedance.

b)	Derive expressions for α and β cut-off frequencies in terms  of transistor parameters.

Contd(2)….

Code No. 10205				      -2-				         Set No.2

5.a)	Explain the differences between low frequency transistor amplifier and R.C coupled amplifiers.

b)	Determine the quiescent current, VCE and stability factors for a silicon transistor with β=50 in self-bias circuit.  The circuit component values are: Vcc=16 Volts, Rc=1.5kΩ, R1=47  kΩ, and R2=12 kΩ.

6.a)	Bringout the advantages of negative feedback and derive the expressions for frequency and nonlinear distortions with feedback.

b)	Draw the circuit diagram of Wein-Bridge oscillator and derive the expression for the frequency of oscillations.

7.a)	Draw the circuit diagram of any class ‘AB’ power amplifier and explain its operation in detail.

b)	Derive an expression for the conversion efficiency of an ideal class ‘B’ amplifier.

8.	Wrtie short notes on any THREE of the following:

(a) Zenerdiode

(b) U.J.T. characteristics

(c) Early effect

(d) Colpitts oscillator

(e) Single tuned voltage amplifier.

- - -

Code No.:10205

Set No.

3

I-B.Tech. Supplementary Examinations, December 2002/January 2003

SEMICONDUCTOR DEVICES AND CIRCUITS

(Common to Electrical and Electronics Engineering, Electronics and Communication Engineering, Computer Science and Engineering, Computer Science and Information Technology, Electronics and Instrumentation Engineering, Bio-medical Engineering, Electronics and Control Engineering, Electronics and Telematics, Electronics and Computer Engineering and Computer Science and Systems Engineering)

Time: 3 hours  							        Max. Marks: 80

Answer any FIVE questions

All questions carry equal marks

- - -

1.a)	Explain the terms: Electric field intensity; Electric and Magnetic deflection sensitivities; and C.R.O.

b)	A cathode emits an electron with an initial energy of 15 volts and a velocity normal to the cathode surface.  The distance between anode and cathode plates is 12 cms and the P.D. is maintained at 1600V (positive w.r.t. cathode).  Calculate the time required for the electron to reach the anode; and the k.E. of an electron as it reaches the anode.

2.a)	Explain the V-I characteristics of p-n junction diode.  Give the important applications of it with diagrams.

b)	Discuss the construction and operation of zener diode and photo diode.

3.a)	Explain the input and output characteristics of a B.J.T.

b)	Bringout the differences between F.E.T and B.J.T.  Also draw the F.E.T characteristics.

4.a)	Explain the following terms related to amplifiers: (i) Biasing (ii) Thermal stability (iii) Input impedance  and (iv) Voltage gain.

b)	Find the input impedance and voltage gain of Biasing circuit of an amplifier shown in fig.4(b).  Given Rc=Rc=2.1 kΩ, hie=1 kΩ, hre=hoe=0 and hfe=80

Contd(2)……

Code No. 10205				     -2-				         Set No.3

5.a)	Explain the concept of gain band width product.

b)	Three identical cascaded stages have an overall upper 3-dB frequency of 20KHz and a lower  3-dB frequency of 20 Hz.  What are fL and fH of each stage?  Assume non-interacting stages.

6.a)	Explain any two configurations of negative feed back amplifiers with the help of block diagrams.

b)	Explain the working of Hartley oscillator with circuit diagram.  Derive the condition for oscillation and frequency of oscillation.

7.a)	Give classification of power amplifiers.  Mention the main feature of each.

b)	A 16Ω speaker is driven by a transformer coupled class-A amplifier through A15:1 transformer.  The circuit delivers 2 watts to the load Vcc=35V.  Calculate A.C.power across the transformer primary; R.M.S. value of load voltage; R.M.S. value of primary voltage; and R.M.S. value of load and primary current.

8.	Write short notes on any THREE of the following:

(a) Varactor Diode

(b) M.O.S.F.E.T;

(c) Concept of feed back in amplifiers;

(d) Crystall oscillators;

(e) Class ‘B’ tuned power amplifiers.

- - -

Code No.:10205

Set No.

4

I-B.Tech. Supplementary Examinations, December 2002/January 2003

SEMICONDUCTOR DEVICES AND CIRCUITS

(Common to Electrical and Electronics Engineering, Electronics and Communication Engineering, Computer Science and Engineering, Computer Science and Information Technology, Electronics and Instrumentation Engineering, Bio-medical Engineering, Electronics and Control Engineering, Electronics and Telematics, Electronics and Computer Engineering and Computer Science and Systems Engineering)

Time: 3 hours  							        Max. Marks: 80

Answer any FIVE questions

All questions carry equal marks

- - -

1.a)	Derive the expression for the magnetic deflection sensitivity of a C.R.O.

b)	A charged particle having charge 4 times that of an electron and mass 3 times that an electron is accelerated through sa potential difference of 60V before it.  Enters a uniform magnetic field of flux density 0.25 wb/m2 at an angle of 300 with the field.  Find the velocity of the charge particle before entering the field. Time of one revolution; And radius of helical path.

2.a)	Draw the U-I characteristics of P-N diode, zener diode, and tunnel diode.  Explain the salient points in each case.

b)	Discuss about the diffusion and transition capacitances of a P-N junction diode.

3.a)	Explain the V-I characteristic of a U.J.T.

b)	Discuss the static drain and gate characteristics of an N-channel enhancement type M.O.S.F.E.T.

4.a)	Explain the criteria for fixing operating point in amplifiers.

b)	Design a self-bias circuit for a silicon transistor having β=50.  Assume VCE=10 volts, RC=3.25 kΩ and S=10.  The operating point has to be set at 4.5V and 1.2 mA.  Take VBE=0.7V.

5.a)	Explain the concept of single stage and multistage amplifiers.

b)	Draw the circuit diagram of two stage R-C coupled amplifier and its equivalent circuit.  Derive the equation for its overall voltage gain.

6.a)	What are the advantages of negative feed back in amplifiers?  An amplifier has a voltage gain of 400, f1=50HZ, f2=200 KHz, distortion 10% without feedback.  Find the voltage gain, f1f, f2f and Df when a negative feed back is applied with feed back ratio of 0.001.

b)	Draw the Hartley oscillator circuit and derive the expression for the frequency of oscillations.

Contd(2)……..

Code No. 10205				     -2-				         Set No.4

7.a)	Draw the circuit diagram of a class ‘B’ push-pull amplifier and explain its operation.  Show the wave forms at various points in this circuit.

b)	Explain the operations of a complementary symmetry push-pull power amplifier.

8.	Write short notes on any THREE of the following:

a) Varactordiode;

b) B.J.T., characteristics;

c) Cascade amplifiers;

d) R.C. phase-shift oscillator;

e) Double tuned voltage oscillator.

- - -