Codes No.10157 OR

I-B.Tech. Supplementary-Examination December 2002/January 2003

ENGINEERING GRAPHICS

(Common to Civil Engineering, Mechanical Engineering, Printing Technology, Metallurgical Engineering, Production  Engineering, Mechatronics, Material Manufacturing Technology, Mechanical Manufacturing Engineering, Aeronautical Enginnering)

Time:3 hours								Max.Marks:80

Answer any Five questions

All questions carry equal Marks

---

1.	An area of 144 sq.cm. on a map represents an area of 36 sq.km on the field. Find the R.F. of the scale for this map and draw a diagonal scale to show kilometers hectometers and decameters and to measure up to 10 kilometers. Indicate on the scale a distance of 7 kilometers, 5 hectometers and 6 decameters.

2.	A circle of 50mm diameter rolls on the circumference of another circle of 175mm diameter and outside it. Trace the locus of a point on the circumference of the rolling circle for one complete revolution. Name the curve. Draw a tangent and a normal to the curve at a point 125mm from the centre of the directing circle.

3.	The front view of a line A B measures 65mm and makes an angle of 45o with xy.

A is in HP and the V.T. of the line is 15mm above H.P. The line is inclined at 30o to the VP. Draw the projections of AB and determine its true length and inclination with the HP. Also locate its HT.

4.	A circular plate of negligible thickness and 50mm diameter appears as an ellipse in the front view, having its major anis 50mm long and miner axis 30mm long. Draw its top view when the major axis of the ellipse in horizontal.

5.	A hexagonal pyramid, base 25mm side and axis 50mm long, has an edge of its base on the ground. Its axis is inclined at 30o to the ground and parallel to the V.P. Draw its projections.

6.	A vertical cylinder of 60mm diameter has a square hole of 30mm side cut through it. The axis of the hole is horizontal, parallel to the V.P. and 6mm away from the axis of the cylinder. The faces of the hole are equally inclined to the H.P. and the V.P. Draw the projections of the cylinder showing the hole in it.

(Contd…2)

Code No.10157			-2-			OR

- Draw the isometric view of the casting shown in three views in figure  below.

- Draw the development of the lateral surface of the figure shown below

***     ***     ***