Code No: 10105

Set No.

1

## I-B.Tech Supplementary Examination December2002/January 2003

## ENGINEERING MECHANICS

(Common to Civil Engineering, Mechanical Engineering, Printing Technology, Metallurgical Engineering, Production Engineering, Mechatronics, Metallurgy & Materials Technology, Mechanical Manufacturing Engineering, Aeronautical Engineering)

Time: 3 Hours							Max. Marks: 80

## Answer any Five questions

All questions carry equal marks

---

1.	A 3000 N sphere shown in FIG.1 is supported by the pull P and a 2000 N weight passing over a frictionless pulley.  If  = 300, Calculate the values of P and .

Fig. 1

2.	Determine the resultant of a system of Concurrent forces having the following magnitudes and passing through the origin and the indicated points.

P = 3000 N	(+12, +6, -4)

Q= 5000 N	(-3,  -4, +12)

F=  2800 N	(+6, -3, -6  )

3.	A single threaded screw-jack has a piteh of  12 mm and a means radius of 45 mm.  The coefficient of static friction is 0.15 and of Kinetic friction is 0.10.

- Determine the force P applied at the end of a lever 600 mm long which will start lifting a weight of 20,000 N.

- What value of P will keep the Jack screw turning .

4.	Find the moment of inertia about the Y-axis for the shaded area shown in Fig. 2

Contd…2

# Code: 10105					.2.				        Set No:1

Fig.2

5.	A particle is Constrained to move upward to the right along the path 2y2 = x3 + 26 where x and y are in cm.  The x coordinate of the particle at any time is x = t2-t + 4.  Determine the y component of velocity and acceleration at x = 6 cms.

6.	Three bodies, each of weight W, are arranged as shown in FIG 3.  The coefficient of Kinetic friction between B and C is 0.30 and is 0.10 between C and the ground.  How far does C move when B has moved S Meters relative to C?

7.	Derive the expression for the moment of inertia of a homogenous right circular cone of mass m, base radius r, and altitude h, with respect to its geometric axis.

8.	A simple pendulum is suspended from the roof of an elevator which is accelerating at a M/sec2.  Assuming that the vibrations are small, determine the period of the pendulum when the elevator is

- accelerating upward

- accelerating downward

- falling freely.

###

Code No: 10105

Set No.

2

## I-B.Tech Supplementary Examination December2002/January 2003

## ENGINEERING MECHANICS

(Common to Civil Engineering, Mechanical Engineering, Printing Technology, Metallurgical Engineering, Production Engineering, Mechatronics, Metallurgy & Materials Technology, Mechanical Manufacturing Engineering, Aeronautical Engineering)

Time: 3 Hours							Max. Marks: 80

## Answer any Five questions

All questions carry equal marks

---

1.	Calculate the tension T in the Cable and the horizontal and Vertical components of the reaction at O. Neglect the weight of the rod OB and the size of the pulley at C. (Fig.1)

Fig. 1

2.	Three cylinders are placed in a rectangular box as shown in FIG.2.  Neglecting friction, determine the reaction between cylinder A and the Vertical well.

Contd…2

#### Code No: 10105				.2.				    Set No. 2

3.	A torque of 400 N.m acts on the brake drum shown in FIG.3. One end of the brake band is attached at B of brake lever where the lever is connected to fulcrum.

Other end of the band is attached to the lever at A.  If the brake band is in contact with the brake drum through 2500 and the coefficient of friction is 0.3, determine the force P at the end of brake lever.

Fig 3

4.	A slender homogenous wire is bent into the shape shown in FIG.4 and suspended from a hinge at A.  Find the angle the neck of the hook will make with the vertical.

Fig.4

##### Contd…3

#### Code No: 10105				.3.				    Set No. 2

5.	A rocket is fired Vertically and tracked by radar an shown in FIG.5.  At the instant  = 600, it is known that r = 10,000 Metres, 0 = 0.02 rad/sec and 00 = 0.002 rad/sec2.  Determine the velocity and acceleration of the rocket.

Fig. 5

6.	In FIG. 6, the body A weighs 1000 N and B weighs 500 N.  Determine the acceleration of the bodies if the coefficient of Kinetic friction is 0.10 between cable and the fixed drum.

7.	A slender bar of weight W and length L is supported at one end and at a distance b from its mass Centre G.  If the support at B is suddenly removed, determine b so that the bar attains a maximum angular velocity after a 900 rotation.

Fig. 7

##### Contd…4

Code No. 10105				.2.				Set No. 2

8. a)	The amplitude of a simple harmonic motion is 0.5 m and the period is 1 sec.  Determine the maximum velocity and maximum acceleration.

b)	Explain (i) Central force motion	(ii)	Work-Energy method.

###

Code No: 10105

Set No.

3

## I-B.Tech Supplementary Examination December2002/January 2003

## ENGINEERING MECHANICS

(Common to Civil Engineering, Mechanical Engineering, Printing Technology, Metallurgical Engineering, Production Engineering, Mechatronics, Metallurgy & Materials Technology, Mechanical Manufacturing Engineering, Aeronautical Engineering)

Time: 3 Hours							Max. Marks: 80

## Answer any Five questions

All questions carry equal marks

- A pulley of 1 M radius, supporting a load of 5000 N, is mounted at B on a horizontal beam shown in Fig. 1.  If the beam weighs 2000 N and the pulley

weighs 500 N, find the hinge force at C.

Fig. 1

2.	Find the resultant of the force system shown in FIG. 2 in which P = 2500 N, T = 2000 N, and F = 1800 N.

Fig .2

Contd..2

#### Code No: 10105				.2.				    Set No. 3

3.	In FIG.3, the coefficient of friction is 0.2 between rope and the fixed drum and between all surfaces in contact.  Determine the minimum W to prevent down plane motion of 5000 N body.

Fig. 3

4.	Locate the centroid of the shaded area enclosed by the curve y2 = ax and the straight line shown in FIG.4

Fig. 4

5.	The velocity of a particle is defined by Vx = 100 – t 3/2 and Vy = 100 + 100 t - 2t2 where V is in Meters/sec and t in seconds.  Determine the radius of curvature at the top of its path.

6.	Determine the acceleration of the bodies shown in FIG. 5 if the coefficient of kinetic friction is 0.2 at all contact surfaces.  Body A weighs 1000 N and Body B weighs 1500 N.

Fig.5

##### Contd…3

Code No: 10105				.3.				    Set No. 3

7.	Body A shown in FIG.6 drops 4 M from rest before striking the ground.  Pulley B is mounted on an axle 150 mm diameter, and the axle friction is constant at 200 N.  How many turns will B make after A stops?  Also Calculate the tension in the cord before A hits the ground. Use work-Energy method.

Fig. 6

8. a)	A particle in Simple harmonic motion has an amplitude of 375mm and a period of /2 sec.  Find the velocity and acceleration of the particle when it has traveled 225mm to the right of the center of its path.  What time is required for this displacement?

b)	Explain Products of Inertia and transfer formula for product of Inertia.

###

Code No: 10105

Set No.

4

## I-B.Tech Supplementary Examination December2002/January 2003

## ENGINEERING MECHANICS

(Common to Civil Engineering, Mechanical Engineering, Printing Technology, Metallurgical Engineering, Production Engineering, Mechatronics, Metallurgy & Materials Technology, Mechanical Manufacturing Engineering, Aeronautical Engineering)

Time: 3 Hours							Max. Marks: 80

## Answer any Five questions

All questions carry equal marks

---

- A rod AB is supported in a horizontal position by a hinge A and a cable which runs from B over  small pulleys at C and D is connected to a weight W rests on rod AB.  Calculate the reaction at A.

Fig.1

2.	Three rods pinned together at  B and C and supported by Hinges at A and D as shown in FIG.2, form a four link mechanism.  Determine the value of P that will prevent motion.

Fig.2

Contd…2

Code No: 10105				.2.				Set No. 4

3.	A uniform bar AB, 10M long and weighing 3000N is hinged at B and rests upon a 4000 N block at A shown in FIG. 3.  If the coefficient of friction is 0.4 at all contact surfaces, find the horizontal force P required to start moving the block.

Fig.3

4.	For the shaded area shown in Fig.4, Calculate the moment of inertia about the x and y axes.

Fig. 4

- An auto is accelerated from rest to a top speed of 100 Km/hour and then immediately decelerated to a stop.  If the total elapsed time is 20 sec, determine the distance covered.  The acceleration and deceleration are both constant but not necessarily of the same magnitude.

Contd..3

Code.No: 10105				..3..			Set.No: 4

- Two bodies A and B weigh 2000 N and 1500 N respectively are connected by a rigid bar of negligible weight and move along the smooth surfaces shown in FIG.5. If they start from rest at the given position, determine the acceleration of B at this instant.

Fig 5

7.	Derive the mass moment of inertia of a thin walled spherical shell of mean radius r and thickness t with respect to any diametral axis.

8.	The compound pendulum shown in FIG.6.  Consists of a slender rod 600 mm long weighing 30 N to which is attached a solid circular disk of 300 mm diameter that weighs 40 N.  Compute the period of small oscillation.

Fig. 6

###