Code No: 10107

Set No.

1

## I-B.Tech Supplementary Examination December2002/January 2003

## ENGINEERING GRAPHICS

(Civil Engineering, Mechanical Engineering, Printing Technology, Metallurgical Engineering, Production Engineering, Mechatronics, Mechanical Manufacturing Engineering, Aeronautical Engineering)

Time: 3 Hours							Max. Marks: 80

## Answer any Five questions

All questions carry equal marks

---

1. a)	Construct a comparative scale to measure up to  (i)	40 miles and 	(ii) 60 km to be used  with a road map.  On this map a distance of 15 miles is shown by a 5 cm long line.

b)	Draw a hyperbola with its foci 70 units apart and the major axis is 45 units. Draw a tangent to the curve at a point 20 units from the focus. Also determine the eccentricity of the curve.

2. a)	A shot is discharged from the ground level at an inclination of 450 to the ground which is horizontal.  The shot returns to the ground at a point 250m from the point of discharge.  Trace the path of the shot.  Find the direction of the shot after it has travelled a horizontal distance of 200m.

b)	Draw an ellipse by concentric circles method given the major and minor axes as 100 mm and 60 mm respectively.

3. a)	A line A B of 65 mm long has its end A, 25mm above HP and 20 mm in front of V.P.  The end B is 40 mm above HP and 65 mm in front of VP. Draw its projections or find its inclinations with HP and VP.  Determine its traces.

b)	Draw the projection of a square pyramid of base 40 side and axis 70 long, when the solid lies with one of its slant edges on HP and the vertical plane passing through that slant edge and axis makes 300 with V.P.

4. 	A pentagonal pyramid, with base 35 side and height 70 rests on one edge of its base on HP so that the highest point on the base is 25 above HP.  Draw its projection, when the axis is parallel to VP.  Draw another front view, on a reference line inclined at 450 to the edge on which it is resting so that the base is visible.

5.	A pentagonal prism of base edge 30 mm and height 70 mm is placed with one of its rectangular faces on the ground and the axis parallel to VP. It is cut by a section plane perpendicular to VP and inclined at 300 to the ground.  It passes through the mid point of the axis.  Develop the remaining surface of the object.

### Contd…2

Code No: 10107				.2.				    Set No. 1

6.	The following figure show the orthographic projections of certain object.  Draw the isometric projection.

Fig. 1

7.	Draw the orthographic projections for the following object shown below.

Fig.2

### Contd…3

Code No: 10107				.3.				    Set No. 1

8.	A square pyramid of base edges 50 mm long and attitude 70 mm is resting on its base on the ground with one of the corners of the base touching the picture plane.  Two adjacent base edges having this corner make equal inclination with V.P.  The station point lies on the central line of the object 60 mm in front of the picture plane and 90 mm above the ground.  Draw the perspective projection of the solid.

###

Code No: 10107

Set No.

2

## I-B.Tech Supplementary Examination December2002/January 2003

## ENGINEERING GRAPHICS

(Civil Engineering, Mechanical Engineering, Printing Technology, Metallurgical Engineering, Production Engineering, Mechatronics, Mechanical Manufacturing Engineering, Aeronautical Engineering)

Time: 3 Hours							Max. Marks: 80

## Answer any Five questions

All questions carry equal marks

---

1. a)	Construct a vernier  scale of R.F = 1/50000 to measure kilometers, hectameters and decametrs and long enough to measure 7 kilometers.  Indicate on the scale a distance of 5 kilometers, 4 hectameters and 5 decameters.

b)	Draw the curve when the distance of 60 cm from directix is 50 mm and eccentricity is 3/2. Name the curve and draw tangent and normal from any point on it.

2. a)	A circle of 40 units diameter rolls without slipping on the inside of another circle of 160 units diameter.  Show the path of the point on the periphery of the rolling circle, diametrically opposite to the initial point of contact between the circles when the rolling circles makes one rotation clockwise.  Name the circle.

b)	A circular disc of diameter AB equal to 80 units rotates about its center with uniform angular velocity.  During one complete revoluation, a point P moves along the diameter AB from A to B. Draw the locus of the point P.

3. a)	The end P of a line PQ is 25 mm above HP and 40 mm in front of VP and end Q is 20 mm behind VP and 22 mm below HP.  The end projections are 50 mm apart. Draw the projections of the line and find its true length and traces.

b)	Draw the projections of a cylinder of 40 diameter and axis 60 long, when it is lying on HP, with its axis inclined at 450 to HP and parallel to V.P.  Follow the change of position method.

4.	A square pyramid, with side of base 50 and axis 70 long, is resting on its base, with one edge perpendicular VP. It is cut by a section plane perpendicular  to VP so that the true shape of the section is an ISO  scales triangle 40 base and altitude 45.  Find  the inclination of the section plane with HP.  Draw the projections and the true shape of the section.

5.	An oblique cone of base 60 diameter and axis 75 long, rests on its base on HP.  The axis of the cone is inclined at 600 to the base.  It is cut by a horizontal section plane, passing through the axis at 55 from the base.  Draw the development surface of the remaining solid.

Contd…2

# Code No. 10107			-2-				       Set No. 2

6.	A cylinder of diameter of base 60 and height 70, rests with its base on HP.  A section plane perpendicular to VP and inclined at 450 to HP, cuts the cylinder such that it passes through the point on the axis at 50 above its base. Draw the isometric projection of the truncated cylinder.

- Draw the orthographic projections for the object shown below:

Fig.1

8.	Draw the perspective of a square prism of edge of base 40 and of length 60, lying on a rectangular face on the ground, with a corner on P.P and the bases are inclined at 450 to P.P. The station point is 60 in front of PP and 80 above G.P and lies in a central plane, which is passing through the center of the prism.

###

Code No: 10107

Set No.

3

## I-B.Tech Supplementary Examination December2002/January 2003

## ENGINEERING GRAPHICS

(Civil Engineering, Mechanical Engineering, Printing Technology, Metallurgical Engineering, Production Engineering, Mechatronics, Mechanical Manufacturing Engineering, Aeronautical Engineering)

Time: 3 Hours							Max. Marks: 80

## Answer any Five questions

All questions carry equal marks

---

1. a)	Construct a scale to be used with a map, the scale of which 1 cm = 40 m. The scale should read in meters, upto 500 m Mark a distance of 456m on it?

b)	Draw a rectangular hyperbola through a point P with the coordinates x = 25 and  Y = 30.

2. a)	Draw an epicycloid of a circle of 40 cm dia which rolls outside as another circle of 150 cm diameter for one revolution clockwise.  Draw a targent and a normal to it at a point 95 cm distance from the center of the directing circle.

b)	Draw the involute of a circle of 40 diameter.  Also draw a langent and a normal to the curve at a point 95 from the center of the circle.

3.  a)	A line KL of 75 long, makes an angle of 300 with VP. and is on the profile plane. One and of the line is HP while the other end is on VP. Draw its projections and also locate the traces.

b)	Draw the projections of a right circular cylinder diameter of base 30 mm and height 62 mm resting on HP on its base, such that the axis is parallel to UP and inclined at 300 to HP.

4. 	A cone of base 75 diameter and axis 75 long, has its axis parallel to HP and inclined at 450 to VP. A vertical section plane, cuts the cone through the mid point of 	the axis.  Draw the sectional front view, top view and an auxillary front view, on a plane parallel to the axis.

5.	Draw the projections of a cone, resting on its base on H.P and show the shortest path by which a point P starting from a point on the circumference of the base, moving around the cone, will return to the same point. Diameter of the base of the cone is 60 and axis 75 long.

6.	A square pyramid of base edge 20 mm and height 30 mm is mounted on a face of the cube of base edge 40 mm.  Draw the isometric projection of the object.

Contd…2

Code No.10107				.2.				 Set No. 3

7.	Draw the orthographic projections of the object shown in the figure.

Fig.1

8.	Draw the perspective view of a square pyramid of base 50 side and 60 high.  An edge of the base is parallel to and 10 behind P.P.  The station point is situated at a distance of 60 in front of P.P. 40 above the ground and 40 to the right of the apex.

###

Code No: 10107

Set No.

4

## I-B.Tech Supplementary Examination December2002/January 2003

## ENGINEERING GRAPHICS

(Civil Engineering, Mechanical Engineering, Printing Technology, Metallurgical Engineering, Production Engineering, Mechatronics, Mechanical Manufacturing Engineering, Aeronautical Engineering)

Time: 3 Hours							Max. Marks: 80

## Answer any Five questions

## All questions carry equal marks

---

1. a)	The distance between two points is 140 mm on a map.  The real distance between them is 28 km.  Draw a diagonal scale of this map to read kilometers and hectameters and to measure upto 30 km. Show a distance of 16.7 Km on the scale.

b)	A circle of 50 units diameter rolls on a straight line without slipping.  In the initial position, the diameter of the circle is parallel to the line, on which it rolls.  Draw the low of the point for one revolution of the circle.

2. a)	A fountain jet discharges water from the ground level at an	inclination of 450 to the ground level.  The jet travels a horizantal distance of 7.5 m from the point of discharge and falls on the ground.  Trace the path of the jet and name the curve.

b)	The major axis of hyperbola is 60 units and a point P on the curve is at a distance of 75 units from the center of the major axis and 35 units from the vertex. Draw the hyperbola.

3. a)	The front view of a 125 long line PQ, measures 75 and its top view measures 100.  Its end Q and the mid point M are in the first quadrant as M being 20 from both the planes.  Draw the projections of the  line PQ.

b)	A pentagonal prism of side of base 25 and axis 45 long, is resting as HP on a corner of its base. Draw the projections of the prism, when the base is inclined at 600 to HP and the axis appears to be inclined at 300 to VP.

4. 	A hexagonal pyramid of base 30 and axis 65 long, is resting on one of its triangular faces on HP and with its axis parallel to VP.  A horizantal section plane bisects the axis of the solid.  Draw the projecting.

5.	A cone of base 50 diameter and axis 60 long, is resting on its base on HP.  It is cut by a section plane, perpendicular to VP and parallel to an extreme generator and passing through a point on the axis at a distance of 20 from the apex. Draw the development of the retained solid.

Contd…2

#### Code No: 10107				.2.				Set No. 4

6.	The following figure shows orthographic projection of the object.  Draw the isometric projection.

Fig. 1

7.	Draw the orthographic projections for the following object shown in the figure.

Fig. 2

8.	A hexagonal prism of side of base 30 and height 60, stands with its base on the ground.  One rectangular face is parallel to P.P. and 30 away from it and the nearest vertical edge of this face is 20 to the left of the eye.  The eye is 100 from P.P. and 80 above G.L.  Draw the perspective of the prism.

###