Code No:10204

Set No.

1

## I-B.Tech Supplementary Examination December2002/January 2003

## NETWORK THEORY

(Common to Electrical and Electronics Engineering, Electronics and Communications Engg., Computer Science and Engg., Computer Science and Information Technology, Electronics and Instrumentation Engg, Bio-Medical Engineering, Electronics and Control Engg., Electronics & Telematics, Electronics & Computer Engg., Computer Science & Systems Engg.,)

Time: 3 Hours							Max. Marks: 80

## Answer any Five questions

All questions carry equal marks

---

1. a)	Obtain the response of R-L-C series cuit for impulse excitations.

- Define reluctance of a magnetic circuit and derive an expression for reluctance.

2. 	In an electrical circuit R1 L and C are connected in Parallel.  R = 10, L = 0.1 H and C = 100 F. The circuit is energized with supply at 230v, 50Hz calculate .

- Impedance

- Current taken from supply

- Power factor of the circuit

- Power consumed by the circuit

3.	A Constant voltage at a frequency of 1 MHz is applied to an inductor in series with a variable capacitor when the capacitor is set to 500 pF, the current has the maximum value, while it is reduced to one half when the capacitance is 600 pF find (i) the resistance (ii) the inductance (iii) the Q factor of the inductor.

4.	For the given graph and tree shown in fig.1 write the tie set matrix and obtain the relation between branch currents and link currents.

Fig.1

5.	Find the equivalent resistance between AB in the circuit shown in fig 2. All resistances are equal to R.

Contd…2

Code No. 10204				.2.				     Set No. 1

Fig.2

6.	Find the thevenins equivalent for the circuit in Fig. 3.

Fig.3

7.	The switch in the circuit shown in fig 4 is in position  (1) for two time constants and then changed to position (2) Find transient response.

Fig.4

8.	Design a m derived low pass filter having cut-off frequency of 1 KHz design impedance of 400 and the resonant frequency 1100 Hz.

###

Code No:10204

Set No.

2

## I-B.Tech Supplementary Examination December2002/January 2003

## NETWORK THEORY

(Common to Electrical and Electronics Engineering, Electronics and Communications Engg., Computer Science and Engg., Computer Science and Information Technology, Electronics and Instrumentation Engg, Bio-Medical Engineering, Electronics and Control Engg., Electronics & Telematics, Electronics & Computer Engg., Computer Science & Systems Engg., )

Time: 3 Hours							Max. Marks: 80

## Answer any Five questions

All questions carry equal marks

---

1. a)	Discuss Kirchoffs laws.

- Derive the expression from self, mutual inductance and coefficient of coupling.

- Explain source transformation with examples.

2. a)	What is the use of operator j?

- For the circuit shown in fig.1 find the current I drawn from the source.

Fig. 1

3. a)	A series RLC circuit with Q = 250 is resonant at 1.5 MHz.  Find the frequencies at half power points and also bandwidth.

b)	Distinguish between the average value and rms value of an alternating current waveform.

4.	Write and solve the equations for the mesh currents in the network shown in fig.2

Fig. 2

5.	Determine the line currents for the unbalanced delta connected load of fig.3 given.  Assume the phase sequence RYB.

Contd…2

#### Code No. 10204				.2.				      Set No. 2

Fig.3

6.	The circuit shown in fig.4 has resitance R which absorbs maximum power.  Computer the value of R and maximum power

Fig. 4

7.	In the fig 5 shown V(t) = 10V.  Find i2 (t).  Assume all initial conditions to be zero.  Use Laplace transform technique

Fig.5

8. a)	Design a T-pad attenuator to give an attenuation of 60dB and to work in a line of 500 impedance.

b)	Design K type band pass filter having a design impedance of 500 and cut off frequencies 1 KHz and 10 KHz.

###

Code No: 10204

Set No.

3

## I-B.Tech Supplementary Examination December2002/January 2003

## NETWORK THEORY

(Common to Electrical and Electronics Engineering, Electronics and Communications Engg., Computer Science and Engg., Computer Science and Information Technology, Electronics and Instrumentation Engg, Bio-Medical Engineering, Electronics and Control Engg., Electronics & Telematics, Electronics & Computer Engg., Computer Science & Systems Engg., )

Time: 3 Hours							Max. Marks: 80

## Answer any Five questions

All questions carry equal marks

---

1. a)	Sate the voltage current relationships for

i)	resistance	ii)	inductance and	iii)	capacitance.

b)	Two coupled coils with respective self inductances L1 = 0.8 H and L2 = 0.2 H have a coupling coefficient of 0.6 Coil has 500 turns.  If the current in coil 1 is I1(t) = 10 sin 200t, determine the voltage at coil 2 and the maximum flux set up by the coil 1.

c)	A torroid is made of steel rod of 2 cm diameter.  The mean radius of torroid is 20 cm Relative permeability of steel is 2000.  Compute the current required to produce 1 m web of flux and 1000 turns in the torroid.

2. a)	If I1 = 10∟0, I2 = 20 ∟600 and I3 = 12 ∟-30 find I1 + I2 + I3.

b)	Prove that the form factor for a sinusoidal current wave form is 1.11.

3. a)	Derive an expression for resonance frequency of a series R-L-C circuit.

b)	A coil of resistance 10 ohms and an inductance of 0.1 H is connected in series with a capacitor of capacitance 150F a cross a 200V, 50Hz supply Calculate

a)	Impedance	b)	Current	c)	Power and power factor of the circuit.

4. a)	Define cut set and tie set.

- Determine the current in the 10 resistor in the circuit shown in fig.1.

Fig.1

5. a)	Draw the dual network for the given network as in fig.2

Contd…2

#### Code No. 10204				.2.				      Set No. 3

Fig.2

b)	A balanced star connected load of 8 + j 6 /phase is connected to a 3, 230 V, 50 Hz supply.  Find the line current, power factor Total Active and Reactive Powers.

6. a)	Obtain the Norton’s equivalent circuit at the tuminals A, B for the fig. 3 shown below

Fig.3

- For the two port network shown find the h parameters.

Fig. 4

7.	A series RLC circuit with R = 5, L = 0.1 H and C = 500F has a D.C voltage of 100 V applied at t=0 through a switch.  Find the resulting current transient.

8. a)	Write short notes on Attenuators

b)	Design a constant K high pass filter for T type and  type to cut off at 5 KHz. Assume a characteristic impedance of 600.  Draw both configurations showing the circuit elements.

###

Code No: 10204

Set No.

4

## I-B.Tech Supplementary Examination December2002/January 2003

## NETWORK THEORY

(Common to Electrical and Electronics Engineering, Electronics and Communications Engg., Computer Science and Engg., Computer Science and Information Technology, Electronics and Instrumentation Engg, Bio-Medical Engineering, Electronics and Control Engg., Electronics & Telematics, Electronics & Computer Engg., Computer Science & Systems Engg., )

Time: 3 Hours							Max. Marks: 80

## Answer any Five questions

All questions carry equal marks

---

1. a)	Explain about Dot convention.

b)	An iron ring of mean length 50cm has an air gap of 1 mm and a winding of 200 turns.  If the permeability of iron is 400 when  a current of 1.25 A flows through the coil. Find the flux density.

2.	Derive the expressions for half power frequencies, Q factor & bandwidth of a series resonant circuit.

3. a)	For the parallel network shown in figure 1 . Determine the value of R 10 resonance.

Fig.1

- Define average value, rms value & for m factor in a.c circuits.

4. 	Determine the current in all branches of the network and the voltage across for resistor by loop method (shown in fig.2)

Fig.2					      Contd….2

Code No. 10204				.2.				Set No. 4

5.	A 440v, 3, 3 wire system is connected to an unbalanced star connected load shown in fig.3 Determine the line currents and the power input to the network.

Fig. 3

6. a)	Verify reciprocity theorem in the circuit shown in fig. 4

fig. 4

- State & explain compensation theorem

7.	Find the transfer frinction Vo(s) for the circuit shown in fig.5

Vi (s)

Fig. 5

Contd…3

# Code No: 10204				.3.				Set No. 4

8. a)	In the circuit shown in fig 6 find the expression for the transient current.

Fig.6

- Explain the principle of equalizers

- What is the necessity of SPICE in circuit analysis?

###