Codes No.10155 OR

I-B.Tech. Supplementary-Examination December 2002/January 2003

ENGINEERING MECHANICS

(Common to Civil Engineering, Mechanical Engineering, Printing Technology, Metallurgical Engineering, Production  Engineering, Mechatronics, Material Manufacturing Technology, Mechanical Manufacturing Engineering, Aeronautical Enginnering)

Time:3 hours								Max.Marks:80

Answer any Five questions

All questions carry equal Marks

---

1.a)	Explain Varignon’s theorem.

b)	A body is free to slide on a smooth vertical circular wire and is connected by a string. Equal in length to the radius of the circle, to the highest point of the circle. Find the tension of the string and the reaction of the circle.

2.a)	State the equations of equilibrium of Coplanar systems.

b)	A smooth sphere is supported in contact with a smooth vertical wall by a string fastened to a point in its surface, the other end being attached to a point in the wall. If the length of the string be equal to the radius of the sphere, find the inclination of the string to vertical, the tension of the string and the reaction of the wall.

3.a)	Explain the working of a differential screw jack.

b)	A heavy rod rests with its extremities on a rough circular hoop fixed in a vertical plane. If the rod subtends an angle of 90o at the centre of the hoop, find its inclination to the horizon, when in limiting equilibrium if the angle of friction is 30o.

4.	Two pulleys on parallel shafts, 1.95m apart, are connected by an open belt. The diameters of the pulley are 400mm and 200mm. Find the length of the belt required. Derive the formula used.

(Contd…2)

Code No.10155			-2-				OR

5.a)	A body consists of a solid hemisphere of radius 200mm and a right circular solid cone of height 24cm. The hemisphere and cone have a common base and are made of the same material. Find the position of the centre of gravity of the compound body.

b)	Find the moment of Inertia about the horizontal axis through the centre of Gravity of the section shown below.

6.a)	The tractive force exerted by a railway car weighing 50KN is 2000N. If the frictional resistance is 5N per KN of the railway Car’s weight, determine the acceleration when the railway car is moving on a level track.

b)	A link AB is moving in a vertical plane. At a certain instant, when the link is inclined at 60o to the horizontal, the point A is moving horizontally at 5m/s while B is moving vertically upwards. Find the velocity of B.

7.a)	A pile hammer weighing 1200 kg drops from a height of 75cm on a pile weighing 600kg. If the pile penetrates a distances of 40mm for every blow. Find the resistance of the ground assuming it to be uniform and the impact to be plastic.

b)	A point executing SHM has velocities u and v and accelerations a and b in two of its positions. Show that the distance between the two positions is

v2-u2

a+b

8.	Write short notes on any Three of the following:

a) Equations of Translation

b) Compound Pendulum

c) Free vibrations

d) Peppers theorem

e) Free body diagrams.

***     ***     ***