Code No.:220452

OR

II B.Tech. II-Semester Examinations April-May 2003

ELECTRONIC CIRCUITS

(common to Electronics and Communications Engineering, Electronics and Instrumentation Engineering, Bio-Medical Engineering, Electronics and Control Engineering, Electronics & Telematics)

Time: 3 hours							Max. Marks: 70

Answer any five questions

All questions carry equal marks

- - -

1.a)	Explain the necessity of biasing a BJT.  Why does the operating point of a BJT shifts.

b)	An npn transistor with (Beta)=50 is used in CE configuration with Vcc=+10v and Rc=4k.  The bias is obtained by connecting a 100 k resistance from collector to base.  Assuming VBE=0.5v find operating point (Ic, VCE).  Also calculate the stability factor ‘s’ of the circuit.

2.a)	Compare CB, CE, and CC configurations with respect to voltage gain, current gain, input resistance and out put resistance.  Give typical values for each.  Justify why CE amplifier is most widely used.

b)	A BJT having hie=1300 ohms, hfe=100, hre=2x10-4 and hoe=10-6 A/V is used as an emitter follower amplifier with Rs=1k and RL=500 ohms.  Determine for the amplifier AVS, Ais, Ri and Ro.

3.a)	Draw the circuit of cascade amplifier.  Find its over all gain, input impedance and output impedance.  What is the effect of bypass and coupling capacitors on the low frequency response of the amplifier?

b)	For the circuit shown determine Ri, Ro and Ai and Av with the data hfe=100, hie=2k, heo=10-5 hre is negligible and Ib=0.1mA

Contd…2

Code No.: 220452				-2-					  OR

4.	For the following low frequency parameters of a given transistor at IC=10ma, VCE=10v and at room temperature

hie=550, 	hoe=10-5A/V

hfe=100	hre=10-4

with fT=50MHz and Cob=3PF compute all values of hybrid- parameters.

5.a)	Explain about JFET biasing for zero drift current and derive the necessary expression.  How JFET can be used as a voltage variable resistor? Explain.

b)	For a JFET amplifier in CS configuration VDD=24V, VGS= 0.4 V , RD=20k, RL=50k.  Determine the output voltage, ID at the quiescent point is 0.7mA and VDS=11v.

6.a)	Draw the circuit of current shunt feedback amplifiers and justifying the type of feedback clearly.  How does input and output impedances change with the type of feedback.

b)	An amplifier with open loop voltage gain Av=1000100, is available.  The variation in the voltage gain is to be restricted to  0.1% find the value of .

7.a)	Draw the circuit for Hartley oscillator and explain how it oscillates?  Derive the expression for fo.  What are its applications?  Compare the circuit with colpitt’s oscillator circuit.

b)	Show that in the case of Wien bridge oscillator circuit, the minimum value of gain of the amplifier must be  3 for sustained oscillations.

8.	Write short notes on any of TWO of the following:

i)      Crystal oscillator.

ii)     Thermal runaway.

- Cascade amplifier.

^^^