Code No.221103

Set No.

1

II-B.Tech. II-Semester Examination April/May 2003

DIGITAL CIRCUITS DESIGN AND APPLICATION

(Bio-Medical Engineering)

Time: 3 hours							Max.Marks:80

Answer any FIVE questions

All questions carry equal marks

---

1.a)	Under what conditions the Low-Pass RC circuit acts as an integrator. Draw the output waveforms of RC integrator circuit for a square wave input for small, medium and large time constants.

b)	Draw the circuit of a double-ended clipper using diodes and draw its transfer characteristics.

2.a)	Illustrate the basic principle involved in Bootstrap sweep circuit. Draw a practical form of a bootstrap sweep circuit and derive expressions for its sweep speed and linearity.

b)	Distinguish between voltage and current sweep circuits.

3.a)	Convert the decimal number (247)10 into (i) OCTAL 	(ii) HEXADECIMAL

iii)  BINARY

b)	Design a parity generator to generate odd parity for a 4-bit word using Ex-OR gates.

4.a)	A NAND gate is used to drive fifty 7400 gates. Show the interfacing circuit.

b)	Explain the operation of the MOS dynamic shift register with a diagram.

5.a)	Design a full-adder using 8:1 multiplexer.

b)	Design a clocked RS Flip-flop.

6.a)	Compare LED and LCD display systems.

b)	Design a BCD to 7-segment (LED display) decoder circuit.

7.a)	Design a 4-bit asynchronous counter using flip-flops.

b)	Draw the circuit of crystal controlled clock generator and discuss about its frequency and amplitude stability.

8.	Write short notes on:

- Diode clamping circuit

- Race-around condition in flip-flops

- Retriggerable monostable.

***     ***     ***

Code No.221103

Set No.

2

II-B.Tech. II-Semester Examination April/May 2003

DIGITAL CIRCUITS DESIGN AND APPLICATION

(Bio-Medical Engineering)

Time: 3 hours							Max.Marks:80

Answer any FIVE questions

All questions carry equal marks

---

1.a)	A symmetrical square wave whose peak-to-peak amplitude is 2V and whose average value is zero is applied to an RC integrating circuit. The time constant is equal to half period of the square-wave. Find the peak-to-peak value of the output amplitude.

b)	Draw the circuit of Emitter-coupled Astable multivibrator and draw the output waveforms.

2.a)	Draw the circuit of transistor constant-current sweep circuit and derive an expression for slope error of the sweep generated.

b)	Draw the circuit of a TV current sweep circuit and explain how it produces linear current sweep.

3.a)	Prove AB+CD = AB.CD__________ using DeMorgen’s theorem.

b)	Design a BCD to 7-segment (LED display) decoder circuit.

4.a)	Design a 4-bit subtractor using 4-bit adders and minimum number of EX-OR gates.

b)	Convert RS flip-flop to JK flip-flop.

5.a)	Design a divide-by-5 ripple counter using flip-flops.

b)	Compare the noise-margins of  TTL and CMOS IC’s.

6.a)	Design an RS Latch using two input NOR gates.

b)	Design an 8-bit adder using two 4-bit adders.

7.a)	Explain DOT matrix display for displaying alphanumeric characters and special symbols. Design a suitable circuit to display ‘R’ using this display.

b)	It is desired to drive standard 74-series TTL gates from 74C-series CMOS gate using 74ALS-series TTL gates for interfacing. How many gates can be driven like this? Draw the interface circuit.

8.	Write short notes on:

a)	SCR triggering circuit.

b)	Crystal controlled clock generator.

c)	LCD display.

***     ***     ***

Code No.221103

Set No.

3

II-B.Tech. II-Semester Examination April/May 2003

DIGITAL CIRCUITS DESIGN AND APPLICATION

(Bio-Medical Engineering)

Time: 3 hours							Max.Marks:80

Answer any FIVE questions

All questions carry equal marks

---

1.a)	Under what conditions the high-pass RC circuit acts as a differentiator? Draw the output waveforms of RC differentiator circuit for a square wave input for small, medium and large time constants.

b)	A square wave whose peak-to-peak value is 1V extends  0.5V with respect to ground. The half period is 0.1sec. This voltage is impressed upon an RC differentiating circuit whose time constant is 0.2sec.  What are the steady-state maximum and minimum values of the output voltage?

2.a)	Draw the circuit of 555 timer as free-running multi vibrator. Draw the capacitor and output waveforms.

b)	Draw a circuit of a transistor as a switching element in a relay driver circuit. What are the desirable characteristics of transistor in switching applications.

3.a)	Design a digital comparator circuit to compare two single bit numbers.

b)	Design 4:1 multiplexer using NAND gates.

4.a)	Design RS flip-flop using two-input NOR gates.

b)	Design a decade counter using JK flip-flops.

5.a)	Explain the operation of 5-stage twisted ring counter.

b)	Design a circuit using 555 timer to divide the frequency of a train of pulses of frequency 10KHz by 3.

6.a)	Draw a typical clamping circuit using diode and illustrate its operation.

b)	Design a one digit BCD to binary converter using NAND gates.

7.a)	Explain the operating principle and applications of PLL.

b)	Explain the following terms with respect to digital IC’s

i)   Noise margin	(ii) Figure of merit	(iii)  Fan-in	(iv)  Fan-out.

8.	Write short notes on:

a)	Bounce free circuit.

b)	Relay driver circuits.

c)	Triggering methods for multivibrators.

***     ***     ***

Code No.221103

II-B.Tech. II-Semester Examination April/May 2003

Set No.

4

DIGITAL CIRCUITS DESIGN AND APPLICATION

(Bio-Medical Engineering)

Time: 3 hours							Max.Marks:80

Answer any FIVE questions

All questions carry equal marks

---

1.a)	A symmetrical square wave whose average value is zero has a peak-to-peak amplitude of 20V and a period of 2 microsec. This wave is applied to a low-pass circuit whose upper 3-dB frequency is 1/2 MHz. Calculate and sketch Peak-to-Peak output waveform.

b)	Draw the circuit of a Schmitt trigger and explain how it exhibits hysteresis.

2.a)	Draw the circuit of a simple current sweep circuit and illustrate the methods of linearity improvement of sweep.

b)	Draw the circuit of 555 Timer as a ramp generator. Draw the trigger and ramp output waveforms.

3.a)	Realize Y=A+B+C+D  using two input NOR gates only.

b)	Design a 1:40 demultiplexer using BCD-to-Decimal decoders 7442.

4.a)	Design a 2-bit comparator using gates.

b)	Design a clocked JK flip-flop.

5.a)	Design a retriggerable monostable using 555 timer and explain its operation.

b)	Draw the circuit of an astable multivibrator using AND gate and NOT gate and explain its operation.

6.a)	i)	Convert (0.6875)10 into octal.

ii)	Add (FFFF)16 and (COFF)16.

- Subtract (3F7A)16 from (5CDO)16.

- Design a 4-digit 7-segment LED display system.

7.a)	Draw the circuit of a double-ended clipper circuit using diodes and draw its transfer characteristics.

b)	Design Hexadecimal to BCD encoder using gates.

8.	Write short notes on:

a)	TTL  vs CMOS IC’s.

b)	LED vs  LCD displays.

c)	Applications of 555 timer.

***     ***     ***