Code No: 221201

Set No

1

II-B.Tech. II Semester Examination April/May, 2003

COMMUNICATION THEORY

(Computer Science and Information Technology)

Time: 3 Hours							Max. Marks: 80

Answer any Five questions

All questions carry equal marks

---

1.a)	Write the exponential form of the Fourier series and give the application of the same in communication theory.

b)	Find the trigonometric Fourier series of the rectified sine wave shown in the figure below.

2.a)	Evaluate the auto correlation of the delta function by considering it as the            limiting form of a rectangular pulse of unit area

b)	Consider a signal g(t) defined by

g(t)=Ao+A1cos(2f1t+) + A2cos(2f2t+).Determine the autocorrelation function Rg() of this signal

3.a)	Explain the working of balanced modulator with the help of circuit diagram.

b)	An AM produces 10kw when the modulation percentage is 75.  How much of this is carrier power?  What would be the percentage power saving if the carrier and one of the sidebands were suppressed?

4.a)	Explain the working of ratio detector for FM demodulation.

b)	The equation of an angle-modulated voltage is = 10 sin (108t + 3sin104t).  What form of angle modulation is this?  Calculate the carrier and modulating frequencies, the modulation index and deviation, the power dissipated in a 100-ohm resistor.

5.a)	Define the terms (i) Quantization Error and Inter symbol interference in Digital communication and discuss the reasons for their existence.

b)	What is Delta modulation?  Compare Delta modulation with PCM and bring out its merits and demerits.

6.a)	Define nyquist rate of sampling and derive the expression for it.

b)	A bandpass signal has a center frequency fo and extends from (fo-5) KHz to (fo+5) 	KHz.  If the sampling rate 25KHz, find the range of center frequency for which 	the sampling satisfies nyquist rate.

Contd…..2

Code No:221201			-2-					Set No:1

7.a)	An analog signal is band limited to 4 kHz, and is sampled at Nyquist rate.  If the samples are quantized into 4 independent levels,  with probabilities of  1/8,  3/8,  3/8,  1/8   respectively,  find the entropy and information rate.

What happens if the quantized levels occur with equally likely probabilities ?

b)	For an AWGN Channel with  8 kHz bandwidth and  noise power spectral density 	of  η = 2   pW/Hz,  calculate the capacity of the channel, if the signal power at the 	receiver is to be  100 µW.

8.a)	Write short notes on parity check bit codes, and list out the different types of such codes.

b)	Write short notes on:  Hamming Codes.

$$$

Code No: 221201

Set No

2

II-B.Tech. II Semester Examination April/May, 2003

COMMUNICATION THEORY

(Computer Science and Information Technology)

Time: 3 Hours							Max. Marks: 80

Answer any Five questions

All questions carry equal marks

---

1.	Draw the transfer characteristics of an idealized low pass filter and an idealized 	band pass filter with pass band from f1 to f2.    Explain why these filters cannot be 	realized physically.

2.a)	 Define cross-correlation of x(t) and g(t) and prove its properties.

b)	Find the cross correlation of the functions sin t and cos t.

3.a)	Explain with a circuit diagram the principle of square-law detector.

b)	Explain the principle of pre-emphasis and de-emphasis with a circuit.

4.a)	What are the advantages and disadvantages of FM over AM?

b)	Explain the working of Fooster-seely discriminator for FM demodulation.

5.a)	Specify various stages of generation of Pulse code modulation and explain the function of each stage  to improve the communication efficiency.

b)	The signal x(t) = 4 cos(2 t x 103) is to be encoded into 16 levels with a sampling rate of 4 KHz.

Find the (i) Number of bits required per sample.

(ii)	Quantization step size and

(iii)	Quantization noise level.

6.a)	Define various types of pulse modulations and distinguish between them.

b)	Explain clearly the differences of sampled signal and PAM and hence discuss the bandwidth criterion for PAM.

c)	Explain how signal can be recovered from PAM modulated signal.  Give necessary conditions.

7.	A discrete memoryless source transmits  8  messages   m1    ,  m2    ,  m3   , …m8

with probabilities  of   ½,  1/8, 1/8,  1/16,  1/16,  1/16,  1/32,  1/32  respectively.  Evaluate the channel capacity and coding efficiency using appropriate Shannon-Fano Coding.  Explain the principles involved,  and define the terms used.

Contd….2

Code No:221201			-2-					Set No:2

8.a)	Describe the characteristic feature of convolutional codes. List out the different 	types of convolutional codes and their applications.

b)	Obtain the code tree for the encoder shown below

$$$

Code No: 221201

Set No

3

II-B.Tech. II Semester Examination April/May, 2003

COMMUNICATION THEORY

(Computer Science and Information Technology)

Time: 3 Hours							Max. Marks: 80

Answer any Five questions

All questions carry equal marks

---

1.a)	Discuss the correspondence between signals and vectors and hence obtain Fourier series representation of signals over a finite time duration.

b)	Obtain the Fourier transform of given function.

f(t)

10

-T/2

0

T/2

t

2.a)	Show that a random variable with a Raleigh density has a mean

Value E(x)=   a mean square value  E(x2) =2 and   a

Variance Where  is a constant.

b)	Determine the mean and variance of the process given that the auto

Correlation function is     Rxx()  = 25 +( 4/1+6(Assume the process is 	stationary). Prove the corresponding relations.

3.a)	Derive an expression for the output power of AM transmitter in terms of depth of modulation.

b)	Explain the method of generating AM wave with its block diagram

4.a)	Explain with the help of diagram the working of ratio detector.  Also explain how the limiter circuit not required in this method.

b)	Compare FM and PM techniques.  Draw the phasor diagram and frequency spectrum of Frequency modulated wave.

5.a)	Explain the concept of quantization of sampled signal and hence discuss the merits and demerits of it.

b)	Give the block diagram for PCM generation and reception and discuss the features of each block.

6.a)	What is sampling?  Explain the need for sampling and hence discuss various types of sampling.

b)	Explain clearly the process of sampling for low pass signals and derive conditions for optimum reconstruction of signal.

Contd…..2

Code No:221201			-2-					Set No:3

7.a)	Explain the significance of the term – coding, and how coding can be used to increase the average information per bit.

b)	An AWGN Channel with  4 k Hz bandwidth has an SNR of 13 dB)  Calculate its channel capacity.  Find the required SNR in dB, to have the same channel capacity, if its bandwidth is increased to 6 kHz.

c)	Distinguish between the terms :  (i) symmetric and asymmetric channels,  and

(ii) Information sources with memory   and    without memory.

8.a)	With a block diagram, explain the generation of convolutional codes and obtain code tree diagram with an example.

b)	Explain clearly the procedure an nk-Hamming code with the help of relevant 	expressions for coding.

$$$

Code No: 221201

Set No

4

II-B.Tech. II Semester Examination April/May, 2003

COMMUNICATION THEORY

(Computer Science and Information Technology)

Time: 3 Hours							Max. Marks: 80

Answer any Five questions

All questions carry equal marks

---

1.a)	A  pulse extending from  0 to A volts and having a duration t is applied to a high-pass RC circuit. Show that the area under the response waveform is zero.

b)	Find the energy spectral density at the output and the relation between input and output energy densities.

2.a)	If characteristic functions of two variables are identical what will be the the 	relation between their distributions ?

b)	Consider the density function f (x)  = k x, 0 <x<1

=  0   elsewhere,

  - Evaluate k

  - Find P (0.3 < x < 0.6) and

  - Find the cumulative distribution function.

c)	A random process is given by v(t) = A cos ( t + ) Where  is a

random variable and A is a constant with probability density

P (

elsewhere

- Show whether V (t) is stationary or  not

If the random variable is replaced by a fixed angle and A is a random variable 	with Ganssian distribution, will v(t) be stationary.

3.a)	Explain the envelope detector with a circuit diagram and waveforms.

b)	Explain the principle of VSB transmission.  What are its applications?

4.a)	Explain the indirect method of FM generation with relevant diagrams.

b)	A 500Hz modulating voltage fed to a PM generator produces a frequency deviation of 2.25kHz.  What is the modulation index, if the amplitude of the modulating voltage is kept constant but its frequency is raised to 6kHz?  Determine the modified frequency deviation.

5.a)	Give the block diagram of Adaptive delta Modulation and explain the procedure for ADM generation.

b)	Discuss clearly the necessity for Linear predictor in DM and hence the improvements of it in case of ADM.

Contd….2

Code No:221201				-2-				Set No:4

6.a)	Explain the concept of Flat top sampling and obtain expression for the spectral density function Fs(w).  Hence discuss clearly the effect of ‘z’ pulse width on the Bandwidth.

b)	Explain clearly the procedure for signal recovery through holding.

7.a)	Compare discrete and continuous channel with respect to information transmission.

b)	In a facsimile transmission of picture, there are about 3 x 105 picture elements per frame. For a good reproduction, 10 brightness levels are necessary. Assume all these levels are equally likely to occur. Calculate the channel bandwidth required to transmit 30 picture frames per second. Assume the signal to noise ratio over channel to be 30 db (1000).

8.a)	Consider a generator matrix G for a non systematic (6,3) code.

011101

G = 	111010

110001

Construct the code for this G and show that dmin, the Minimum distance between the code words is 3 consequently this code can correct at least one error.

b)	Design a (7,4) cyclic encoder  with a generator polynomial 1+X+X3.

$$$