Set No.

1

Code No.: 220101

II B.Tech. Regular II-Semester Examinations April-May 2003

APPLIED ELECTRONICS

(Civil Engineering)

Time: 3 hours							Max. Marks: 80

Answer any five questions

All questions carry equal marks

---

1.a)	Draw the V-I characteristics of  p-n junction diode & explain its shape.

b)	Write the truth tables for

i)  AND gate	ii)  OR gate	iii)  NOT gate		iv) EX-OR gate.

2.a)	Compare CB, CE, & CC configurations.

b)	Compare & contrast silicon and germanium diodes.

c)	Explain how the temperature affects the V-I characteristics of the diode.

3.a)	Draw the circuit diagram of Full wave bridge rectifier  & explain its operation. Derive the expression for  ripple factor of this rectifier.

b)	For the following network IcQ=2mA and  VCEQ
             =10V.   Find R1 and RC

4.	For the network given below determine

a) re   	b) Zi	c)Zo	d) voltage gain Av   e)  current gain Ai  for ro=

Contd..2

Code No.: 220101				-2-			Set No.1

5.a)	Draw the circuit of a capacitor filter and explain its operation along with waveforms.

b)	Define the following terms.

i) Ripple factor		ii) Regulation		iii) PIV

6.a)	Compare & contrast positive and negative feedback circuits with respect to their input and output Impedances.  Voltage gain & Bandwidth.

b)	With the help of neat circuit diagram explain the operation of RC phase shift oscillator & derive the expression for frequency of oscillator.

7.a)	With neat block diagram explain operation of CRO.

b)	Derive the expression for electrostatic deflection sensitivity of a CRT.

8.	Write short notes on:

a)	LVDT

b)	Thermistor

c)	Piezo-electric Transducer.

&&&&&

Code No.: 220101

Set No.

2

II B.Tech. Regular II-Semester Examination April-May, 2003

APPLIED ELECTRONICS

(Civil Engineering)

Time: 3 hours							Max. Marks: 80

Answer any five questions

All questions carry equal marks

---

1.a)	Draw the V-I characteristics of Zener diode & explain its operation.

b)	Determine the voltage Vo for the network shown below.

Find also the currents Io,I1  flowing in the circuit.

2.a)	Draw the input & output characteristics of CE configuration & explain its shape.

b)	Explain the construction & working of JFET with the help of its output characteristics.

3.a)	Calculate the dc & ac components of the output signal across load RL in the circuit.  Calculate the ripple of the output waveform.

Vdc=150v, Vr(rms)=15v at the output of FWR.

b)	Explain different types of  regulation & compare them.

Cont..2

Code No.: 220101				-2-				Set No.2

4.a)	For the fixed biased circuit shown.  Find

i)  IBQ    &    ICQ

ii)   VCEQ

iii)  VBC

b)	Compare  & Contrast positive & negative feedback circuits with respect to their Input & Output impedances voltage gain & bandwidth.

5.a)	With neat circuit diagram explain the  operation of Hartley oscillator & derive the expression for frequency of oscillator.

b)	Write short notes on crystal oscillator.

6.a)	With block diagram explain the operation of PH meter.

b)	Derive the expression for electrostatic deflection sensitivity of CRT.

7.a)	Explain where transducers are used & its principle of  operation.

b)	With neat block diagram explain how thermistor is used for temperature measurement.

8.	Write short notes on

i)    Load & Line Regulation.

ii)   Universal gates.

iii)   Avalanche & Zener breakdown.

&&&&&

Code No.: 220101

Set No.

3

II B.Tech. Regular II-Semester Examination April-May 2003

APPLIED ELECTRONICS

(Civil Engineering)

Time: 3 hours							Max. Marks: 80

Answer any five questions

All questions carry equal marks

---

1.a)	Compare Avalanche and Zener break down.

b)	Explain how a zener diode is used as a Regulator.

2.a)	Draw the piece-wise Linear equivalent circuit,  simplified equivalent circuit & ideal equivalent circuit of p-n diode & explain.

b)	Determine Vo for the following circuit.

3.a)	Draw the block diagram of a regulated power supply and explain each block.

b)	Compare & contrast  CB, CE & CC amplifiers & list out their applications.

4.a)	Draw the circuit diagram of colpitts oscillatior & derive the expression for frequency of oscillation.

b)	Draw the block diagram of voltage series feedback amplifier & derive the expression of voltage gain, input impedance & output impedance.

5.a)	Draw the equivalent circuit of CE h-parameter model & derive the expression for current gain.  Input & output impedances & voltage gain.

b)	Compare & contrast positive & negative feedback amplifier.

6.a)	Derive the expression for magnetostatic deflection sensitivity of CRT.

b)	 Draw the block diagram & principle of operation of multimeter.

7.a)	Explain what is a transducer & explain the operation of LVDT.

b)	With block diagram explain the operation of PH meter.

8.	Write short notes on:

a) Accuracy, resolution & precision	b) Crystal oscillators.	c) Universal gates.

&&&&&

Set No.

4

Code No.: 220101

II B.Tech. Regular II-Semester Examination April-May 2003

APPLIED ELECTRONICS

(Civil Engineering)

Time: 3 hours							Max. Marks: 80

Answer any five questions

All questions carry equal marks

---

1.a)	Compare & contrast FET & BJT.

b)	What are universal gates.  Implement Ex-OR & OR operations using only NAND gates.

2.a)	Explain what do you mean by Avalanche & Zener break down.

b)	Determine Vo, IO  & I1  for the circuit shown below.

3.a)	Design an emitter bias circuit for the following  specifications.

Vcc=12V, VCE=2v,  IC=4MA,   HFE=80.

b)	Calculate the output voltage and zenor current in regulator circuit for RL=1k

Cont..2

Code No.: 220101				(2)				Set No.4

4.a)	Draw an voltage series feedback amplifier & derive the expression for voltage gain.

b)	Design a phase shift oscillator using FET having 9m=5000s, rd=40K.  Select & feedback circuit value of 10K. Select the value of C for oscillator operation at 1KHZ & RD= for A>29 to ensure oscillator action.

5.a)	Draw an RC coupled two stage amplifier and derive the expression for voltage  gain input impedance and output impedance.

b)	Compare the performance    of CB, CE, CC and amplifier  and their application.

6.a)	Briefly explain the applications of CRO..

b)	Explain the principle of operation of  PH  meter with block diagram.

7.a)	Explain the following terms:

i) Accuracy  ii)  Resolution   iii) Precision   iv) linearity  in measurements.

b)     Explain the operation of strain gauge with block diagram.

8.	Write short notes on:

a)  	Crystal Oscillator

b)  	Comparison of BJT & FET

c)	LVDT.

&&&&&