Code No:221102

Set No:

1

II B.Tech. II-Semester Examination, April/May 2003

COMMUNICATION ENGINEERING

(Bio-Medical Engineering)

Time: 3 hours							Max.Marks:80

Answer any FIVE questions

All questions carry equal marks

- - -

1.a)	Write the expressions for AM,DSB-SC and SSB signals.  Draw their time and frequency plots.

b)	An AM wave 101+0.6cos2000πt cos 200000πt is to be detected by a linear diode detector.  Find the time constant and the resistance for C= 100PF.

c)	Describe the generation of VSB signal.

2.a)	Derive the expression for wideband FM signal.

b)	How is FM generated indirectly.

c)	A modulating signal 5 cos 30000 πt angle modulates a carrier A cos 2πfct.  Assume Kδ=Kp = 15 KHz/volt.  Determine the modulation index and bandwidth for FM and PM signals.

3.a)	Draw the block diagram of a highlevel AM radio transmitter and explain.

b)	List out and compare the frequency allocations of AM and FM broadcast standards.

c)	Draw the FM radio transmitter block diagram and show how the indirect FM signal is transmitted.

4.a)	Describe the tracking principle of the superhet receiver.

b)	With the help of circuit schematic, explain the principle of AGC in a radio receiver.  What is the advantage of delayed AGC?

c)	What is the cause for fading of radio signals?  Show that diversity reception can suppress its effects.

5.a)	Prove the sampling and recovery using only frequency domain plots.

b)	With neat schematic, explain the generation of PPM signal.

c)	Explain the DPSK signal generation.

6.a)	Briefly describe the serial interface standard.

b)	How are modems interfaced to a communication network.

c)	Introduce the essentials of circuit switched networks.

Contd….2

Code No:221102			-2-				Set No:1

7.a)	Define shot noise, solar noise and transit time noise.

b)	Determine the output noise PSD of an ideal LPF excited by a white noise with constant PSD.

c)	An amplifier has input signal power of  5 W, noise input power of 1 W and the power given of 40 dB.  Determine the Noise figure of it.

8.	Write short notes on any THREE of the following

a)	PM

b)	Amplitude limiter

c)	DM

d)	Adapters.

**********

Code No:221102

Set No:

2

II B.Tech. II-Semester Examination, April/May 2003

COMMUNICATION ENGINEERING

(Bio-Medical Engineering)

Time: 3 hours							Max.Marks:80

Answer any FIVE questions

All questions carry equal marks

- - -

1.a)    Derive the Expression for AM and DSB-SC signals.

b)   Sketch the spectrum of s(t)=50(1+0.7 cos 500t – 0.3 cos 1000t)x sin 100000 πt.

Evaluate the modulated and sideband powers.

- With the help of neat block diagram, explain the generation and detection of SSB singnal.

2.a)	Illustrate the relation between frequency and phase and hence show the interconversion between FM and PM utilizing this concept.

b)	How is Narrow Band FM generated?

c)	An FM signal is given by s(t) = 2 cos 20000πt+cos 2000πt + 3 cos 40000πt.  Determine the bandwidth and  β assuming Kf=104 Hz/volt.

3.a)	Explain and compare the low level and highlevel AM radio transmissions.

b)	List out the frequency bands of electromagnetic spectrum and give their applications.

c)	Draw the block diagram and explain the Armstrong FM radio transmitter.

4.a)	Explain the operation of TRF receiver.  Mention its demerits.

b)	Define the selectivity, sensitivity and  Fidelity of a radio receiver.

c)	Brief the salient features of a communications receiver.

5.a)	Explain the generation of PAM signals practically.

b)	Draw the block diagram and explain about ADM.

c)	Sketch the waveforms and signal constellationo diagrams for ASK,PSK and FSK signals.

6.a)	Distinguish between synchronous and asynchronous communications.

b)	Classify communication media and compare their bandwidths.

c)	Present the basic principle of packet switching.

7.a)	Classify noise sources and give their PSD.

b)	Find out the output noise PSD  for an RCLPF with an input noise PSD of 1 W.

c)	Define SNR and Noise Figure.  How is NF useful in identifying a receiver’s noise performance?

8.	Write short notes on any THREE of the following:

a) Emphasized FM  (b) Buffer amplifier. (c) Tone control (d) TDM

**********

Code No:221102

Set No:

3

II B.Tech. II-Semester Examination, April/May 2003

COMMUNICATION ENGINEERING

(Bio-Medical Engineering)

Time: 3 hours							Max.Marks:80

Answer any FIVE questions

All questions carry equal marks

- - -

1.a)	A 10V, 1MHz sinusolid is modulated by a sinusoid modulation signal with A1=1V, f1=1KHz and A2 = 1V, f2=10KHz.  Sketch the spectra of corresponding AM, DSB-SC and SSB signals.

b)	Discuss the generation of DSB signal using Balanced modulator.

c)	Explain envelope detection process with neat waveform.  What is the constraint on the time constant of the circuit?

2.a)	Describe the generation of FM signal using reactance modulation.

b)	Give the expressions for NBFM and WBFM signals for single tone and two tone modulations.

c)	Draw the preemphasis and deemphasis circuits and their frequency responses.

3.a)	Explain the role of class C amplifier in radio transmitter.  Give the circuit and explain.

b)	List the specifications for FM broadcast standard.

c)	Draw the block diagram of direct FM radio transmitter and explain.

4.a)	Explain the importance of 1F section.  Why is 1F a fixed frequency?

b)	Draw the block diagram of FM receiver and explain.

c)	Give the principle of volume control in a radio receiver.

5.a)	Present the principles of TDM  and FDM.

b)	Discuss about Quantization and coding in PCM.

c)	Distinguish between binary and 8 phase PSK signals.

6.a)	Draw the basic block diagram and explain the data communication network.

b)	What is a modem?  How is it interfaced to a host and the network?

c)	Compare circuit switching and packet switching networks.

7.a)	Explain the thermal noise and its characteristics.

b)	Define noise PSD.  Give it for a white noise and colored noise.

c)	Derive the Noise Figure for a cascaded system.

8.	Write short notes on  any THREE of the following.

a)	Ring modulator  	(b)	Ratio detector

c)	PWM			(d)  	Image frequency.

*********

Code No:221102

Set No:

4

II B.Tech. II-Semester Examination, April,May 2003

COMMUNICATION ENGINEERING

(Bio-Medical Engineering)

Time: 3 hours							Max.Marks:80

Answer any FIVE questions

All questions carry equal marks

- - -

1.a)	Bring out the need and benefits with modulation.  Classify modulation techniques.

b)	With neat schematic, explain the generation of VSB signal.

c)	What is the effect of phase shift in the local carrier on demodulation of DSB-SC signal?

2.a)	With the help of neat block diagram, explain Armstrong FM generation.

b)	Draw the circuit and explain FM demodulation by ration detector.

c)	An FM signal is 10 sin (16πx106t + 20 sin 2πx103 t).  Find the modulation index and power of FM signal.

3.a)	Explain  the exciter section of a highlevel AM transmitter.

b)	List the broadcast standards for  AM radio.

c)	Draw the block diagram of FM radio transmitter and explain.

4.a)	Draw the block schematic of superhet receiver.  Give the circuit of mixer and explain.

b)	Discuss about alignment of radio receiver in RF and LO sections.

c)	Explain the principle of muting and BFO.

5.a)	Describe the process of flat top sampling.  Mention its drawback.

b)	Explain the generation of DM signal.

c)	Sketch the waveforms and phasor diagram for PSK and ASK signals.

6.a)	With the help of neat block schematic explain the working detail of a telephone modern.

b)	Explain the synchronous data communication.

c)	Present the role of switching in computer communication networks.

7.a)	Discuss about external noise in detail.

b)	What is white noise?  Present its time and frequency domain characteristics.

c)	Define Noise Figure.  Mention the value for an ideal receiver.

8.	Write short notes on any THREE of the following:

a)	Balanced modulator

b)	Amplitude limiter

c)	Choice of IF

d)	Multiplexing.

*********