Code No. 220252

OR

II-B.Tech. II-Semester Supplementary Examination April/May 2003

THERMODYNAMICS

(Electrical and Electronics Engineering)

Time: 3 hours							Max. Marks: 70

Answer any Five questions

All questions carry equal marks

---

1. a)	What do you mean by property?  Distinguish between intensive and extensive properties.

b)	Explain the ‘First Law of Thermodynamics’ as referred to closed systems undergoing a cyclic change.

c)	A compressor draws in 720m3/hr of air at p1 = 1 bar and t1 = 22oC and discharges at p2 = 5 bar and t2 = 90oC.  Heat rejected by air to cooling medii is 30kJ/kg.  Determine the shaft work-done an air in kJ/kg.  Change in kinetic energy and potential energies may be neglected.  Determine also the mass of air handled.  Assume R=0.2868kJ/kg oK, Cv = 0.718kJ/kg oK

2. a)	What do you mean by throttling process.

b)	For polytropic process prove the following equation for a closed system (i.e, non-flow process)     W=P1V1−P2V2n−1.

c)	1m3 of air at a pressure of 2 bar and a temperature of 35oC is compressed to a volume of 0.2m3 according to the law P V1.2 = constant.  Find the work-done during the process.

3. a)	What do you mean by a phase of a substance?

b)	Represent the following processes on T-S and h-s diagram for steam.

c)	Calculate th enthalpy, entropy and internal energy for each of the following.

i)	5kg of steam at 15 bar and 0.7dry.

ii)	5kg of steam at 2 bar and 0.8 dry.

iii)	5 kg of steam at 15 bar and 400oC.

4. a)	What do you mean by air standard cycles?  What are the assumption for air standard cycles?

b)	 Derive an expression of or the efficiency of the diesel cycle.

c)	In a carnot cycle, the maximum pressure and temperature of the cycle are 17.5 bar and 175oC.  The isothermal expansion ratio is 3 while isentropic expansion ratio  is 2.  Calculate the thermal efficiency and mean effective pressure.

5. a)	Discuss the important points of difference between petrol and diesel engine.

b)	Describe the working of a two stroke cycle internal combustion engine with the help of a diagram.

Contd…2

Code No. 220252				.2.					OR

6. a)	Explain various types of nozzles and their distinguishing features.

b)	A single stage impulse turbine rotor has a diameter of 1.2m running at 3000 r.p.m.  The nozzle angle is 18o.  Blade speed ratio is 0.42.  The ratio of the relative velocity at outlet angle of the blade is 3o smaller than the inlet angle.  The steam flow rate is 5kg/s.  Draw the velocity diagram and find.

(i) velocity a whirl 			ii) Blade angle

iii) Power developed.

7. a)	Differentiate between closed end open cycle gas turbines.

b)	Explain with the help of a neat sketch the working of the open cycle gas turbine.

8. a)	Explain the difference in valve mechanism of internal combustion engine and reciprocating air compressor.

b)	Explain the working of a single stage reciprocating air compressor with the help of a line diagram.

###