Code No.221854

OR

II-B.Tech. II-Semester Examination April/May 2003

FLUID MECHANICS

(Metallurgical and Material Technology)

Time: 3 hours							Max.Marks:70

Answer any FIVE questions

All questions carry equal marks

---

1.a)	Graphically represent and give one example of Newtonian fluid, pseudoplastic, dilatant, ideal plastic and thyxotropic fluid.

b)	Through a very narrow gap of height h, a thin plate of very large extent is being pulled at constant velocity V.   On one side of the plate is oil of viscosity  and on the other side oil of viscosity K.  Calculate the position of the plate so that the drag force on it will be a minimum.

2.a)	Define and distinguish between stream lines, streak lines and path line.  When do these three lines coincide?

b)	For the following velocity vectors, determine the magnitude of the velocity at A(x=2, y=-3,Z =1, t = 2)

i)	V  =  (10t + xy) i  + ( - yz –10t )  j  + ( - yz + z2/2) k

ii)	V =  4x i  + ( - 4y + 3t) j

3.a)	Define and Derive energy correction factor.

b)	A pipe 200 m long slopes down at 1 in 100 and tapers from 800 mm diameter at the higher end to 400 mm diameter at the lower end and carries 100 lps of oil

(S = 0.85). If the pressure gauge reading at the higher end reads 50 kN / m2, determine, 	(i) Velocities at the two ends and

(ii) pressure at the lower end. Neglect losses.

4.a)	How do you explain the difference between surface drag, form drag and deformation drag and give one example each.

b)	A kite weighing 12 N has an effective area of 1 m2. It is maintained in air at an angle of 10o to the horizontal. The string attached to the kite makes an angle of 45o to the horizontal and at the position the values of CD and CL are 0.6 and 0.8 respectively. Find the speed of the kite and tension in the string. Take unit weight of air = 12.5 N / m3.

5.a)	What is meant by co-efficient of compressibility?

b)	A diffuser of area ratio 2 :1 operates at the inlet condition

P1= 500 kN/m2 ,T1= 500 K , M1 = 0.6 ,= 1.4 .Estimate the

following at the exit

(i) 	velocity 	(ii) 	pressure

(iii) 	Temperature  	(iv)	Mach number

Contd….2

Code No.221854				-2-					OR

6.a)	Explain the velocity and shear stress distribution for laminar flow in a circular pipe with a neat sketch.

b)	A fluid of mass density 1790 kg/m3 and viscosity of 2.1 pascal-sec flow at a velocity of 3 m/sec in a 6 cm diameter pipe.  Estimate the head loss in a length of 12 m of pipe.

7.a)	Derive an expression for loss of head due to sudden contraction of a pipe. Sketch the flow pattern and show how the hydraulic gradient and total energy line vary as the flow takes place though the contraction

b)	Water flows through a 25cm diameter which is suddenly contracted to 10cm diameter . The flow through the pipe is 75lit/sec. If coefficient of contraction is 0.62, determine the head loss. The pressure of water before the contraction is 80kPa. What is the pressure after contraction

8.	Explain venturimeter in detail with diagram.  Also derive an expression for finding out the actual discharge from a given venturimeter.

$$$