Code No.220203

Set No.

1

II-B.Tech. II-Semester Examination April/May 2003

LINEAR AND DIGITAL I.C. APPLICATIONS

(common to Electrical and Electronics Engineering, Electronics and Computer Engineering)

Time: 3 hours							Max.Marks:80

Answer any FIVE questions

All questions carry equal marks

---

1.a)	Define input offset voltage and CMRR as applied to opamp ICs.

b)	Explain how the above parameters can be measured?

2.a)	Define slew rate and derive the expression for it. List causes of the slew rate and explain its significance in applications.

b)	Explain the difference between slew rate and transient response.

3.a)	Explain  the  operation  of  Monostable  multivibrator  using  555 timer. Derive the  expression of time delay of a Monostable multivibrator using 555 timer.

b)	Design a Monostable multivibrator using 555 timer to produce a pulse width of 	100 m sec.

4.a)	What is all pass filter? Explain.

b)	Design a narrow band pass filter using one op-Amp. The resonant frequency is 128Hz and Q=1.5. Select C=0.1F.

5.a)	Draw the second order high pass butter worth filter circuit and it’s frequency response.

b)	For the above circuit C2 = C3 = 0.047 F, R2 = 3.3kilo ohm,R1 = 27 kilo ohm and  Rf  = 15.8 kilo ohm. Determine the lower cutoff frequency fL of the filter.

6.a)	With the help of neat circuit diagram explain the operation of a DTL NAND gate

b)	List out the important properties of DTL circuit.

7.a)	List out the advantages , disadvantages & application of ECL logic.

b)	Show how TTL & MOS gates can be interfaced with ECL gates.

8.a)	Draw and compare the conversion times for tracking and successive           approximation ADC devices.

b)	A  dual  slope  ADC  uses  a  12bit counter and a 8 MHz clock rate. The max 	input voltage is +10V. The max. integrator o/p voltage should be -8V, when the 	counter  has cycled through 2n counts. The capacitor used in the integrator is 0.1 	micro farad. Find the value of the resistor of the integrator.

$$$

Code No.220203

Set No.

2

II-B.Tech. II-Semester Examination April/May 2003

LINEAR AND DIGITAL I.C. APPLICATIONS

(common to Electrical and Electronics Engineering, Electronics and Computer Engineering)

Time: 3 hours							Max.Marks:80

Answer any FIVE questions

All questions carry equal marks

---

1.a)	Briefly explain the characteristics of an ideal opamp.

b)	What do you mean by input offset voltage and input offset current of an opamp?

Explain and give the experimental setup to measure these parameters.

2.a)	Design a practical integrator circuit to properly process input sinusoidal waveforms upto 1 KHz. The input amplitude is 10mV.

b)	Design a differentiator that will differentiate an input signal with fmax=100Hz.

3.a)	Explain the operation of Astable multivibrator using 555 timer.

b)	Design a square waveform generator of frequency 100 MHz and duty cycle

of 75% using 555 timer.

4.a)	Design suitable component values for a low pass filter to achieve a corner frequency of 1kHz with a dc gain of 20dB and an input impedance of at least 10k.

b)	Calculate the frequency at which gain drops to unity.

5.a)	Design a wide band reject filter using first order high pass and low pass filter having fL = 2kHz and fH = 400 hz respectively.

b)	Draw an approximate frequency response plot for the above filter.

6.a)	With neat sketch explain the operation of TTL NAND gate.

b)	A two input NAND gate has Vcc = +5v  & 1 K ohm  load connected to its

output. Calculate the output Voltage (i) when both inputs are low. (ii) When both 			inputs are high.

7.a)	Define edge trigged flip flops.

b)	Draw the circuit of edge trigged SR flip flop made up of by basic gates &

explain the operation . Sketch the wave form.

8.a)	A counting type ADC uses a 8bit DAC. The MSB of DAC output voltage is +5V

i)	 If the analog i/p voltage is +6.85 v, what will be the R-2R ladder o/p voltage when the clock stops?

ii)	What is the no. of clock pulses that occur between the release of reset and   stopping of   the clock?

b)	 Calculate the values of the LSB, MSB ad full scale o/p for an 8 bit DAC for the

0 to 10 V  range.

$$$

Code No.220203

Set No.

3

II-B.Tech. II-Semester Examination April/May 2003

LINEAR AND DIGITAL I.C. APPLICATIONS

(common to Electrical and Electronics Engineering, Electronics and Computer Engineering)

Time: 3 hours					 		Max.Marks:80

Answer any FIVE questions

All questions carry equal marks

---

1.a)	Why is it necessary to use an external offset voltage compensating network with practical opamp circuits?

b)	Compare and contrast an ideal opamp and practical opamp.

c)	Explain the precautions that can be taken to minimize the effect of noise on an opamp circuit.

d)	Calculate the effect of variation in power supply voltages on the output offset voltage for an inverting amplifier circuit.

2.a)	Draw the schematic diagram of Wien Bridge Oscillator and derive the expression for frequency of oscillation.

b)	What are the conditions to be satisfied by a circuit to produce oscillations?

3.a)	Explain the role of the basic building blocks of PLL.

b)	Determine the DC control voltage vc at lock if signal frequency fs = 10 KHz,

VCO free running frequency is 10.66 KHz and the voltage to frequency transfer

co-efficient of VCO is 6600 Hz / v.

4.a)	Design suitable component values to achieve a band pass response with a gain of 20dB over the audio range.

b)	Sketch the frequency response of this filter in the frequency range 0-1MHz

5.a)	Design a multiple feed back narrow band pass filter for fC = 1KHz ,Q = 3 and

AF = 10

- Change the centre frequency to 1.5KHz keeping AF  and the bandwidth constant for the above filter.

6.a)	What are the draw backs of TTL gate? explain how they are overcome using schottky clamp?

b)	Compare the various parameters of all TTL series NAND gates.

7.a)	Explain with an example why asynchronous i/p are required in flip flops.

b)	Explain the operation of edge triggered T flip- flop.

8.	Write short notes on the following:

  - Multiplying ADC.

  - Applications of analog multiplexers.

  - Semi tracking A/D converters.

$$$

Code No.220203

Set No.

4

II-B.Tech. II-Semester Examination April/May 2003

LINEAR AND DIGITAL I.C. APPLICATIONS

(common to Electrical and Electronics Engineering, Electronics and Computer Engineering)

Time: 3 hours							Max.Marks:80

Answer any FIVE questions

All questions carry equal marks

---

1.a)	Discuss about the practical differentiator and derive the necessary expression.

b)	Design an opamp differentiator that will differentiate the input signal with

fmax = 100 Hz.

2.a)	What are the advantages of adjustable voltage regulator over the fixed regulator?

b)	Explain the importance of voltage references and voltage inverters.

c)	Explain the operation of a phase detector.

3.a)	What are the two basic modes in which the 555 timer operates? Briefly explain

the differences between the two operating modes of the 555 timer.

b)	Design a ramp generator using 555 timer having an output frequency of

approximately 5KHz.

4.a)	Sketch the frequency response of various second order filters.

b)	Briefly explain the steps involved in constructing phase plots of filters.

5.a)	The cutoff frequency of a certain first order low pass filter is 2 KHz Covert this low pass filter to have a cutoff frequency of 3KHz by using the frequency scaling technique.

b)	What is the butter worth response?

6.a)	With neat circuit explain the operation of  TTL inverter.

b)	Briefly explain the different types of  TTL gates

7.	What do you understand by the term “Race around” in JK flip flop? Explain how

this is removed in a master slave F/F with the circuit diagram.

8.a)	Explain in detail the following characteristics of Analog to Digital converter

i)	 Resolution

ii)	 Quantization error

b)	An 8 bit ADC is capable of accepting an input  unipolar (positive values only) voltage 0 to 10V.

i)	 What is the minimum value of 1 LSB

ii)	What is the digital output code if  the applied input voltage is 5.4V?

$$$