Code No. 221901

Set No.

1

II-B.Tech. II-Semester Examinations April/May, 2003

DATA COMMUNICATIONS

(Electronics and Computer Engineering)

Time: 3 hours								Max. Marks: 80

Answer any Five questions

All questions carry equal marks

---

1.	For the QPSK modulator in the figure  determine the I and Q bits for an input of  a= + sinwct- coswct. Explain the functioning of this QPSK modulator.

I Channel			a sinwct

I-Product

Detector

LPF

a

Input											         -vlogic0/2

BPf

Power

Splitter

Carrier

Recovery

(sinwct)

QPSK

Signal												      RBD

+90o

Q    I

+vlogic1/2

a			 	  a coswct

Q-Product

Detector

LPF

Q Channel

Where	a= (- sinwct+ coswct);  RBD=Reactive Binary Data

2. a)   	Derive the output voltage relationship of an FSK modulator.

b) 	Explain coherent demodulator and tribit.

3. a) 	What are the characteristics of a serial interface?

b)  	Define Baud rate

c) 	What are the applications of USART?

4. a) 	Explain the various parts of a terminal

b)  	Explain how to construct the Hamming code for the bit sequence 10011101.

5. a)	What are the basic features of IBM’s bisync protocol?

b)	Explain bisync frame format with neat diagram

c)	Explain how bisync protocol is used for point-to-point communication.

6.	What is codes and combo chips?  Explain the major function of combo chips and draw the block diagram of combo chip.

7.	Explain in detail statistical TDM.  Compare it with synchronous TDM.

8.	Illustrate the point-to-point block transfer sequences in basic mode half duplex protocol.

###

Code No. 221901

Set No.

2

II-B.Tech. II-Semester Examinations April/May, 2003

DATA COMMUNICATIONS

(Electronics and Computer Engineering)

Time: 3 hours								Max. Marks: 80

Answer any Five questions

All questions carry equal marks

---

1.	For the QPSK modulator shown in the figure change the +900 phase shift network to -900 and construct the truth table, phasor diagram and constellation diagram. Explain its functioning.

I channel        fb /2

fb                                                logic 1=+1v	          I-Balanced	+,- sinwct

Input        1	          logic 0=-1v             Modulator

Buf-                         Reference                     sinwct

fer           0               Carier

Oscillator           900 phase               Linear                     QPSK

Bit             +2      fb /2       sinwct                   shift                    Summer     BPF        output

Clock			logic 1=+1v

logic 0=-1v         Q-Balanced

Q channel   fb /2    Modulator	+,-coswct

2. a)   	Describe the bandwidth relationship of an FSK modulation scheme.

b)	Explain frequency deviation and modulation index for FSK modulation.

3.	Explain 2 wire and 4 wire operations of transmission mode in data communication with examples.

4.	Write short notes on the following

- CRT terminals

- PC terminals

5. a)	Discuss the merits and demerits of synchronous protocol.

b)	Explain the different fields of bisync frame.  Mention the number of bits required for each field.

6.	Explain in detail line encoding with block diagram.

7.	Explain muldem, digital cross connect.  Tabulate the digital signal, bit rate, channel capacities and service offered to various line types.

8.	Explain fully the full duplex point to point protocols.

###

Code No. 221901

Set No.

3

II-B.Tech. II-Semester Examinations April/May, 2003

DATA COMMUNICATIONS

(Electronics and Computer Engineering)

Time: 3 hours								Max. Marks: 80

Answer any Five questions

All questions carry equal marks

---

1.	For the QPSK modulator shown in the figure change the reference Oscillator to coswct  and construct the truth table, phasor diagram and constellation diagram. Explain its functioning.

I channel        fb /2

fb                                                logic 1=+1v	          I-Balanced	+,- sinwct

Input        1	          logic 0=-1v             Modulator

Buf-                         Reference                     sinwct

fer           0               Carier

Oscillator           900 phase               Linear                     QPSK

Bit             +2      fb /2       sinwct                   shift                    Summer     BPF        output

Clock			logic 1=+1v

logic 0=-1v         Q-Balanced

Q channel   fb /2    Modulator	+,-coswct

2.	Describe the difference between FSK and MSK modulation schemes.

3. a) 	What is the use of transmission mode?

b) 	Write the use of an acoustic coupler?

c) 	What is the need for synchronization in data communication? Explain

4.	Explain the operation of different codes used in data communication.

5. a)	Explain briefly the different synchronous protocols in brief for data communication.

b)	Which mechanism bisync protocol uses for the flow control?  Explain with an example.

6.	Describe the super frame format with block diagram.  Why it is needed?

7.	Describe the process of sampling data signals.

8.	Write about full duplex multipoint protocols.

###

Code No. 221901

Set No.

4

II-B.Tech. II-Semester Examinations April/May, 2003

DATA COMMUNICATIONS

(Electronics and Computer Engineering)

Time: 3 hours								Max. Marks: 80

Answer any Five questions

All questions carry equal marks

---

1.	Explain the functioning of QPSK modulator given in the figure below. Compute the output of I product detector and Q product detector.

I Channel			a sinwct

I-Product

Detector

LPF

a

Input											         -vlogic0/2

BPf

Power

Splitter

Carrier

Recovery

(sinwct)

QPSK

Signal												      RBD

+90o

Q    I

+vlogic1/2

a			 	  a coswct

Q-Product

Detector

LPF

Q Channel

Where	a= (- sinwct+ coswct);  RBD=Reactive Binary Data

2.	Describe the characteristics of a phase – shift – keying (PSK) modulation scheme.

3. a) 	Write short notes on portable terminals

b)  	What are the steps required for synchronization?

4.	Explain the different types of data modems

5.	Explain the different phases of the operation of bisync protocol.

6.	Explain Line coding in detail with block diagram.

7.	Explain in detail the various types of hybrid data with block diagram.

8.	Compare the protocol sequences of half and full duplex for a four wire multipoint line?

###