Code No.:221101

Set No.

1

II-B.Tech. II Semester Regular Examinations April/May, 2003

PROBABILITY AND RANDOM VARIABLES

(Bio-Medical Engineering)

Time: 3 hours.							      Max. Marks: 80

Answer any FIVE questions

All questions carry equal marks

- - -

1.a)	Define the following:

i)	Discrete Random Variable

ii)	Joint pdf

- Expected value of a function of a random variable.

- A joint density function is specified as

f(x,y) = xe-x(y+1)	in the range of 0  x, y  

= 0 		otherwise

find f(x), f(y) and their independence.

2.a)	Determine the expected value of function g(X) = X2 where X is a random variable with pdf f(x) = 12u(x)e−x2

b)	A random variable X has the pdf

f(x) = π6cosπx8	-4  x   4

= 0		else where

Find E(X), E(X2) and X2.

3.a)	Present the importance of characteristic function. Determine it for a uniform random variable X in [a, b].

b)	Two random variables X and Y are having joint pdf

f(x, y) = 10x-5y.	for o < x,y < 

= 0		otherwise.

Determine the mean and variance of each.

4.a)	Define the following with respect to random processes

i)	Ensemble			ii)	Statistical Correlation

iii)	Power spectral density	iv)	Power transfer function

b)	Z(t) = M(t) cos (ot + )

M(t) is a random process with zero mean and unit variance. Find E(Z2) if  is zero. Is Z(t) stationary?

Contd…2

Code No.: 221101				-2-				      Set No.1

5.a)	A Gaussian random process with zero mean and unit variance is passed through a linear system that has the power transfer function given by

G(f) = 1	0  f  fc

= 0	otherwise

Determine the output autocorrelation.

b)	Define the following with respect to noise.

i)	noise PSD		ii)	noise bandwidth

iii)	Noise figure		iv)	quadratine components.

6.a)	Distinct between available noise power and available noise power spectral density. How are they determined?

b)	Define Noise Figure. Derive it for a 5 stage cascaded electronic system.

7.a)	Derive the channel capacity for a Gaussian noise limited channel. Compare this with the Shannon Hartley theorem.

b)	Consider the source alphabet of 5 symbols with probabilities ½, ¼, 1/8, 1/16 and 1/16. Calculate H. Use any source coding technique and find average number of bits per symbol. Compare with H.

8.	Write short notes on the following:

a)	joint and conditional density functions.

b)	Central limit theorem.

c)	Mutual Information.

^^^

Code No.:221101

Set No.

2

II-B.Tech. II Semester Regular Examinations April/May, 2003

PROBABILITY AND RANDOM VARIABLES

(Bio-Medical Engineering)

Time: 3 hours.							      Max. Marks: 80

Answer any FIVE questions

All questions carry equal marks

- - -

1.a)	State the properties of the distribution and density functions. Relate these two functions for a random variable X.

b)	If the following function were to be a valid density function, determine the real constant a

fx = ae−x−mb

m and b are real constants greater than zero.

2.a)	A random variable X has the pdf fx = 12 u (x) e−x2 Find the probabilities

p[1<X3], p[X 2.5] and p[1<X3] p[X2.5].

b)	A random variable X represents the value of coins (in paise) given in the change when purchases are made at a retail store. Suppose that the probabilities of 5, 10, 20, 25 and 50 being present in the change are 0.35, 0.25, 0.20, 0.15 and 0.05 respectively. Determine f(X), X and X2.

3.a)	Show that the characteristic function of a random variable X having binomial density is

x (w) = [1-p+pejw]N.

b)	Present the concept of correlation between random variables. Explain how do you quantify this measure, using simple illustration.

4.a)	Define the following with respect to random processes.

i)	sample function		ii)	auto correlation

iii)	random pulse sequence	iv)	power spectrum.

b)	A time function Z(t) = X coswot – Y sin wot is a random process. If X and Y are independent Gaussian random variables each with zero mean and variance 2, find E(Z) and fZ(z).

5.a)	A random process X(t) has the power spectrum.

SX(w) = 6w21+w23

This is applied to an ideal differentiator. Find the output power spectrum.

b)	Illustrate the generation and statistical properties of any two internal and two external noises for a communication system.

Contd…2

Code No.: 221101				-2-				      Set No.2

6.a)	an antenna has a noise temperature of 4K. It is connected to a receiver which has an equivalent Te = 100K. The mid band available gain of the receiver is gao = 1010. It can be represented by a parallel RLC filter with a 3 dB bandwidth of 10 MHz. Find the noise bandwidth, available output noise power and Noise figure.

b)	Define narrowband noise. Present its quadrature representation clearly.

7.a)	A source emits six messages with probabilities of 0.3, 0.25, 0.15, 0.12, 0.1 and 0.08 respectively. Find the Huffman code, average code length and redundancy.

b)	Define and distinguish between self information and mutual information. How are they corrupted?

8.	Write short notes on the following:

a)	Moments

b)	Stationary and ergodicity

c)	Available noise power.

^^^

Code No.:221101

Set No.

3

II-B.Tech. II Semester Regular Examinations April/May, 2003

PROBABILITY AND RANDOM VARIABLES

(Bio-Medical Engineering)

Time: 3 hours.							      Max. Marks: 80

Answer any FIVE questions

All questions carry equal marks

- - -

1.a)	Present the concept of probability, random variable and density functions. Give an example to illustrate.

b)	The joint pdf of random variable X and Y is given by

f(x, y) = ke-(x+y) 	in the range 0< x, y <

= 0 		otherwise

Determine the constant k, f(x) and

P[0X10, 0Y10].

2.a)	A random variable X has the uniform density in [a, b]. The random variable Y is defined by Y = 5x2+ 10. Determine the density function of Y.

b)	Z = X1 + X2 +…..+XN.

E(Xi) = m

E(Xi Xj) = 1	j = i

= 	j = i  1

= 0	otherwise

Find i)	     E(Z) 	ii)	E(Z2)

3.a)	The random variables X1, X2, X3,…. are independent but uncorrelated. Z=X1+X2+X3+…. Show that Z2= 12+ 22+ 32+….

b)	State and prove central limit theorem.

4.a)	Define and distinguish between the continuous and discrete random processes. When are two random processes X(t) and Y(t) statistically independent.

b)	A random process is defined by Z(t) = X + Yt where X and Y are independent variables which are uniform in [x1, x2] and [y1, y2] respectively. Find the mean and autocorrelation of Z(t).

5.a)	A random process X(t) with uniform density in [-10,10] is passed through an RC low pass network with R = 100K and C = 1F. Determine the input and output autocorrelation functions and compare them.

b)	Define and give the computational details of the following:

i)	Noise bandwidth		ii)	Available noise power

iii)	Noise figure of cascaded systems.

Contd…2

Code No.: 221101				-2-				      Set No.3

6.	The noise voltage en is passed through a LPF with cut off frequency 1100Hz and then through an amplifier of gain A=10. Develop an expression for the total noise output power of the amplifier.

7.a)	Define and quantify the following:

i)	Entropy		ii)	mutual information.

iii)	channel capacity	iv)	code efficiency.

b)	Apply Shannon-famo coding procedure to the increase the bit carrying efficiencuy for transmitting 8 message symbols with respective probabilities of ½ ,1/8, 1/8, 1/16, 1/16, 1/16, 1/32, 1/32.

8.	Write short notes on the following:

a)	conditional density and its properties

b)	cross correlation and covariance

c)	Effective input noise temperature.

^^^

Set No.

4

Code No.:221101

II-B.Tech. II Semester Regular Examinations April/May, 2003

PROBABILITY AND RANDOM VARIABLES

(Bio-Medical Engineering)

Time: 3 hours.							      Max. Marks: 80

Answer any FIVE questions

All questions carry equal marks

- - -

1.a)	Give the properties of conditional density function and explain the determination of the probability of a conditioned event from it.

b)	Prove that the Rayleigh density function satisfies the properties of a probability density function.

2.a)	A random variable Y=b+X, where Y is a Gaussian variable with zero mean and variance X2 and b is a constant. Show that Y is also a Gaussian variable with different mean and same variance.

b)	Consider the following density functions. Determine their mean and variances.

i)	f(x) = 12π e−(x−5)22	for all x

ii)	f(x) = 1a 		-a2  x  a2

3.a)	Prove that the independent random variables X and Y are uncorrelated while the vice versa may not be true.

b)	The random variable X is uniform in [0, 1]. The independent random variable Y is uniform in [0, 2]. Find and plot the density of Z = X+Y.

4.a)	Define the random process. How is it related to random variable? Give the classification of random processes.

b)	Let X(t) and Y(t) be mutually independent WSS real processes having same autocorrelation function R(Z). Define the complex process Z(t) = X(t) +jY(t). Find the autocorrelation function of Z(t). Is Z(t) wide sense stationary?

5.a)	A white noise n(t) with G(f) = η2 is passed through a lowpass RC Filter with 3dB frequency fC. Find the autocorrelation and PSD of the output noise of the filter.

b)	A parallel RLC circuit is centered at 3GHz with a bandwidth of 10MHz. If the resistance R is 10k, calculate R(f) and PSD G(f) of the noisy circuit.

Contd…2

Code No.: 221101				-2-				      Set No.4

6.a)	Define noise temperature and noise figure. Relate them. The available noise power for a receiver is 1N when measured in a noise bandwidth of 10 MHz. Find Te.

b)	Present the concept of narrow band noise in detail. Derive the quadrature components and the canonical form of it.

7.a)	Define the following with respect to Information Theory

i)	Self information		ii)	Redundancy

iii)	Mutual information		iv)	Channel capacity.

b)	State Shannon-Hartley Law. Demonstrate the trade off between bandwidth and SNR.

8.	Write short notes on the following:

a)	Characteristic function

b)	Stationary of random process

c)	Effective input noise temperature.

^^^