Code No.:220103

Set No.

1

II-B.Tech. Regular II Semester Examinations April/May, 2003

STRUCTURAL ENGINEERING-I (RCC) DESIGN AND DRAWING

(Civil Engineering)

Time: 3 hours.							      Max. Marks: 80

NOTE:	Answer any ONE question from part-A

Answer any THREE questions from part-B

Use of IS: 456-2000 and only design charts of sp-16 are allowed.

- - -

PART-A	                            	  (32 Marks)

1.	Design an isolated rectangular footing for column dimension of 230mm  550mm carrying an axial compressive load of 1200 kN. Assume the safe bearing capacity of the soil as 120 kN/m2. Use M25 grade of concrete and Fe 415 grade of steel. Use limit state method.

Draw the following to a suitable scale.

a)	Longitudinal section of the beam showing reinforcement details.

b)	The cross section of the beam at mid span showing the reinforcement details.

2.	A hall of internal dimensions 5m  15m has beams spaced at 3 m cc and a slab of 120mm thick. The beams are supported by walls around 300mm thick. Design the T-beam completely including shear reinforcement. Draw following to a suitable scale.

a)	The longitudinal section of the beam.

b)	The cross section at center and also at support showing details of reinforcement.

PART-B			     (3×16=48 Marks)

3.	What are the assumptions made in the working stress method of design of members in flexure? Determine the moment of resistance of a 230mm  550mm rectangular beam reinforced with 4-25mm diameter HYSD bars of Fe415 grade in tension zone. Grade of concrete is M20. Adopt working stress method of design. Assume a clear cover of 25mm.

4.	Design a rectangular beam at mid span having clear span of 5m, simply supported. It is subjected to a dead load of 20kN/m and live load of 25kN/m. Use M25 grade concrete and Fe 415 grade steel. Adopt limit state method.

5.	A simply supported rectangular beam of 8m span is carrying a live load of 40kN/m. The section of the beam is 230  460mm effective depth. It is reinforced with 6no’s 25mm diameter bars. Design the beam for shear using

(a)  Vertical stirrups and

(b)  2no’s 25mm diameter bars are bent up at 120mm from the face of the support.

Use M20 grade concrete and Fe415 grade steel. Use limit state method.

Contd…2

Code No.: 220103				-2-				       Set No.1

6.	Design a column with the size of 400mm  500mm, Pu=2000kN, Mux=100kNm and MuY=60kNm. Adopt M30 grade concrete and Fe415 grade steel. Use limit state method.

7.	A concrete beam of 230mm  600mm size with 4no’s 20mm diameter bars in tension at an effective cover of 50mm is simply supported over an effective span of 4m. The bending moment at mid span is 120kNm, out of which 50% is due to permanent loads. Check the beam for short term deflection and deflection due to creep only. Use M20 grade concrete, Fe 415 grade steel and limit state method.

^^^

Code No.:220103

Set No.

2

II-B.Tech. Regular II Semester Examinations April/May, 2003

STRUCTURAL ENGINEERING-I (RCC) DESIGN AND DRAWING

(Civil Engineering)

Time: 3 hours.							      Max. Marks: 80

NOTE:	Answer any ONE question from part-A

Answer any THREE questions from part-B

Use of IS: 456-2000 and only design charts of sp-16 are allowed.

- - -

PART-A				(32 Marks)

1.	Design an end L-beam for an office building with slab thickness of 150mm monolithic with 230mm wide beams. The spacing of the beams are at 3.65m centre to centre. The effective span of the beam is 10m. Also design the beam for shear. Use M25 grade concrete and Fe 415 grade steel. Use limit state method. Draw the following to a suitable scale:

(a)   Longitudinal section of the beam showing reinforcement details.

(b)   The cross section of the beam at mid span showing the reinforcement details.

2.	Design an isolated square footing for a column size of 300mm  300mm carrying an axial load of 700kN. Safe bearing capacity of the soil is 100kN/m2. Use M20 grade concrete and Fe 415 grade steel. Use limit state method.

Draw to scale:

- Plan of the footing showing the reinforcement details.

- Sectional elevation of the footing.

PART-B			     (3×16=48 Marks)

3.	Design a singly reinforced beam of rectangular cross section to carry a superimposed load of 30kN/m over a simply supported span of 5m. Use M20 grade of concrete and Fe415 grade of steel. Use working stress method.

4.	What are the assumptions made in the limit state design method in flexure? Design a rectangular beam for an effective span of 6m, which carries a superimposed load of 100kN/m. The overall size of the beam is restricted to 350mm650mm. Adopt M20 grade concrete and Fe415 grade steel. Use limit state method.

5.	A beam 300   1000mm effective has a span of 7m. Total load on the beam is 50kN/m. Tensile steel is 6 no’s-25mm diameter bars. If M20 grade concrete and Fe 415 grade steel are used, Design shear reinforcement. Use limit state method.

Contd…2

Code No.:220103				-2-				      Set No.2

6.	A rectangular column of an effective height of 4m is subjected to an axial load of 800kN and bending moment of 100kNm about the major axis of the column. Design a suitable section for the column so that the width should not exceed 400mm. Assume M25 grade concrete and Fe 415 grade steel. Use limit state method and design charts of sp-16.

- Design a reinforced concrete slab for a room of 4m  5m. The slab is simply supported on all four sides of 300mm thick walls and its corners are free to lift. It has to carry 3kN/m2 live load and 25mm thick floor finishes of 24kN/m3. Use M20 grade concrete and Fe415 grade steel. Adopt limit state method.

^^^

Code No.:220103

Set No.

3

II-B.Tech. Regular II Semester Examinations April/May, 2003

STRUCTURAL ENGINEERING-I (RCC) DESIGN AND DRAWING

(Civil Engineering)

Time: 3 hours.							      Max. Marks: 80

NOTE:	Answer any ONE question from part-A.

Answer any THREE questions from part-B.

Use of IS: 456-2000 and only design charts of sp-16 are allowed.

- - -

PART-A				(32 Marks)

1.	Design an intermediate T-beam for an office building with 150mm thick slab monolithic with 230mm wide beams 	spaced at 3.65m center to center. The effective span of the T-beam is 10m. Use M25 grade concrete and Fe 415 grade steel. Also design the beam for shear using bent up bars. Use limit state method.

Draw to a suitable scale:

- Longitudinal section of the beam showing the reinforcement details.

- The cross-section of the beam at mid span showing the reinforcement details.

2.	Design an inverted T-beam roof for a hall of 6m wide and 15m to support a dead load of 3kN/sq.m and a live load of 4kN/sq.m. Use M20 grade concrete and Fe 415 grade steel. Use limit state method. Draw to a scale:

(a) Longitudinal section showing reinforcement details.

(b) The cross section of the beam showing reinforcement details.

PART-B			     (3×16=48 Marks)

3.	A rectangular R.C beam of size 350mm wide and 450mm effective depth is reinforced with 4no’s 12mm diameter bars. Find the safe concentrated load at center on an effective span of 4m, which the beam can resist in addition to its self weight. The materials are M25 grade concrete and Fe 415 grade steel. Use working stress method.

4.	An L-beam has the flange width of 900mm, with the thickness of slab 100mm. The web below the slab is 250mm  500mm. Determine the reinforcement required to carry a limiting moment of 500kNm. Assume M20 grade concrete and Fe 415 grade steel. Use limit state method.

- Design a rectangular R.C. beam to take up factored moment of120kNm, factored torque of 70 kNm and factored shear of 60kN. Use M20 grade concrete and

Fe 415 grade steel for main reinforcement and Fe250 grade steel for stirrup reinforcement.

Contd…2

Code No.: 220103				-2-				      Set No. 3

6.	Design a circular short column to carry an axial load of 1800kN. Using

(a)	Lateral ties and

(b)	Helical reinforcement.

Use M30 grade concrete and Fe 415 grade steel. Use limit state method.

7.	A simply supported one-way slab of effective span 4m is supported on masonary walls of 230mm thickness. Design the slab. Take live load equal to 4KN/m2 and floor finish equal to 1KN/m2. The materials are M20 grade concrete and HYSD reinforcement of grade Fe 415. Use limit state method.

^^^

Code No.:220103

II-B.Tech. Regular II Semester Examinations April/May, 2003

Set No.

4

STRUCTURAL ENGINEERING-I (RCC) DESIGN AND DRAWING

(Civil Engineering)

Time: 3 hours.							      Max. Marks: 80

NOTE:	Answer any ONE question from part-A.

Answer any THREE questions from part-B.

Use of IS: 456-2000 and only design charts of sp-16 are allowed.

- - -

PART-A				(32 Marks)

1.	Design a simply supported rectangular beam to carry 30kN/m superimposed load over a span of 6m on 460mm wide supports. Use M20 grade concrete and Fe 415 grade steel. Check the design for all necessary conditions. Use limit state method.

Draw to a suitable scale:

- Longitudinal section showing reinforcement details.

- The cross section of the beam at salient points, showing reinforcement details.

2.	A circular column 450mm diameter transfers an axial load of 1000kN. The column is having 8no’s 12mm diameter bars. The safe bearing capacity of the soil is 120kN/m2. Use M20 grade concrete and Fe 415 grade steel. Design a circular footing to support the circular column. Use limit state method.

Draw to a suitable scale:

- Plan showing the reinforcement details.

- Cross sectional evaluation of the footing with full details.

PART-B			     (3×16=48 Marks)

3.	A rectangular beam 230mm wide  460mm effective depth is subjected to a moment of 45kNm. The effective cover of compressive reinforcement is 40mm. Find out the reinforcing steel. The materials are M20 grade concrete and HYSD reinforcement of grade Fe 415. Use working stress method.

- Determine the area of steel in a T-beam with the following dimensions for a moment of resistance of 460kNm. Depth of slab=120mm, breadth of flange 900mm, breadth of web 300mm, total depth 530mm. Assume M25 grade concrete and Fe 415 grade steel. Use limit state method.

- A doubly reinforced rectangular beam of size 230mm  420mm effective depth requires 820mm2 area as tension reinforcement and 220mm2 area as compressive steel. It is reinforced with 2 no’s 12mm diameter bars in compression at an effective cover of 40mm and 3-20mm diameter bars in tension. Calculate the development length of the bars. The materials are M20 grade concrete and mild steel reinforcement. Use limit state method.

Contd…2

Code No.: 220103				-2-				    Set No.4

- An R.C.C. column with effective height of 6m and size 230mm  450mm is subjected to an axial load of 800 kN. Design the column section. The materials are M25 grade concrete and Fe415 grade steel. Use limit state method.

- Design a continuous one-way slab having three equal spans of 3m each for the imposed load of 4kN/m2, concrete grade M20 and steel of grade Fe415. Use limit state method.

^^^