Code No.221002

Set No.

1

II-B.Tech. II-Semester Examination April/May 2003

TRANSDUCERS IN INSTRUMENTATION

(Common to Electronics and Instrumentation Engineering and Electronics and Control Engineering)

Time:3 hours								Max.Marks: 80

Answer any FIVE questions

All questions carry equal marks

---

1.a)	Define the terms

(i) 	Linearity	(ii)	Hysterisis

(iii) 	Threshold  	(iv) 	Resolution.

b)	A 10 KΩ variable resistance has a linearity of 0.1% and the movement of contact arm is 320o (i) determine the maximum position deviation in degrees and the resistance deviation in ohm. If the instrument is to be used as a potentiometer with a linear scale of 0 to 1.6 volt, determine the maximum voltage error.

2.a)	Define and discuss the dynamic characteristics of a measurement system.

b)	Derive the equations for time response of a first order system when subjected to unit ramp input. Draw the response curves and find the steady error.

3.a)	What are the different types of noise present in resistive potentiometer? Explain how are they overcome.

b)	Derive an expression for gauge factor?

4.a)	Explain the construction and principle of Variable inductance transducer

b)	The output of a LVDT is connected to a 5V voltmeter through an amplifier whose amplification factor is 250. An output of 2mV appears across the terminals of a LVDT when the core moves to a distance of O.5mm. Calculate the sensitivity of the LVDT and that of the whole setup. The mill voltmeter scale has 100 divisions. The scale can be read to 1/5 of a division. Calculate the resolution of the instrument in mm.

5.a)	Explain how by using a differential arrangement a capacitive transducer which works on the principle of variation of capacitance with displacement between two plates, the response can be made linear.

- Describe the frequency response of capacitive transducers.

6.a)	Derive the expressions for frequency response characteristics of piezo  electric

transducers.

b)	Describe the uses of piezo electric materials and transducers.

Contd…2

Code No.221002				-2-                                        	Set No:1

7.a)	With schematic diagram, explain about Electrostatic Force-balance transducer for the measurement of gas pressures.

b)	Starting from fundamentals, derive an expression for sensitivity of

Electrostatic Force-balance pressure transducer.

8.	Compare a total radiation pyrometer with an optical pyrometer in terms  
of operation and construction details.

$$$

Code No.221002

Set No.

2

II-B.Tech. II-Semester Examination April/May 2003

TRANSDUCERS IN INSTRUMENTATION

(Common to Electronics and Instrumentation Engineering and Electronics and Control Engineering)

Time:3 hours								Max.Marks: 80

Answer any FIVE questions

All questions carry equal marks

---

1.a)	Classify various transducers and give an example of each and mention their applications.

b)	What is the true value of voltage across the 500 KΩ resistor connected between terminals A and B as shown in fig. below ?  What would a voltmeter with a sensitivity of 20 KΩ/v read on the following ranges: 50, 15,5 volts when connected across terminals C and D.

2.a)	Classify various errors and discuss them.

b)	The true value of voltage across a resistor is 50V. The measurement find a value of 49V. Calculate (i) the absolute error (ii) the percent error (iii) the percent accuracy.

3.a)	Explain the working principle of potentiometer. Derive an expression for its loading error.

b)	Discuss the application of resistive strain gauge

4.a)	Give an overview of the inductive transducer explaining their principle of operation like variation of number of turns, geometric configuration and permeability.

b)	A steel cantilever is 0.25m long, 20 mm wide and 4mm thick.

i)	Calculate the value of deflection at free end for the cantilever when a force of 25 N is applied at this end. The modulus of elasticity for steel is 200 GN/m.2

ii) 	An L VDT with a sensitivity of 0.5 V /mm is used to sense the deflection. The tenths of a division can be read with certainty. Calculate the minimum and maximum value of force that can be measured with this arrangement.

Contd…2

Code No.221002				-2-                                        	Set No:2

5.a)	Mention the advantages and disadvantages of capacitive transducers.

b)	Give the applications of capacitive transducers.

6.a)	Draw the equivalent  circuit of piezo-electric transducers.

b)	Derive the expression for magnitude of voltage across the load by making simplifying assumptions.

c)	Prove that medium and high frequencies the magnitude of the voltage across

the load is independent of frequency.

7.a)	With schematic diagram, explain about Electrodynamic Force-balance transducer for the measurement of linear acceleration.

b)	Explain the static performance of an Electrodynamic Force-balance

acceleration transducer with respect to sensitivity and linearity.

8.	Sketch the thermoelectric characteristics of commonly used industrial 
thermocouples. What is meant by cold junction compensation of a 
thermocouple? How is it carried out in practice?  Explain.

$$$

Code No.221002

Set No.

3

II-B.Tech. II-Semester Examination April/May 2003

TRANSDUCERS IN INSTRUMENTATION

(Common to Electronics and Instrumentation Engineering and Electronics and Control Engineering)

Time:3 hours								Max.Marks: 80

Answer any FIVE questions

All questions carry equal marks

---

1.a)	Explain an instrumentation system with a block diagram.

b)	The dead-zone of a certain plyrometer is 0.125 percent of the span. The calibration is 800oC to 1800oC what temperature change must occur before it is detected?

2.a)	Describe mathematical model of measurement systems.

b)	A 0-150V voltmeter has a guaranteed accuracy of 1% of full scale reading. The voltage measured by this instrument is 75V. Calculate the limiting error in percent. Comment upon the result.

3.a)	Name the different types of resistive strain gauges. Explain the principle of operation with suitable examples.

b)	How is temperature effect compensated in strain gauges? Explain.

4.a)	Describe in brief, a piezo resistive type strain gauge, mentioning its merits and demerits.

b)	Name  two materials, which are used to construct piezo electric transducers.

5. 	Draw the schematic diagram of a parallel plate capacitive transducer and explain

how it is useful for measuring linear displacement. Describe the same using

different methods.

6.a)	Describe the different modes of operation of Piezo electric transducers.

b)	Define and sketch binders and twisters.

c)	Explain the application of Piezo electric transducers.

7.a)	Explain about  ‘The Force-balance principle’, related to force-balance transducers.

b)	List out the advantages and the disadvantages of Force-balance trans-ducers and briefly explain about them.

8.	Explain the principle of working, constructional details and applications of

photodiodes.

$$$

Code No.221002

Set No.

4

II-B.Tech. II-Semester Examination April/May 2003

TRANSDUCERS IN INSTRUMENTATION

(Common to Electronics and Instrumentation Engineering and Electronics and Control Engineering)

Time:3 hours								Max.Marks: 80

Answer any FIVE questions

All questions carry equal marks

---

1.a)	Explain the phenomenon of hysteresis in measurement systems. Also explain the terms,

(i)	Static error,

(ii)	Relative error

(iii)	Static correlation.

b)	A mercury thermometer has a capillary tube of 0.3mm diameter. If the bulb is made of zero expansion material, what volume must it have if a sensitivity of 3mm/oC is desired. Assume operating temperature to be 20oC and coefficient of volumetric expansion of mercury is 0.181 x 10-3.

2.a)	Define the terms:

(i)	 Precision	(ii)	Dead zone

(iii) 	Hysteresis  	(iv) 	Resolution.

b)	An RC circuit consists of 10F in series with a resistor of 5K. A D.C voltage of 25 volts is suddenly applied across the circuit. Calculate the value of voltage after

(i) 	4 msec. 	(ii) 	25 msec.

3.a)	List the different types of strain gauges. Explain the construction and materials used for foil type strain gauges.

b)	Discuss the advantages, disadvantages and application of bonded semiconductor gauges.

4.a)	Explain why inductive transducers are mostly iron cored. Suggest the

important considerations to be applied while designing and using them.

b)	Describe the push pull type self-inductive transducer and discuss the merits

of this over single coil transducer.

5.a)	How the angular displacement is measured using capacitive transducer.

b)	How the rotational displacement is measured by using capacitive

transducer.

6.a)	Describe the properties of materials used for Piezo-electric transducers.

b)	Derive the expressions for voltage and charge sensitivities.

Contd…2

Code No.221002				-2-                               		Set No:4

7.a)	Starting from fundamentals, derive the expression for linearized sensitivity of Electrostatic Force-balance transducer.

b)	List out the performance characteristics of typical electrostatic Force-balance

transducers.

8.	Explain the principle of working and applications of optical pyrometers.

$$$