Code No:221052

OR

II-B.Tech. II-Semester Examinations, April/May, 2003

SIGNALS AND SYSTEMS

(common to Electronics and Instrumentation Engineering and Electronics and Control Engineering)

Time: 3 hours							        Max. Marks: 70

Answer any FIVE questions

All questions carry equal marks

- - -

1.a)	State Dirichlet’s conditions for expanding periodic functions in Fourier series.

b)	Find the exponential Fourier series of a periodic function shown below.

c)	What is half-wave symmetry? What does it tell of the harmonies?

2.a)	Find Fourier transform of a Gaussion signal  f(t)=e−at2.

b)	Find the inverse Fourier transform of the function sinWW cos W.

3.a)	A certain LTI system has an impulse response h(t) = e-2t ut. Find the output for an input of 0.5t for all t.

b)	By proving Parseval’s theorem show that Fourier transform is an isometry.

4.a)	Write three properties of auto correlation function.

b)	Let r(t) represent a ramp function. Convolve the two signals e-2tu(t) and

[2r (t) – 4r (t-1)+r(t-2)].

5.a)	Find the Laplace transform of 12tu(t).

b)	Let f(t) return zero for t<0 and the fractional part for t>0. Find the Laplace transform of f(t).

c)	Find inverse Laplace transform of log10S+aS+b.

(Contd…2)

Code No:221052				-2-			OR

6.a)	Find the Z-transform of Fibonacci sequence.

b)	Show that the family of complex exponential sequences

expj2πNkn;n=0,1,......N−1,K=0,1,......N−1 constitutes a closed set.

c)	Obtain inverse z-transform of exp1Z.

7.a)	Discuss the philosophy of Filter design based image impedance.

b)	Draw the variation of Z.  of the following constant k filter in pass and stop beams.

c)	How do you select m value for terminating half sections?

8.	Write short notes on any Three of the following:

- Singularity functions.

- Pole-zero plot and its significance on the transient response.

- Distortionless transmission.

- Lattice equalizers.

- All-pass filters.

***     ***     ***