Code No.220304

Set No.

1

II-B.Tech. II-Semester Examination April/May 2003

KINEMATICS OF MACHINERY

(Common to Mechanical Engineering, Production Engineering, Mechatronics and Aeronautical Engineering)

Time: 3 hours							Max.Marks:80

Answer any FIVE questions

All questions carry equal marks

---

1.a) 	Distinguish clearly between a 'structure' and a 'Machine'.

b) 	Explain various inversions of single and double slider crank chains.

2.a) 	Show that Peaucellier mechanism generates an exact straight line as its path.

b) 	What is the difference between lower and higher pairs? Give examples in each 	case.

3.	Explain how corili’s component of acceleration arises. Derive an expression for determining its magnitude. How do you find its direction for any given direction of crank rotation?

4.a)	Define the term instantaneous centre of rotation.

b)	Sketch a quick return motion of the crank and slotted lever type and explain the procedure of drawing the velocity and acceleration diagram, for any given configuration of the mechanism.

5.a.)	Describe the working of Davis steering gear mechanism with neat sketch.  Derive the condition for correct steering of the above mechanism.

b)	The Distance between the steering pivots of a Davis Steering gear is 1.3m. The wheel base is 2.75 metres. What will be the inclination of the track arms to the longitudinal axis of the vehicle if it is moving in a straight path?

6.	A cam rotating at 150 R.P.M. operates a reciprocating roller follower of radius 2.5 cm. The follower axis is offset by 2.5 cm to the right. The least radius of the cam is 5 cm and the stroke of the follower is 5 cm. Ascent and descent both take place by uniform acceleration and retardation. Ascent takes place during 75 and descent during 90 of cam rotation. Dwell between ascent and descent is 60. Draw cam profile and mark salient values.

(Contd…2)

Code No.220304				-2-				Set No.1

7.a)	Define the following terms: (i) Pitch circle diameter (ii) Circular pitch

(iii) Diametral pitch (iv) Module (v) Pressure angle (vi) Pressure line

b) 	A pair of gears have 16 and 22 teeth, module pitch is 12.5 mm, addendum 12.5 	         mm, dedendum 14.25 mm and pressure 200.   The pinion drives in anti-clock

wise direction.   Determine (i) Pitch circle radii (ii) Base circle radii (iii) the 	circular pitch (iv) Angle of approach and recess

8.  	In an epicyclic gear train, as shown in the figure below the wheel C is keyed to 	the Shaft B and wheel F is keyed to shaft A.  The wheels D and E rotate 	together on a pin fixed to the arm G.  The number of teeth on wheels C, D, E 	and F are 35,65,32 and 68 respectively. If the shaft A rotates at 60 r.p.m. and the 	shaft B rotates at 28 r.p.m. in the opposite direction, find the speed and direction 	of rotation of arm G.

***	***	***

Code No.220304

Set No.

2

II-B.Tech. II-Semester Examination April/May 2003

KINEMATICS OF MACHINERY

(Common to Mechanical Engineering, Production Engineering, Mechatronics and Aeronautical Engineering)

Time: 3 hours							Max.Marks:80

Answer any FIVE questions

All questions carry equal marks

---

1.a) 	Show that the locus of the mid-point of the link connecting to the two slides in an elliptical trammel is a circle.

b) 	Define frame of a machine? Which link of the machine is known as frame?

2.a)	Explain Hart’s mechanism in detail. What are its applications?

b)	Explain the working of Scott – Russel mechanism and modified

Scott – Russel mechanism.

3.		In a Whit worth quick return motion, the crank AB rotates about the fixed centre 	A.  The end B operates a slider reciprocating in a slotted link, rotating about a 	fixed centre D, 5 cm vertically above A.  The crank AB which is 10 cm long, 	rotates in a clockwise direction at a speed of 100 r.p.m.  Find the angular 	acceleration of a slotted link for the configuration in which AB has turned through 	an angle of 450 past its lowest position

4.a)	Explain, with the help of a neat sketch, the space centrode and body centrode

b)	In a pin jointed four bar mechanism ABCD, the lengths of various links are as follows: AB = 25 mm; BC = 87.5 mm; CD = 50 mm and AD = 80 mm.  The link AD is fixed and the angle BAD = 1350.   If the velocity of B is 1.8 m/s in the clockwise direction, find (i) velocity and acceleration of the mid point of BC, and  (ii) angular velocity and angular acceleration of the link CB and CD

5.a)	What is the function of a steeing gear.  What are the mechanisms used in general.  Explain any one of them.

b)	In a Davis Steering gear the distance between the pivot's of the front axle is 1 metre and the wheel base is 2.5 metres find the inclination of the track arm to the longitudinal axis of the car. When it is moving along a straight path.

6.a)	Explain the procedure for drawing the displacement, velocity and acceleration diagrams for a radial cam with simple Harmonic motion of the follower.

b)	Give the expression for maximum velocity and acceleration of follower during ascent and descent with respect to the above motion

(Contd…2)

Code No.220304				-2-				     Set No.2

7.a) 	State and prove the law of gearing for constant velocity ratio and show how the involute profile satisfies the condition?

b) 	A pair of spur wheels with involute teeth is to give a gear ratio 3:1.   The arc of approach is not being less than the circular pitch and the smaller wheel is the driver.   Pressure angle is 20 0.   What is the least number of teeth that can be used on each wheel and find the addendum of the wheel in terms of the circular 	pitch?

8.  	An epicyclic gear train, as shown in figure below is composed of a fixed annular 	wheel A having 150 teeth.  The wheel A is meshing with wheel B which drives 	wheel D through an idle wheel C,D being concentric with A.  The wheels B and C 	are carried on an arm which revolves clockwise at 100 r.p.m. about the axis 	of A and D.  If the wheels B and D have 25 teeth and 40 teeth respectively, 	find the number of teeth on C and the speed and sense of rotation of C.

***	***	***

Code No.220304

Set No.

3

II-B.Tech. II-Semester Examination April/May 2003

KINEMATICS OF MACHINERY

(Common to Mechanical Engineering, Production Engineering, Mechatronics and Aeronautical Engineering)

Time: 3 hours							Max.Marks:80

Answer any FIVE questions

All questions carry equal marks

---

1.a) 	State the difference between the closed and unclosed pairs giving examples in each case.

b) 	What is a Kinematic chain? How is it formed from the links?

2.a)	What are  various approximate straight line motion mechanisms?

b)	Explain Watt’s Straight line mechanism. What are the practical uses of this 	mechanism?

3.a)	Explain, with the help of a neat sketch, the space centrode and body centrode

b)	In a pin jointed four bar mechanism ABCD, the lengths of various links are as follows: AB = 25 mm; BC = 87.5 mm; CD = 50 mm and AD = 80 mm.  The link AD is fixed and the angle BAD = 1350.   If the velocity of B is 1.8 m/s in the clockwise direction, find (i) velocity and acceleration of the mid point of BC, and  (ii) angular velocity and angular acceleration of the link CB and CD.

4.a)	Discuss the three types of instantaneous centers for a mechanism.

b)	The stroke of a steam engine is 15 cm and the connecting rod is 30 cm in length.  Determine the velocity and acceleration of the piston when the crank has made 450 measured from the inner dead centre and rotates at 600 r.p.m.  Also calculate the velocity and acceleration of point p, 5 cm on the connecting rod produced from the junction of the crank.

5.a)	Derive an expression for the ratio of the shaft velocities for Hooke's joint and draw 	the polar diagram depicting the salient features of driven shaft speed.

b)	Two shafts with an included angle of 160 are connected by a Hooke's Joint. The driving shaft runs at a uniform speed of 1500 R.P.M. The driven shaft carries a fly wheel of mass 12kg and 100 m.m radius of gyration. Find the maximum angular acceleration of the driven shaft and the maximum torque required.

(Contd…2)

Code No.220304				-2-				         Set No.3

6.	A cam is to be designed for a knife edged follower with the following data.

1.	Cam lift = 40mm during 90 of cam rotation with S.H.M

2.	Dwell for the next 30

3.	During the next 60 of cam rotation the follower returns to its original position with simple harmonic motion.

4.	Dwell during the remaining 180

Draw the profile of the cam when the line of stroke is offset by 20 mm from the axis of the cam shaft.

The radius of the base circle of the cam is 40mm. Determine the maximum velocity 	and acceleration of the follower during its ascent and descent if the cam rotates at 	240 R.P.M.

7.a) 	Derive an expression for the velocity of sliding between a pair of involute teeth.

b) 	A pinion with 20 teeth is in mesh with a gear of 60 teeth.   The pressure angle is 	200 and addendum is 7 mm.   Determine (i) the number of pairs of teeth in 	contact 	and the angle turned by the pinion and the gear wheel when one pair of 	teeth is 	in contact. (ii) Sliding velocity at the beginning of engagement, at the pitch 	point and at the termination of the engagement.

8.    	The figure shows an epicyclic gear train with the following details:

A has 40 teeth external (fixed gear); B has 80 teeth interna; C-D is a compound 	wheel having 20 	and 50 teeth (external) respectively, E-F is a compound 	wheel having 20 and 40 teeth (external) 	respectively, and G has 90 teeth 	(external). The arm runs at 100 r.p.m. in clockwise direction.  	Determine the 	speeds for gears C,E, and B.

***	***	***

Code No.220304

Set No.

4

II-B.Tech. II-Semester Examination April/May 2003

KINEMATICS OF MACHINERY

(Common to Mechanical Engineering, Production Engineering, Mechatronics and Aeronautical Engineering)

Time: 3 hours							Max.Marks:80

Answer any FIVE questions

All questions carry equal marks

---

1.a) 	Name different types of mechanism.  Give examples of each of them.

b) 	What is a Kinematic pair?  State different methods of classifying them and state 	the salient features of each method of classification.

2.	Explain the following mechanisms:

- Grasshopper mechanism

- Tchebicheff’s mechanism

- Robert’s mechanism

3.a)	For the position shown in figure determine angular velocity of the ladder AB m long and velocity of its upper point B along with the vertical wall when the lower end is constrained to move on the horizontal floor with a linear velocity of 10cm/sec.

b)	At the position shown if linear acceleration of end A is 5cm/sec2. What would be linear acceleration of the end B and resultant acceleration of the ladder AB?

60O



B

A

Va

Ladder

Vb

4.	Explain with sketches how the instantaneous centers of the following mechanisms are obtained.

(a) Four bar chain	(b) simple slider crank chains.

Contd….2

Code No:220304				-2-				Set No:4

5.a)	Explain why two Hooke's Joints are used to transmit motion from the engine to the differential of an automobile.

b)	A double universal joint is used to connect two shafts in the same plane. The intermediate shaft is inclined at an angle of 20 to the driving shaft as well as the driven shaft. Find the maximum and minimum speed of the intermediate shaft and the driven shaft if the driving shaft has a constant speed of 500 R.P.M.

6.a.)	Explain a radial cam mechanism and describe the types of cams with sketches.

b)	Sketch the different types of cam followers

c)	Explain angle of Ascent, dwell and angle of descent.

d)	Briefly explain the different types of follower motions.

7.a) 	Name the curves, which satisfy the condition for, correct gearing and compare them giving atleast six points.

b) 	A gear wheel having 20 teeth of involute form of module pitch 6 mm with an 	angle of obliquity of 200, drives another wheel of the same dimensions.   	Calculate (i) the length of the arc of contact if the addendum is one module. (ii) If 	the addendum was altered so that the arc of contact was the maximum possible 	what would be the length of this arc, and the addendum required for this 	condition.

8.    	An epicyclic gear train, as shown in the figure below has a sun wheel S of 30 	teeth and two 	planet wheel P.P. of 50 teeth.  The planet wheels mesh with the 	internal teeth of a fixed annual wheel A. the driving shaft carrying the sunwheel, 	transmits 4 kW at 300 r.p.m. the driven shaft is connected to an arm which 	carried the planet wheels.  Determine the speed of the driven shaft and the torque 	transmitted, if 	the overall efficiency is 95%.

***	***	***