Code No.: 220853

OR

II-B.Tech. II-Semester Examinations April/May 2003

CHEMICAL ENGINEERING THERMODYNAMICS

(Chemical Engineering)

Time: 3 hours							Max. Marks: 70

Answer any FIVE questions

All questions carry equal marks

- - -

1.	Prove that work done in a steady flow process is given by integral 2 to 1 v dp 	when volume changes from v1 to v2.

2.a)	State the first law of thermodynamics.

b)	One kilogram of air is heated reversibly at constant pressure from an initial state 	of 300 K and 1 bar until its volume triples. Calculate W, Q, ∆U, ∆H for the 	process. Assume that air obeys the relation PV/T = 83.14 bar cm3/mol K and that 	Cp=29J/mol K.

3.a)	What is equation of state? Discuss the ideal gas equation of state.		      	            b)	A room has a dimensions of 3m  4m  5m. What is the mass of air in this room 	if the air pressure and temperature are 1 bar and 300K respectively?

c)	An ideal gas undergoes a reversible adiabatic process. Derive an expression for 	the changes in internal energy and enthalpy in terms of the pressure ratio.

4.a)	Define reduced pressure and reduced temperature.

b)	Two rigid tanks are connected by a valve. Tank I has a volume of 1 m3 contains 	air at 0.55 MPa and 27C. Tank II contains 6 kg of air at 0.25 MPa and 37C. The 	valve is now opened and the system is allowed to reach thermal equilibrium with 	the surroundings which are at a temperature of 25C. Find the volume of the 	second tank and the final equilibrium pressure of air.

5.	Write briefly about the limitations and real process of second law of 	thermodynamics.

6.	An inventor claims to have devised a process using only saturated steam at 100C 	which will, by a complicated series of steps, make heat continuously available at a 	temperature of 175C. He claims further that, for every pound of steam taken into 	the process, 200 Kcal of energy as heat is liberated at this higher temperature of 	175C. Show whether or not such a process is possible. In order to give the 	inventor the benefit of any doubt, assume cooling water to be available in 	unlimited quantity at a temperature of 0C.

Contd…2

Code No.:220853				-2-				      OR

7.	If the heat capacity of a substance is correctly represented by an equation of 	the form, 	CP = A + BT + CT2

Show that the error resulting when < CP>H is assumed equal to CP evaluated at 	the arithmetic mean of the initial and final temperature is C(T2 – T1)2 / 12.

8.	The standard heat of combustion of graphite at 298(K) is (-393.778)kJ/Mol. 	Determine the heat of combustion at 800K. The heat of capacities are (J/mol.K):

Carbon: 11.19 + 1.096 * 10-2 T – 4.894 * 105/T2

Oxygen: 34.62 + 1.08 * 10-3T – 7.859 * 105/T2

CO2 : 43.29 + 1.147 * 10-2T – 8.185*105/T2

Explain process steps and the thermodynamic principle involved.

^^^