Code No.220106

Set No.

1

II-B.Tech. II-Semester Examination April/May 2003

SURVEYING-II

(Civil Engineering)

Time: 3 hours							Max.Marks:80

Answer any FIVE questions

All questions carry equal marks

---

1.a)	Discuss different methods of balancing the traverse.

b)	Describe how to measure a horizontal angle using theodolite. Also explain briefly 	two methods of minimizing the error in such calculations.

2.a)	Explain about station marks-signals and towers.

b)	Find the elevation of top of a chimney from the following data.

| Instrument station | Reading on BM | Angle of Elevation | Remarks |
| --- | --- | --- | --- |
| A B | 0.862 1.222 | 18o36' 10o12' | RL of BM = 421.380m Distance AB = 50m |

Stations ‘A’ and ‘B’ and the top of chimney are in the same vertical plane.

3.	Following observations were recorded with a tacheometer fitted with an 	anallactic lens (K =100; C =0).

| Instrument Station | HI | Staff Station | Vertical Angle | Staff readings |
| --- | --- | --- | --- | --- |
| O O T | 1.500 1.500 1.350 | BM CP CP | -4o30' +6o12' -7o45' | 1.250, 1.440, 1.550 1.550, 1.750, 1.950 1.390, 1.550, 1.710 |

Calculate RL of ‘CP’ and station ‘T’. The staff was held vertical during 	observations and R.L. of BM = 500.0m.

4.a)	Explain about calculation of minimum sight distance on horizontal curves?

b)	Two tangents of a circular curve of radius 300m have a deflection angle of 90o. It 	is proposed to change the position of forward tangent by rotating it through 20o, 	thus making the deflection angle = 110o. Calculate radius of new curve if P.C. is 	unchanged.

5.a)	Differentiate systematic errors and accidental errors.

b)	The following are the three angles ,  and   observed at a station ‘P’ closing the 	horizon, along with their probable error of measurement. Determine their 	corrected values.

 = 78o12' 12'' 2''

 = 136o48'30o4''

 =  144o59'08''5''

(Contd…2)

Code No.220106				-2-				Set No.1

6.a)	Explain the procedure for setting compound curves.

b)	Two straights on the centre line of a proposed railway curve intersect at 2610m, 	the deflection angle being 46o. A circular curve with 400m radius and transitions 	of 90m length are to be inserted. Calculate necessary data to set out first transition 	and circular curve, by tangential angles.

7.a)	Discuss about triangulation adjustments.

b)	Explain axis signal correction.

8.	Write short notes on:

- Well conditioned triangle.

- Gale’s traverse table.

- Theory of least squares.

***     ***     ***

Code No.220106

Set No.

2

II-B.Tech. II-Semester Examination April/May 2003

SURVEYING-II

(Civil Engineering)

Time: 3 hours							Max.Marks:80

Answer any FIVE questions

All questions carry equal marks

---

1.a)	Draw the schematic diagram of a theodolite and explain the main parts and their 	functions.

b)	Describe temporary adjustments of theodolite.

2.a)	Explain classification of triangulation systems.

b)	Directions are observed from a satellite station ‘S’, 62.18m from station ‘C’. 	Following were the results

A = 0o0'06'';	B = 71o54'32'';       C = 296o12'02''

The approximate lengths of AC and BC were 8240.60m and 10863.60m. 	Calculate ACB.

3.a)	Stadia readings were taken with a theodolite on a vertical staff with a theodolite 	inclined at an angle of 3o30'. The staff readings were 2.990, 2.055 and 1.120. The 	R.L. of staff station is 100.000 and height of instrument is 1.40m. What is R.L. of 	ground at the instrument.

b)	Two distances of 50 and 80m were accurately measured out and the intercepts on 	the staff between the outer stadia webs were 0.496 at the former distance and 	0.796 at the latter. Calculate the tacheometric constants.

4.a)	A reverse curve having two branches of common radius is to connect two parallel 	straights 9m apart. If the distance between the tangent points is 72m, find 	necessary radius.

b)	Derive formula for sight distance on horizontal curves.

5.a)	Given that the intersection angle of a two-degree curve is 120o, compute the 	various elements of a simple curve.

b)	Discuss the miscellaneous problems in theodolite survey.

6.a)	Discuss the cases where the most probable values are likely to be calculated.

b)	Discuss the distribution of error to field measurements.

(Contd…2)

Code No.220106				-2-			Set No.2

7.a)	An angle A was measured by different persons and the following are the results.

| Angle | No. of measurements |
| --- | --- |
| 65o30'10'' 65o29'50'' 65o30'00' 65o30'20'' 65o30'10'' | 2 3 3 4 3 |

Find most probable value.

b)	How do you eliminate errors by (i) Method of repetition	(ii) Direction method.

8.	Write short notes on:

a)	Satellite station installation.

b)	Transition curves.

c)	Signals in triangulation stations.

***     ***     ***

Code No.220106

II-B.Tech. II-Semester Examination April/May 2003

Set No.

3

SURVEYING-II

(Civil Engineering)

Time: 3 hours							Max.Marks:80

Answer any FIVE questions

All questions carry equal marks

---

1.a)	Describe how you would measure vertical angle using theodalite.

b)	Explain the principles of Electronic Distance measurement (EDM).

2.a)	Explain the test and adjustment to be made for axis of plate level to be 	perpendicular to vertical axis.

b)	The record of a closed traverse is given below, with two distances missing.

| Line | Length, m. | Bearing |
| --- | --- | --- |
| AB | 100.5 | N30o30'E |
| BC | ? | S45o30'W |
| CD | 75 | S40o30'W |
| DE | 50.5 | S60oW |
| EA | ? | N40o15'W |

Calculate the length of BC and EA.

3.a)	Write a brief note on Tacheometer and stadia rod.

b)	A Tacheometer was set up at station P and the following readings were obtained 	on a staff held vertically.

| Instrument Station | Staff Station | Vertical Angle | Hair readings (m) | Remark |
| --- | --- | --- | --- | --- |
| P P | B.M Q | -5''20' +8o12' | 1.150, 1.800, 2.450 0.750, 1.500, 2.250 | R.L of BM = 750.50 |

Calculate the horizontal distance PQ and RL of Q, when the constants of the 	instrument are 100 and 0.15.

4.a)	What are the different methods of designating the sharpness of curve? Obtain the 	relationship between them.

b)	Two tangents intersect at chainage 1250m. The angle of intersection is 150o. 	Calculate all data necessary for setting out a curve of radius 250m by the 	deflection angle method. Peg interval may be taken as 30m.

5.a)	Show that the shift bisects the transition curve.

b)	Two straight lines AB and BC intersect at point B of chainage 100m. To avoid an 	obstacle, another line EF is taken to connect AB and BC, so that AEF = 135o 	and EFC = 145o. The radius of the first arc is 400m and that of second, 200m. 	Calculate the chainages of

i) The tangent points  and 	(ii) The point of compound curvature.

(Contd…2)

Code No.220106				-2-				Set No.3

6.a)	Derive an expression for curvature correction and for combined curvature and 	refraction correction.

b)	What are the reciprocal observations in trigonometric leveling? How is this 	approach superior to a single observation.

7.a)	Briefly explain the factors to be considered in selecting the triangulation stations.

b)	A, B and C are the stations in a minor triangulation survey. A satellite station S is 	set up near C such that AC and BC fall within the triangle ASB. If AC = 	7246.4m, BC = 6756.1m, CS = 44.5m

ASC = 62o35'20'' and ASB = 72o58'30'', calculate angle ACB.

8.a)	What is the principle of least squares? List the objectives of the “method of least 	squares”.

b)	Angles were measured on a station and the observations were recorded as follows:

| Angle | Value | Weight |
| --- | --- | --- |
| A | 45o30'10'' | 2 |
| B | 40o20'20'' | 3 |
| A+B | 85o50'10'' | 1 |

Find the most probable values of the angles A and B.

***     ***     ***

Code No.220106

Set No.

4

II-B.Tech. II-Semester Examination April/May 2003

SURVEYING-II

(Civil Engineering)

Time: 3 hours							Max.Marks:80

Answer any FIVE questions

All questions carry equal marks

---

1.a)	Define the following terms:

i)   Line of collimation	(ii) Axis of telescope	(iii) Trunnion axis

iv) Bubble axis and 	(v) Parallax.

b)	Explain the method of measuring horizontal angles by reiteration method. What 	errors are eliminated by this method?

2.a)	What do you understand by open traverse and closed traverse? Explain the 	various checks in closed traverse.

b)	Following are the lengths and bearings of a closed traverse ABCD.

| Line | Length,m | Bearing |
| --- | --- | --- |
| AB | 248 | 30o |
| BC | 320 | 140o |
| CD | 180 | 210o |

Calculate the length and bearing of line DA.

3.a)	Derive the distance equation in stadia method for horizontal line of sight, when 	the staff is held vertical.

b)	Determine the gradient from a point P to another point Q from the following 	observations made with a tacheometer fitted with an anallactic lens. The constant 	of the instrument was 100 and the staff was held vertical.

| Instrumentation Station | Staff Station | Bearing | Vertical angle | Staff readings |
| --- | --- | --- | --- | --- |
| R | P | 130o | +10o32' | 1.255, 1.810, 2.365 |
| R | Q | 220o | +5o06' | 1.300, 2.120, 2.940 |

4.a)	With the aid of a neat sketch, obtain the expressions for Tangent length, length of 	curve, length of long chord and mid-ordinate for a simple circular curve.

b)	Two tangents intersect at a chainage of 1000m, the deflection angle being 30o. 	Calculate all the necessary data for setting out a circular curve of 5.73o, by 	method of offsets from the chord produced. Take peg interval as 20m.

5.a)	What do you understand by “vertical curve”? What are the types? With sketches 	indicate the different cases, when each type of vertical curve is provided.

b)	A 3% rising gradient meets a 2% downgradient. A vertical curve 200m long is to 	be used.The pegs are to be fixed at 20m interval. Calculate the elevation of curve 	points (four points) by tangent corrections. The chainage and R.L. of the apex are 	1000m and 350m respectively.

(Contd…2)

Code No.220106				-2-				Set No.4

6.a)	What is a transition curve? What are its objectives and requirements?

b)	While surveying along the alignment of a road, the magnetic bearings of the lines 	AB, BC and CD are measured as 80o, 110o and 60o respectively. The length of BC 	is 200m. The chainage of B is 950m. It is proposed to introduce a reverse curve of 	equal radii, connecting the lines AB and CD and with BC as common tangent. 	Calculate the chainages of tangent points and point of reverse curvature.

7.a)	Explain the term ‘Axis-signal correction’ as used in trigonometric levelling.

b)	Derive an expression for the difference in elevation, when the observed angle is 	one of elevation.

8.a)	What is a satellite station? How would you reduce the horizontal angle (for any 	one case).

b)	With an example, explain the various laws of weight.

***     ***     ***