Code No.: 220802

Set No.

1

II-B.Tech. II-Semester Examinations April/May 2003

CHEMICAL ENGINEERING THERMODYNAMICS-1

(Chemical Engineering)

Time: 3 hours							Max. Marks: 80

Answer any FIVE questions

All questions carry equal marks

- - -

1.	State the first law of thermodynamics and prove that for a non flow process, it 	leads to the energy equation Q = W  +  du.

2.a)	What is a closed system? Show that ΔUt = Q + W for a closed system.

b)	Heat in the amount of 7.5 kJ is added to a closed system while its internal energy decreases by 12 kJ. How much energy is transferred as work? For a process causing the same change of state but for which the work is zero, how much heat is transferred?

3.	Consider an adiabatic vessel divided into 2 chambers. One chamber contains dry 		saturated steam of mass 4 kg at a pressure of 2.5 bar while the other chamber 	contains 8 kg of steam, 0.9 quality at 4 bar. Now the partition between the 	chambers is removed and the steam is mixed thoroughly and reaches thermal 	equilibrium. Calculate the entropy change, steam quality and final pressure.

4.	Hydrogen gas is contained in two constant volume vessels, each having a capacity 	of 500 liters. The pressure and temperature in vessel A are 5 bar and 300 K 	respectively, while in vessel B the values are 1.7 bar and 310 K. A valve connects 	the two vessels. The valve is now opened and the system is allowed to reach 	thermal equilibrium with the surroundings. If the surrounding temperature is 290 	K, what is the final pressure in the vessel?

5.	Explain in detail the carnot cycle of the heat engine and applicability of second 	law of thermodynamics.

6.	A reversible heat engine absorbs 252 kcal at 260C, produces work, and discards 	heat at 38C. What is the change in entropy of the heat source and the heat sink, 	and what is the total entropy change resulting from the process?

7.a)	Explain the Sensible Heat Effects with Homogeneous substance of 	constant 	composition.

b)	Evaluate the Sensible-Heat Integral. Derive the expression for it.

Contd…2

Code No.:220802				-2-				      Set No.1

8.a)	What is adiabatic flame temperature? Explain.

b)	Carbon monoxide reacts with water vapor to form carbon dioxide and hydrogen 	CO(g) + H2(g)  CO2(g) +H2(g)

∆Ho298 = -41.190kJ

the reactants are at 298K and is stoichiometric proportions. Seventy five percent 	of Co is converted in the reaction. The product leaves the reaction chamber at 	800K. The mean heat capacity 9J/mol.K) are 30.35 for CO, 45.64 for CO2, 36.00 	for water vapor, and 29.30 for H2. Determine the quantity of heat to be added 	or removed is the reaction chamber per 1000Kg of hydrogen produced.

^^^

Code No.: 220802

Set No.

2

II-B.Tech. II-Semester Examinations April/May 2003

CHEMICAL ENGINEERING THERMODYNAMICS-1

(Chemical Engineering)

Time: 3 hours							Max. Marks: 80

Answer any FIVE questions

All questions carry equal marks

- - -

1.	Prove that the work interaction in a steady flow process is given by -  Pdv. State 	clearly the assumptions, if any.

2.a)      What do you understand by the terms system and surroundings? Explain briefly.

b)	If a tank containing 20 kg of water at 20oC is fitted with a stirrer that delivers work to the water at the rate of 0.25 kW, how long does it take for the temperature of the water to rise to 30oC if no heat is lost from the water to its surroundings? For water, Cp = 4.18 kJ/kg oC.

3.	A diesel engine has a compression ratio of 17.5:1. Air at 300 K and 101 kPa 	enters the engine during the intake stroke and is compressed reversibly and 	adiabatically during the compression stroke. Determine the final temperature and 	pressure of the air at the end of compression stroke and the work required per kg 	of air present using,

a) the gas tables

b) the constant specific heat ideal gas isentropic process equations.

4.	Nitrogen gas at a temperature of 175 K has a specific volume of 0.0037 m3/kg. 	Determine the pressure using

a) the nitrogen table.

b) the van der Waals equation of state

c) the Beattie-Bridgeman equation of state, and

d) the Benedict-Webb-Rubin equation of state.

5.	Derive an expression for the efficiency of heat engine.

6.	A steel casting [CP=0.12kcal/kgC] weighing 34kg and having a temperature of 	427C is quenched in 135 of oil Cp = 0.6 k cal/kg oC. If there areno heat losses 	what is the change in entropy of (a) the casting (b) the oil, and (c)	both 	considered together?

7.	Explain the Temperature dependence of the Heat Capacity for Ideal gases 	such as 	Argon, Nitrogen, Water and Carbon dioxide.

Contd…2

Code No.:220802				-2-				      Set No.2

8.a)	A gas mixture consisting of 20% ethane and 80$ oxygen at 298K is burned 	completely  after diluting it with double the volume of CO2 at the same 	temperature 298K. the mean heat capacities is (J/mol.K) are 54.56 for CO2, 43.02 	for water vapor, and 35.52 for O2. The standard heat of reaction of ethane at 298K 	is(-1478)kJ/mol. Determine the theoretical flame tem-perature.

b)	What is the necessity of standard states for calculations involving heat effects.

^^^

Code No.: 220802

Set No.

3

II-B.Tech. II-Semester Examinations April/May 2003

CHEMICAL ENGINEERING THERMODYNAMICS-1

(Chemical Engineering)

Time: 3 hours							Max. Marks: 80

Answer any FIVE questions

All questions carry equal marks

- - -

1.	Prove that work done in a steady flow process is given by integral 2 to 1 v dp 	when volume changes from v1 to v2.

2.a)	State the first law of thermodynamics.

b)	One kilogram of air is heated reversibly at constant pressure from an initial state 	of 300 K and 1 bar until its volume triples. Calculate W, Q, ∆U, ∆H for the 	process. Assume that air obeys the relation PV/T = 83.14 bar cm3/mol K and that 	Cp=29J/mol K.

3.a)	What is equation of state? Discuss the ideal gas equation of state.		      	            b)	A room has a dimensions of 3m  4m  5m. What is the mass of air in this room 	if the air pressure and temperature are 1 bar and 300K respectively?

c)	An ideal gas undergoes a reversible adiabatic process. Derive an expression for 	the changes in internal energy and enthalpy in terms of the pressure ratio.

4.a)	Define reduced pressure and reduced temperature.

b)	Two rigid tanks are connected by a valve. Tank I has a volume of 1 m3 contains 	air at 0.55 MPa and 27C. Tank II contains 6 kg of air at 0.25 MPa and 37C. The 	valve is now opened and the system is allowed to reach thermal equilibrium with 	the surroundings which are at a temperature of 25C. Find the volume of the 	second tank and the final equilibrium pressure of air.

5.	Write briefly about the limitations and real process of second law of 	thermodynamics.

6.	An inventor claims to have devised a process using only saturated steam at 100C 	which will, by a complicated series of steps, make heat continuously available at a 	temperature of 175C. He claims further that, for every pound of steam taken into 	the process, 200 Kcal of energy as heat is liberated at this higher temperature of 	175C. Show whether or not such a process is possible. In order to give the 	inventor the benefit of any doubt, assume cooling water to be available in 	unlimited quantity at a temperature of 0C.

Contd…2

Code No.:220802				-2-				      Set No.3

7.	If the heat capacity of a substance is correctly represented by an equation of 	the form, 	CP = A + BT + CT2

Show that the error resulting when < CP>H is assumed equal to CP evaluated at 	the arithmetic mean of the initial and final temperature is C(T2 – T1)2 / 12.

8.	The standard heat of combustion of graphite at 298(K) is (-393.778)kJ/Mol. 	Determine the heat of combustion at 800K. The heat of capacities are (J/mol.K):

Carbon: 11.19 + 1.096 * 10-2 T – 4.894 * 105/T2

Oxygen: 34.62 + 1.08 * 10-3T – 7.859 * 105/T2

CO2 : 43.29 + 1.147 * 10-2T – 8.185*105/T2

Explain process steps and the thermodynamic principle involved.

^^^

Code No.: 220802

Set No.

4

II-B.Tech. II-Semester Examinations April/May 2003

CHEMICAL ENGINEERING THERMODYNAMICS-1

(Chemical Engineering)

Time: 3 hours							Max. Marks: 80

Answer any FIVE questions

All questions carry equal marks

- - -

1.	Derive the governing equation for the adiabatic process.

2.a)	What is internal energy of a substance? Explain briefly.

b)	Liquid water at 180oC and 1,002.7 kPa has an internal energy (on an arbitrary 	scale) of 762.0 kJ/kg and a specific volume of 1.128 cm3/g.

i)	What is its enthalpy?

ii)	The water is brought to the vapor state at 300oC and 1,500 kPa, where its 				internal energy is 2,784.4 kJ/kg and its specific volume is 169.7 cm3/g. 				Calculate ΔU and ΔH for the process.

3.	Consider isentropic compression of air from 1.01 bar, 35C to 5.5 bar. The air is 	then expanded isobarically to the original volume. Show the processes on the p-v 	and T-s planes. Also determine the total heat transfer and the total work transfer. 	Assume the mass of air as 0.75 kg.

4.a)	Discuss the principle of corresponding states.					              b)	Find the specific volume of refrigerant-12 at 0.8 MPa and 40C using

i) the R-12 tables

ii) the ideal-gas equation of state, and

iii) the generalized compressibility chart.

5.	Derive an expression for the efficiency of reversible heat engine.

6.	A metal block of specific heat 0.09kcal/kg C and mass of 10kg is initially at 	50C

i)	2 kg of water at 35C is used to quench the block, what is the total entropy 		change?

ii)	Repeat if 2 kg of water at 15C is used

Comment on the above process.

7.	For steady flow in a heat exchanger at appropriate atmospheric pressure, 	what 	is the final temperature?

(a) When heat in the amount of 800 kJ is added to 10 mol of ethylene initially at 	    	      200C?

(b) When heat in the amount of 2,500 kJ is added to 15 mol of 1-butene initially 		      at 260C?

Contd…2

Code No.:220802				-2-				       Set No.4

8.a)	Explain: Heat of formation and heat of combustion with examples.

b)	What is “ Mole of reaction”? Explain.

c)	Heat of combustion of solid carbon and gaseous carbon monoxide are 	respectively. (-393.78 kJ/mol and (-283.18) kJ/mol. Determine the heat of 	formation of CO.

^^^