Code No:220803

Set No.

1

II-B.Tech. II Semester Examinations April/May, 2003

FLUID MECHANICS

(Chemical Engineering)

Time: 3 Hours							  Max. Marks: 80

Answer any FIVE questions

All questions carry equal marks

---

1.	A circular plate 3metres in diameter is submerged in water in such a way that 	the greatest and least depth of the surface (below water surface) are 2 metres and 	1 metre respectively. Calculate (i) the total pressure on one side of the surface 	(ii) the position of center of pressure.

2.a)	What do you mean by turbulence ?

b) 	What do you mean by one, two, three dimensional flow?

c)	Derive the continuity equation for one dimensional fluid flow.

3.a)	Derive an expression for the loss due to sudden contraction in pipe flow .

b)	A horizontal pipe of diameter 450 mm is suddenly contracted to a diameter of 200mm. The pressure intensities in the large and smaller pipe are given as          14 N/cm2 and 12 N/cm2 respectively.  Find the loss of head due to contraction if Cc = 0.62. Also determine the rate of flow of water.

4.a)	State the Bernoulli’s theorem for compressible flow. Derive an expression for

Bernoulli’s equation when the process is isothermal and adiabatic.

b)	Measuring the Mach angles  is an accurate means of measuring supersonic velocities for sharp nosed objects such as projectiles. Estimate the velocity and the Mach number of a projectile which reflects a shock wave of angle =30o in air at a pressure of 93.2kN/m2 and at a temperature of 3oC.

5.	A fluidized bed containing 32,500 kgs of 0.15 mm sand is to be fluidized with air at 4000C and a pressure of 17 atm abs in a cylindrical vessel of 3.05 m in diameter. The density of particle is 2690 kg/m3. The viscosity of air is 0.032 C.P. Calculate (i) Minimum height of fluidized bed (ii) The pressure drop in the bed and (iii) Critical superficial air velocity. Minimum bed porosity is 0.55.

6.	Explain the following in brief:

a) Gate valves and globe valves

b) Plug cocks and ball valves

c) Check valves.

Contd…..2.

Code No. 220803				::2::				Set No.1

7.	A centrifugal fan is used to take flue gas at rest and at a pressure of 730mm Hg and a temperature of 900C  and discharge it at a pressure of 760mm Hg and a velocity of 50 m/sec.  Calculate the power needed to move 17000 m3/hour of gas.  The efficiency of the fan is 60% and the molecular weight of the gas is 32.0.

8.	Explain Venturimeter in detail. Also derive the expression for discharge through a 	venturimeter.

@@@@@

Code No:220803

Set No.

2

II-B.Tech. II Semester Examinations April/May, 2003

FLUID MECHANICS

(Chemical Engineering)

Time: 3 Hours							  Max. Marks: 80

Answer any FIVE questions

All questions carry equal marks

---

1.	A hollow circular plate of 2m external and 1m internal diameter is immersed 	vertically in water such that the center of the plate is 4m deep from water 	surface, find the total hydrostatic pressure and the depth of center of pressure.

2.a)	State the different types of energy of a flowing fluid. Give expressions for them 	in fluid flow.

b) 	What is continuity equation?  Derive the equation.

3.a)	Find an expression for the power transmission through pipes.  What is the condition for maximum tramission of power.

b)	A pipe of diameter 250 mm and length 3500 m is used for the transmission of power by water.  The total head at the inlet of the pipe is 500 m.  Find the max. power available at the out let of the pipe.  Take f=0.006

4.a)	What do you understand by stagnation pressure ? Obtain an expression for stagnation pressure of a compressible fluid in terms of approaching Mach number and pressure.

b)	A gas with a velocity of 300m/s is flowing through a horizontal pipe at a section where pressure is 6 x 104 N/m2 (absolute) and temperature 40oC. The pipe changes in diameter and at this section the pressure is 9 x 104 N/m2. Find the velocity of the gas at this section if the flow of the gas is adiabatic.

5.a)	Derive the Euler’s equation for flow through packed beds.

b)	A column of a 942cm2 cross section and 185 cm high is packed with spherical particles of diameter 2 mm when the pressure difference of 10.75 atmosphere is maintained across the bed, a 60% aqueous sucrose solution with density 1.2865 gm/cc and viscosity 56.5 c.p, flows through a bed at a rate of 6640 kg/hr. What is the void fraction of the bed.

6.a)	What is the necessity of providing bends or loops in the pipe?

b)	Name different types of fluid moving machinery.

c)	Write short notes on pipe fittings.

Contd…..2.

Code No. 220803				::2::				Set No.2

7.	Air entering at 210C and atmospheric pressure is to be compressed to 2756x104 N/m2 gauge in a reciprocating compressor at the rate  of 125   m3 /s. How 	many stages should be used?  What is the theoretical shaft work for frictionless adiabatic compression?  What is the brake horsepower if the efficiency of each stage is 85 percent?  For air r (gamma) =1.40.

8.	Explain Orificemeter in detail.  Also derive an expression for discharge through an orificemeter.

@@@@@

Code No:220803

Set No.

3

II-B.Tech. II Semester Examinations April/May, 2003

FLUID MECHANICS

(Chemical Engineering)

Time: 3 Hours							  Max. Marks: 80

Answer any FIVE questions

All questions carry equal marks

---

1.	One side of a water tank is inclined at 60 to horizontal in which a gate is 	provided. The gate is 1.5m long along the slope and is 1m wide. Water stands

to a height of 10m over the top edge of the gate. Determine the total hydrostatic 	force on the gate and its center of action.

2.a)	Write three important basic equations of fluid flow.  Explain them.

b)	What are types of flow?

3.a)	The velocity distribution for laminar flow in a round pipe of radius ‘R’ is given by the equation uUmax=1rR2

Where ‘u’ is the velocity at a radius r and Umax is the maximum velocity at the centre line.  Determine the value of the kinetic energy and momentum correction factors if the average velocity is used to represent the flow.

b)	State the Hazen Poiseulle equation and explain.

4.a)	Explain the concept of velocity of sound or pressure wave in a fluid.

b)	Differentiate between isentropic and adiabatic process.

c)	Find the mass rate of flow of air through a venturimeter having inlet diameter as 30cm and throat diameter 15cm. The pressure at the inlet of venturimeter is 1.4 kgf/cm2 ( 1.4 x 9.81 N/ cm2 ) absolute and temperature of air at inlet 15oC. The pressure at the throat is given as 1.3 kgf/cm2 ( 1.3 x 9.81 N/cm2) absolute.

Take  R= 287 J/ kg K and k=1.4.

5.	Catalytic cracking of the vapors of petroleum fraction of density 0.648 kg/m3, viscosity 0.15x10-4 kg/m. sec are carried out in a packed bed reactor of 0.162 M diameter and packed randomly with the cube shaped catalyst particles 0.005 meter size. The height of packing is 1.8 m. The density of the material of the particle is 1620 kg/m3 and the bulk density is 972 kg/m3. If the superficial velocity is 0.9m/sec, calculate the pressure drop across the bed.

Contd…..2.

Code No. 220803				::2::				Set No.3

6.a)	In designing and installing  a piping system what are the points that are to be 	considered.

b)	What are the advantages and disadvantages of positive displacement pumps?

7)	It is proposed to pump 10,000 kg/h of toluene at 1140C and 1.1 atm abs. Pressure from the reboiler of a distillation tower to a second distillation unit without cooling the toluene before it enters the pump  .If the friction loss in the line between the reboiler and pump is 7KN/m2 and the density of toluene is  866 Kg/m3.  How far above the pump must the liquid level in the reboiler be maintained to give a net positive suction head of 2.5m?

8)	Explain Pitot tube in detail. What is the difference between pitot tube and pitot static tube?

@@@@@

Code No:220803

Set No.

4

II-B.Tech. II Semester Examinations April/May, 2003

FLUID MECHANICS

(Chemical Engineering)

Time: 3 Hours							  Max. Marks: 80

Answer any FIVE questions

All questions carry equal marks

---

1.a)	What is the hydrostatic paradox? Obtain an expression for the pressure intensity 	at a point in a static fluid.

b)	Calculate the pressure at the top surface of a submarine 1.5km below the free 	surface of the sea. The mass density of sea water may be taken as 1025kg/m

2.a)	Define Velocity head, Datum head and pressure head.  What do they represent.

b)	What is head lost between two sections in the direction of flow.

3.a)	Derive an expression of velocity distribution for turbulent flow through smooth pipes.

b)	A pipe line carrying water has average height of irregularities projecting from the surface of the boundary of the pipe as 0.15 mm. What type of boundary is it?  The shear stress developed is 4.8 N/m2 Take kinematic viscosity as 0.01 stokes.

4.a)	Derive the governing differential equation for adiabatic flow with friction in long ducts.

b)	Air flows isentropically through a duct of non – uniform section. The pressure, temperature and velocity at a particular section are 0.85 bars (abs), 38oC and 350 m/s respectively. Determine the temperature and velocity at another larger section where the pressure is 1.32 bars( abs). Also find the Mach numbers at the two sections. Assume isentropic flow.

5.a)	What is Stoke’s law ? Is hindered settling more efficient than free settling? If so why?

b)	Spherical particles of quartz 15µm in diameter are to be settled from their mixture with water at 200 C. The specific gravity of quartz is 2.65. The viscosity of water may be taken as 1 c.p. A settling time of 1 min. is available. How high should be chambers be to allow settling of these particles.

6.a) 	Explain vaccum pumps and Jet ejectors in detail.

b) 	Compare the various devices used for moving fluids.

Contd…..2.

Code No. 220803				::2::				Set No.4

7.	Air entering at 15.60C and atmosphere pressure is to be compressed to            2067 x 104N/m2 gauge in a reciprocating compressor at the rate of 120 m3/sec.  How many stages should be used?  What is the brake horse power if the efficiency of each stage is 75 percent?  What is the discharge temperature of the air from the first stage?

8.	Write shorts notes on:

a)   Measurement of flowing fluids.

b)   Full boremeters.

c)   Insertion meters.

@@@@@