Code No:222102

Set No.

1

II-B.Tech. II-Semester Examinations, April/May, 2003

ELECTRONICS AND INSTRUMENTATION

(Aeronautical Engineering)

Time: 3 hours							         Max. Marks:80

Answer any FIVE questions

All questions carry equal marks

- - -

1.a)	Describe any typical relay used with pneumatic controllers.

b)	A wheel has 48 teeth of diametral pitch 4.  Find the pitch diameter and the circular pitch.

2.a)	Describe the working of a double bellows differential pressure gauge.

b)	Deduce the equation for the limiting ratio of the tensions at the two ends of a belt or rope which is in contact with a pulley rim when (i) the pulley rim is flat         (ii) when the pulley rim is grooved and the belt or rope wedges between the sides of the groove.

3.a)	Discuss the chief advantages of ball and roller bearings over plain journal bearings.

b)	What is the smallest number of teeth theoretically required in order to avoid interference as a pinion which is to gear with (i) rack (ii) an equal pinion (iii) a wheel to give ratio 3 to 1.

4.a)	Describe how the phase angle and time delay are measured using an oscilloscope.

b)	The reverse bias saturation for a P-N junction diode is 1A at 300K.  Determine its ac resistance at 150mv forward bias.

5.a)	Show the different current components in a transistor and prove

IC = IPC + ICO.

b)	Describe the working of a zener diode.  Distinguish between zener break down and avalanche break down.

6.a)	Explain the following:

(i) why is a transistor low powered device.

(ii) why is base made thin.

(iii) why is collector current is slightly less than emitter current.

(iv) why is collector wider than emitter and base.

b)	Define  and  in a transistor circuit and show =α1−α.  Also show  is always less than unity.

Contd……2

Code No:222102				-2-				Set No:1

7.a)	Differentiate between magnetic-deflection sensitivity and electrostatic sensitivity in a cathode ray tube.

b)	In a transistor circuit  = 45, the voltage across 5k resistance which is connected in collector circuit is 5 volts.  Find the base current.

8.	Write short notes on:

(a) Counters

(b) Rack and pinion

(c) Boosters

(d) Applications of photo diode.

- - -

Code No:222102

Set No.

2

II-B.Tech. II-Semester Examinations, April/May, 2003

ELECTRONICS AND INSTRUMENTATION

(Aeronautical Engineering)

Time: 3 hours							         Max. Marks:80

Answer any FIVE questions

All questions carry equal marks

- - -

1.a)	Explain how bellows elements are used for measurement of  absolute, gauge and differential pressures.

b)	A wheel has 48 teeth of diametral pitch 4.  Find the pitch diameter and the circular pitch.

2.a)	Discuss the relation between the nozzle back pressure to the nozzle flapper distance in a pneumatic flapper arrangement.

b)	What is the smallest number of teeth theoretically required in order to avoid interference on a pinion which is required to gear with (i) a rack  (ii) an equal pinion (iii) a wheel to give ratio 3:1.

3.a)	In a transistor =45, the voltage across 5 k resistance which is connected in collector circuit is 5 volts.  Find the base current.

b)	What are the advantages and disadvantages of FET over the conventional transistor.

4.a)	Explain the following: (i) why is a transistor low powered device?  (ii) why is collector wider than emitter and base?  (iii) why is base made thin?  (iv) why is collector current slightly less than emitter current?

b)	A potential difference of 2kV is applied in a cathode ray oscilloscope to accelerate an electron.  Find its final velocity.

5.a)	Discuss the volt-ampere characteristics of an avalanche diode.  What is meant by knee of the curve, dynamic resistance and temperature coefficient.

b)	A silicon diode has a forward voltage drop of 1.1 volt for a forward d.c. current of 100mA.  It has reverse current of 1A for a reverse voltage of 10V.  Calculate: (i) bulk and reverse resistance of the diode.  (ii) ac current at forward dc current of (i) 2.5mA and (ii) 25mA.

6.a)	How much voltage is required across two deflection plates separated by 1cm to deflect an electron beam 1o if the effective length of deflection plates is 2cm and the accelerating potential is 1000V.

b)	Show the different current components in a transistor and prove

IC = IPC + ICO.

Contd……2

Code No:222102				-2-				Set No:2

7.a)	Explain how a tunnel diode may be thought of as a heavily doped zener diode.  Does a tunnel diode have rectifying property?  Give two applications of tunnel diode.

b)	Differentiate between magnetic-deflection sensitivity and electrostatic sensitivity in a cathode ray tube.

8.	Write short notes on:

(a) Geneva mechanism

(b) Varactor diode

(c) Applications of photo diode.

(d) Frequency measurement using CRO.

- - -

Code No:222102

Set No.

3

II-B.Tech. II-Semester Examinations, April/May, 2003

ELECTRONICS AND INSTRUMENTATION

(Aeronautical Engineering)

Time: 3 hours							         Max. Marks:80

Answer any FIVE questions

All questions carry equal marks

- - -

1.a)	Discuss the chief advantages of ball and roller bearings over plain journal bearings.

b)	A wheel has 48 teeth of diametral pitch 4.  Find the pitch diameter and the circular pitch.

2.a)	Deduce the equation for the limiting ratio of the tensions at the two ends of a belt or rope which is in contact with a pulley rim when (i) the pulley rim is flat  (ii) when the pulley rim is grooved and the belt or rope wedges between the sides of the groove.

b)	Describe any typical relay used with pneumatic controllers.

3.a)	Describe the working of a double bellows differential pressure gauge.

b)	What is the smallest number of teeth theoretically required in order to avoid interference on a pinion which is to gear with (i) a rack (ii) an equal pinion (iii) a wheel to give ratio 3 to 1.

4.a)	Describe the working of a zener diode.  Distinguish between zener break down and avalanche break down.

b)	The reverse biasd saturation for a P-N junction diode is 1 A at 300k.  Determine its ac resistance at 150 mv forward bias.

5.a)	Describe how the phase angle and time delay are measured using an oscilloscope.

b)	A silicon diode has a forward voltage drop of 1.1 volt for a forward d.c current of 100 mA.  It has reverse current of 1 A for a reverse voltage of 10V.  Calculate (i) bulk and reverse resistance of the diode.  (ii) a.c. resistance at forward d.c. current of (i) 2.5 mA and (ii) 25 mA.

6.a)	Describe the working of a varactor diode.

b)	Given a 50 W mercury vapour lamp.  Assume that 0.1 percent of the electric energy supplied to the lamp appears in the ultraviolet line, 2537 Ao.  Calculate the number of photons per second of this wave length emitted by the lamp.

Contd……2

Code No:222102				-2-				Set No:3

7.a)	Differentiate between magnetic-deflection sensitivity and electrostatic sensitivity in a cathode ray tube.

b)	In a PNP transistor operating in active region the collector current equals 6.6 mA.  Compute the value of .  Neglect ICo.

8.	Write short notes on:

a) current flow in NPN transistor.

b) rectifying property and applications of tunnel diode.

c) Geneva mechanism.

d) Applications of photo diodes.

- - -

Code No:222102

Set No.

4

II-B.Tech. II-Semester Examinations, April/May, 2003

ELECTRONICS AND INSTRUMENTATION

(Aeronautical Engineering)

Time: 3 hours							         Max. Marks:80

Answer any FIVE questions

All questions carry equal marks

- - -

1.a)	In a PNP transistor operating in active region, the collector current equals to 6.6 mA.  Compute the value of .  Neglect ICO.

b)	Describe the motion of an electron in a transverse magnetic field and show that the period and the angular velocity are independent of speed and radius.

2.a)	Define deflection factor of a CRT.  What is the velocity of electrons that have been accelerated through a potential of 2000V.

b)	Differentiate between intrinsic and extrinsic semiconductors with examples.

3.a)	Discuss how a P-N diode may be used as a switch in electrical circuit.

b)	Show the different current components in a transistor and prove

IC = IPC + ICO.

4.a)	Describe any typical relay used with pneumatic controllers.

b)	Deduce the equation for the limiting ratio of the tensions at the two ends of a belt or rope which is in contact with a pulley rim when (i) the pulley rim is flat (ii) when the pulley rim is grooved and the belt or rope wedges between the sides of the groove.

5.a)	Derive the equation to find the minimum number of teeth on the pinion in order to avoid interference.

b)	What is the smallest number of teeth theoretically required in order to avoid interference on a pinion which is to gear with (i) a rack (ii) an equal pinion (iii) a wheel to give a ratio 3:1.

6.a)	Explain how a tunnel diode may be thought of as a heavily doped zener diode.  Does a tunnel diode have rectifying property?  Give two applications of tunnel diode.

b)	The reverse bias saturation for a P-N junction diode is 1A at 300K.  Determine it’s a.c. resistance at 150 mV forward bias.

7.a)	Describe the working of a varactor diode.

b)	A wheel has 48 teeth of diametral pitch 4.  Find the pitch diameter and the circular pitch.

8.	Write short notes on:	(a) Applications of photo diodes. (b) Rack and pinion.

(c) Geneva mechanism. (d) Frequency measurement using CRO.

- - -