Code No. 221401

Set No.

1

II-B.Tech. II-Semester Examinations April/May, 2003

ELECTRONIC DEVICES AND CIRCUITS

(Mechatronics)

Time: 3 hours							Max. Marks: 80

Answer any Five questions

All questions carry equal marks

---

1. a)	What are the principal features of a bridge rectifier?

b)	How a zener diode used as a voltage regulator in power supply system?  Explain.

c)	Calculate the voltage drop across 5 resistance and current possing through the zener diode for the circuit given.

Fig.

2. a)	Explain the working of a pnp transistor with input and output characteristics.  Hence explain cut-off; active and saturation regions.

b)	Compare MOSFET with JFET.  Discuss the handling precautions for MOSFET.

3. a)	What is meant by load line and what is its importance?

b)	Draw the RC coupled amplifier and its equivalent and find the voltage gain.  Why the gain decreases in high frequency region?

c)	A 3-stage RC coupled CE amplifier uses transistor having hie = 4.5k, hfe = 330. If the load resistance is 10k and source resistance negligible, compared to hie.  Calculate the over all gain for mid frequency range.

4. a)	Draw a MOSFET biasing  circuit and obtain an expression for forward biasing of gate to source junction.

b)	Draw a small signal FET model for CS connection and derive expression for input, output impedances and voltage gain.

5. a)	What is Barkhausen Criterion?  Explain the working of Hartley and colpitts’s oscillators.  Derive the necessary expressions for frequency.

b)	A CE amplifier with voltage shunt feedback has the following values.

Rs = 600; hie= 5k; hfe = 80; RL = 2k; RB = 40 k. Calculate Av; Rif; Avf; Rof and Rof’.

Contd…2

Code No. 221401				.2.				Set No. 1

6. a)	(i) Draw the circuit diagram of a Wein Bridge Oscillator and explain its principle of operation.

(ii) Derive an expression for frequency of oscillation.

(iii) Show that amplifier used in a Wein Bridge oscillator must have a gain of greater than 3 for sustained oscillation.

b)	Discuss the application of UJT.

7. a)	Describe the construction and MOSFET working.

b)	What do you understand by the term “ripple factor”?  Derive an expression for the ripple factor of a FWR using L-C filter.

8.	Write notes on

(a) Advantages of Bridge rectifier over FWR.

(b) UJT

(c) Merits and demerits of Darlington pair.

###

Code No. 221401

Set No.

2

II-B.Tech. II-Semester Examinations April/May, 2003

ELECTRONIC DEVICES AND CIRCUITS

(Mechatronics)

Time: 3 hours							Max. Marks: 80

Answer any Five questions

All questions carry equal marks

---

1. a)	What is “tunneling”?  Explain the V-I characteristics of a tunnel diode from energy band diagram.

b)	The diode current is 0.6ma when applied voltage is 400mV and 20ma when applied voltage is 500mv. Determine the constant .  Given KTe = 25mV.

2. a)	What is a MOSFET?  How many types of MOSFET are there?  With suitable diagrams explain the working of different MOSFETS.

b)	Compare CB; CE and CC Configurations.

3. a)	Explain self bias and fixed bias arrangements in an amplifier.  Discuss the importance of load line.

b)	Draw the ac equivalent of a CE amplifier with unbypassed emitter resistor using h-parameter model and re model and derive the equations for input and output impedances.

4. a)	Draw a FET RC-coupled amplifier circuit in CS connection and derive expressions for voltage gain at low frequency, mid frequency and at high frequency.

b)	For the given circuit calculate the voltage gain Rs = 2k, µ=20, rd=40

Fig…

Contd…2

Code No. 221401				.2.				Set No. 2

5. a)	How does negative feedback reduce distortion in an amplifier? The distortion in an amplifier is found to be 3% when the feedback ratio of a negative feedback amplifier is 0.04.  When the feedback removed, the distortion becomes 15% find the open loop gain and close loop gain.  Derive the expression you have used.

b)	Describe the constriction and working of a RC-phaseshift oscillator.

6. a)	Draw the circuit diagram of high input resistance amplifier using a Darlington pair and derive  expressions for its current gain, voltage gain, input impedance and output impedance.

b)	A wein bridge oscillator is to cover a frequency range from 20Hz to 20kHz.  The capacity of the variable capacitor varies from 30pf to 300pf.  Calculate the values of resistances required to cover the frequency range.

7. a)	A silicon diode has a saturation current of 0.1 pa at 20oC.  Find its current when it is forward biased by 0.55 volts.  Find the current in the same diode when the temperature rises to 100oc.

b)	Derive the relation

(i) (1 – α) = (1 +)-1

(ii) ICEO = (1 + ) ICBO.

8.	Write notes on

a)	Comparison of BJT and JFET

b)	Frequency and amplitude stability of an oscillator.

c)	UJT.

###

Code No. 221401

Set No.

3

II-B.Tech. II-Semester Examinations April/May, 2003

ELECTRONIC DEVICES AND CIRCUITS

(Mechatronics)

Time: 3 hours							Max. Marks: 80

Answer any Five questions

All questions carry equal marks

---

1. a)	Distinguish between avalanche and Zener mechanism.

b)	Show that a p-n diode works as a rectifier.

c)	Find the conductivity of silicon.

(i) intrinsic condition at room temperature 300ok.

(ii) With donor impurities of 1 in 108.

(iii) With acceptor impurity of 1 in 107.

(iv) Both the impurities present simultaneously.

Given ni for silicon at 300ok is 1.5 x 1010/cm3.  µn = 1300 cm2 /s.v,                       µp = 500 cm2/V-S. Number of Si atoms per cm3 = 5 x 1022.

2. a)	Describe working of a npn transistor and draw the input and output characteristics.

b)	Discuss the advantages of JFET over BJT.

For a N-channel FET, find the values of (i) ID (ii) gmo (iii) gm if IDSS = 6.3ma,      vp = -3v; VGS = -1Volt.

3. a)	Draw the circuit diagram of a CC amplifier and explain its working.

b)	Determine the input impedance output impedance, voltage gain and current gain of a CE amplifier using h- parameters and re model equivalents for the transistor with hie= 3.2k and hfe = 100 at the operating conditions.

4. a)	Draw the biasing circuit for enhancement MOSFET.  Explain how gate –source junction is forward biased.

b)	Draw the small signal equivalent circuit of  FET amplifier in CS connection and derive the equation for voltage gain.

5. a)	Explain the main difference between an amplifier and an oscillator?  What are the main constituents parts of an oscillator.

b)	An amplifier has voltage gain with feedback is 100.  If the gain without feedback changes by 20% and gain with feedback should not vary more than 2%.  Determine the open loop gain A and feedback ratio B.

Contd…2

Code No. 221401				.2.				Set No.3

6. a)	Derive an expression for the frequency of oscillation and minimum gain required for sustained oscillations of the RC phase shift oscillator.

b)	Draw the over all voltage gain of a Cascade amplifier shown Given

RB1 = 7k; RB2 = 5.6; RB3 = 4k.

RC = 2k; RE = 1k

1 = 2 = 100 and VCC = 18V.

Fig.

7. 	A C E RC coupled amplifier uses transistors with the following parameters.

hfe = 50; hie = 1.2k;  hoe = 3 x 10-6 ℧; hre = 2.5 x 10-4.  The value of gm at operating point is 50m℧.  The biasing  resistor R1 between Vcc and base is 100k and R2 between base and ground is 10k.  The total resistance Rc = 5 k. Let C = 160pf be the total shunt capacitance in the input circuit and Cc = 6µf.  Calculate for one stage of amplifier.

(i) Mid bond current gain	(ii) Mid band voltage gain.

(iii) Lower and higher 3dB frequencies and 	(iv) Gain Bandwidth product.

8.	Write notes on:

a) Diffusion and Transition capacitance.

b) UJT

c) Crystal oscillator.

###

Code No. 221401

Set No.

4

II-B.Tech. II-Semester Examinations April/May, 2003

ELECTRONIC DEVICES AND CIRCUITS

(Mechatronics)

Time: 3 hours							Max. Marks: 80

Answer any Five questions

All questions carry equal marks

---

1. a)	Show that in an intrinsic semiconductor, the Fermi level is located at the middle of the unallowable energy gap.

b)	What is a rectifier?  Compare the advantages and disadvantages of HWR and FWR.

c)	A FWR with a load resistance of 15k uses an inductor filter of 15 henry.  The peak value of the applied voltage is 250 volts and frequency is 50Hz.  Calculate the d.c. load current, ripple factor and d.c output voltage.

2. a)	Explain the construction and characteristics of a JFET.  How do you determine its characteristics.

b)	A change of 0.5ma in the emitter current of a transistor produces a change of     0.49 ma in collector current.

Calculate i) CB short circuit current gain α .

ii) CE short circuit current gain 

3. a)	Define stability factor with reference to reverse saturation current and obtain the stability factor for a self-bias circuit.

b)	The mid frequency gain of an RC coupled amplifier is 200.  The values of lower and upper cutoff frequencies are f1 = 50Hz and f2 = 100kHz.  Find the frequencies at which the gain is reduced to 150.

4. a)	Draw a small signal equivalent circuit of a FET amplifier in CG connection and derive an expression for voltage gain.

b)	In a FET amplifier, R2 = 20k; Rs = 1k; RG = 1M Cs = 25µf and µ = 20,       rd = 100k.  If input voltage is 2 volts at 1kHz frequency.  Calculate amplifier output signal.

5. a)	What is negative feedback?  Discuss the effect of negative feedback on (i) gain (ii) bandwidth (iii) Input impedance and output impedance of an amplifier.

b)	Calculate C2 of a Colpitt’s oscillator is L = 100mH; C1 = 0.1µf and resonance frequency f = 100 kHz.

Contd…2

Code No. 221401				.2.				Set No. 4

6. a)	What is a unijunction transistor?  Why is it so called?  Draw its equivalent circuit and explain.

b)	A UJT has a firing potential of 10 volts.  If its connected across the capacitor of a series R-C circuit with R = 100k and C = 100µf supplied by a source of 20 volt dc; Calculate the time period of the sawtooth waveform generated.

7. a)	Explain how UJT works as a relaxation oscillator.

b)	Why is Crystal oscillator is used in radio transmitter?  How does crystal oscillator gives constant frequency?

8.	Write notes on:

a)	L.C oscillators

b)	Tunnel diode

c)	Wein bridge oscillator.

###