Code No.221003

Set No.

1

II-B.Tech. II-Semester Examination April/May 2003

PULSE AND DIGITAL CIRCUITS

(Common to Electronics and Information Engineering, Electrical and Control Engineering)

Time:3 hours								Max.Marks: 80

Answer any FIVE questions

All questions carry equal marks

---

1.a)	Verify V1 = 	   V	  ,       V11 =		V	for a

1+e– T/2RC			    1+e+ T/2RC

symmetrical square wave applied to a high pass RC circuit.

b)	Draw the RC high pass circuit and explain its working with step voltage input.

2.a)	Give the circuits of series clipper circuits and explain their operation

with the help of transfer characteristics.

b)	For the circuit shown in figure below.

VR = 10V,    Vi = 20 Sin t

Rf = 100  Rr = 

V = 0  Sketch the input and output waveforms if R = 1 K

3. 	Define rise time and fall time of transistor switch. Derive expressions for these in term of the transistor parameters and operating currents.

4.a)	With the help of a diagram explain the working of a fixed Bias transistor binary.

b)	The fixed bias binary uses n-p-n silicon transistors with hfe=20. Assume that Vcc=12volts, Vbb=3volts,Rc=1kΩ, R1=10kΩ R2=20kΩ. Find the stable state current and voltages if Vce(sat)=0 and Vbe(sat)=0.

5.a)	Define the terms

- Slope  or  sweep speed error and

- Displacement error

- An exponential sweep results when a capacitor is charged from a supply voltage V through a resistor R. If the peak sweep voltage is Vs , Derive an expression for slope error.

Contd…2

Code No.221003				-2-                                        Set- I

6.a)	Illustrate the terms “Synchronization and ‘frequency division’ of a sweep generator.

b)	A free-running relaxation oscillator has sweep amplitude of 100V and a period of 1msec. Synchronizing pulses are applied to the device such that breakdown voltage is lowered by 50V at each pulse. The synchronizing pulse frequency is 4 kHz. What is the amplitude and frequency of synchronized oscillator waveform?

7.	Design  a  sampling  gate  employed  with  opamp  and  FET. Sketch  the  circuit  and briefly  explain  the  operation  with  necessary  waveforms.

8.a)	Discuss  the  methods  of  controlling  output  current  pulse  width  in  blocking

oscillators.

- Write notes on hysteresis effect in blocking oscillators.

- Discuss the method of improving rise time in blocking oscillator.

$$$

Code No.221003

Set No.

2

II-B.Tech. II-Semester Examination April/May 2003

PULSE AND DIGITAL CIRCUITS

(Common to Electronics and Information Engineering, Electrical and Control Engineering)

Time:3 hours								Max.Marks: 80

Answer any FIVE questions

All questions carry equal marks

---

1.a)	A symmetrical square wave of peak to peak amplitude V and frequency f is applied to a high pass RC circuit.  Find the percentage tilt.

b)	How can this tilt be reduced?

2.a)	Draw the circuit diagram of an emitter-coupled clipping circuit and draw

its transfer characteristic indicating all intercepts, slopes and voltage levels etc.

b)	The input voltage Vi to the clipper shown in fig. 5 is a 10 micro.sec. pulse whose voltage varies between 0 and 10V.

If Rf = 100, Vr = 0.5 V and Rr = , sketch the output wave form V0 and indicate the

time constants of the exponential portions.

3.a)	Explain the behaviour of BJT as a switch in electronic circuits. Give examples

b)	Write a short note on switching times of a transistor.

4.	The circuit parameters of a fixed bias binary are:  Vcc=Vbb=6v, Rc=1kΩ, R1=5kΩ, R2=20kΩ. The n-p-n silicon transistors with hfe(min)=20 are used. Assume that vce(sat)=0.1 and vbe(sat)=0.7v.

a)	Calculate the stable state currents and voltages.

b)	Find the heaviest load the binary can drive.(The minimum resistance from collector to ground)

c)	Find the maximum Icbo such that the circuit function is dependable.

- Design a sweep circuit using UJT with   = 0.5. The sweep amplitude is to be 10V, the sweep duration is 1 msec. and the sweep-speed error is 10%. Select

suitable values of VBB, Vyy, R and C. Estimate the recovery time.

Contd…2

Code No.221003				-2-                                        Set- II

6.a)	Explain about synchronization of a sweep circuits with symmetrical signals.

b)	A UJT sweep operates with a valley voltage of Vv=2V and a peak voltage Vp=12V. A sinusoidal synchronizing voltage of 2V peak is applied between bases. The stand-off ratio is =0.5. If the natural frequency of sweep is 1kHz, over what range of synchronizing signal frequency will the sweep remain in 1:1 synchronization with the synchronizing signal?

7.a)	Sketch  the  circuit  of  FET  shunt  sampling  gate  and  the  related  voltage  waveforms.

b)	Explain  the  operation  of  the  circuit  and  discuss  the  error  sources.

8.	In  an  astable  transistor  (diode  controlled)  blocking  oscillator,  if  L = 5.2mH,

C = 90 pF,  VCC  = 10V, R = 500 Ω, Vr   = 6V, η = 1  and  VBB = 0.5V. Calculate

a)	the  period  and  duty  cycle  of  free  oscillations

b)  	the  peak  voltage  and  currents

c)  	the  current  in  the  magnetizing  inductance  at  the  end  one  cycle.

Neglect  saturation  junction  voltages.

$$$

Code No.221003

Set No.

3

II-B.Tech. II-Semester Examination April/May 2003

PULSE AND DIGITAL CIRCUITS

(Common to Electronics and Information Engineering, Electrical and Control Engineering)

Time:3 hours								Max.Marks: 80

Answer any FIVE questions

All questions carry equal marks

---

1.a)	A 10-Hz symmetrical square wave whose peak-to-peak amplitude is 2V is impressed on a high-pass circuit whose lower 3-dB frequency is 5 Hz.  Calculate and sketch the output waveform.

b)	What is the peak to peak output amplitude of the above wave form.

2.a)	Draw the basic circuit diagram negative peak clamper circuit

and explain its operation.

b)	For the circuit shown in fig. 10, an input voltage Vi linearly varies from 0 to 150V is applied.  Sketch the output voltage V0 to the same time scale.

3.a)	Explain in detail the junction diode switching times.

b)	Give a brief note on piece – wise liner diode characteristics.

4.a)	Explain asymmetrical triggering in a binary and mention its uses.

b)	Design a nonsaturated bistable multivibrator using silicon n-p-n transistors. Assume that Vcc=15v; hfe(min)=60 & hfe(max)=250.If  Ic=1.5mA, find the max & min levels of collector, base & emitter voltages of both transistors.

5.a)	Draw a neat circuit diagram of UJT relaxation oscillator and explain its operation. Can it be used as time-base circuit of a CRO? If so how and if not why?

b)	Draw a Miller time-base circuit with n-p-n transistors and explain its operation. Compare its performance with that of UJT relaxation oscillator.

6.a)	Explain the operation of a sweep circuit as a divider.

b)	The UJT  relaxation oscillator is to be used as a 3:1 divider for pulses which occur at a 2500 Hz rate. The available supply voltage is 30V. The pulses are applied at base B2. Draw the circuit and calculate pulse amplitude.

Contd…2

Code No.221003				-2-                                        Set- III

7.a)	Sketch  the  circuit  of  FET  series  sampling  gate.  Show the voltage waveforms.

b)	Explain  the  operation  of  the  circuit  and  discuss  the  error  sources.

8.	Design  a  free  running  oscillator  using  transistor  with  diode  controlled  circuit  subject to the  following  specifications.  VCC  = 30V, VBB = 1V,  = 20 KHz, the

duty  cycle  is  1/10, the  peak  of  the  pulse  at  the collector  is  10V  and  the  peak

Emitter current is  5 mA.  Find the values for η, Vr,  R   and L. Make  reasonable   assumptions.

$$$

Set No.

4

Code No.221003

II-B.Tech. II-Semester Examination April/May 2003

PULSE AND DIGITAL CIRCUITS

(Common to Electronics and Information Engineering, Electrical and Control Engineering)

Time:3 hours								Max.Marks: 80

Answer any FIVE questions

All questions carry equal marks

---

1.a)	A square wave whose peak-to-peak value is 1V extends + 0.5 V with respect to ground.  The duration of the positive section is 0.1 Sec and of the negative section is 0.2 sec.  If this wave form is impressed on an RC differentiating circuit whose time constant is 0.2 sec; what are the steady state maximum and minimum values of the output waveforms?

b)	Prove that the area under the positive section is equal to the area under the negative section of the output waveform.

2.a)	State and prove clamping-circuit theorem

b)	A square wave input as shown in Fig. 8 is applied to the clamping circuit.  Sketch the steady-state output waveform and derive the necessary expressions.

3.a)	Explain how transistor can be used as a switch in the circuit, under what condition a transistor in said to be ‘OFF’ and ‘ON’ respectively.

b)	A germanium transistor is operated at room temperature in the CE configuration. The supply voltage is 6V, the collector – circuit resistance is 200and the base current is 20 percent higher than the minimum value required to drive the transistor in to saturation. Assume the following transistor parameters :

ICO = -5A, IEO = -2 A hFE = 100 ad rbb1 = 250. Find VBE(sat)and VCE(sat)

4.	A transistor Schmitt trigger uses n-p-n silicion transistors with hfe(min)=30,Vce(sat)=0.1v,VBE(sat)=0.7v Vcc=12v,Rc1=2k   Rc2=1kΩ, R1=20kΩ, R2=100kΩ, Re=500Ω,Rs=2kΩ.

i)	Draw the circuit diagram. Calculate all the steady-state currents & voltages.

ii)	Determine UTP & LTP.

- Find Hystersis voltage.

Contd…2

Code No.221003				-2-                                        Set- 4

iv)	 Sketch the output waveform if a triangular waveform as shown is applied to the circuit.

Vi

0

5.a)	Draw the circuit of a constant current sweep circuit and derive the expression for sweep voltage.

b)	In transistor constant current sweep circuit what happens if temperature is increased? What sort of circuit provides temperature compensation?

6.a)	Draw the block diagram and waveforms for a Divider without phase jitter.

b)	Frequency  division of 6:1 is obtained with an astable multivibrator negative pulses are applied to both bases of the n-p-n  transistors. The OFF time of Q1 in twice that of Q2. Sketch the wave shapes at B1 and  B2 , showing superimposed pulses.

7.a)	What  is  a   sampling  gate?

b)	Explain the operation of series gate using JFET.  Sketch the input and output   Waveforms.

8.	A  common  base  triggered   blocking  oscillator  is  shown  below.  The  transformer

turns  ratio  are  2:1: 1 and  the  collector  magnetizing  inductance  is  10mH.

The transistor  has  an α  =  - hfB = 0.98  and  saturation  junction  voltages  may be neglected.

- Calculate  and  plot  iC ,  iE  and  load  voltage  as  a  function  of  time.

- Calculate pulse width.

+6V

2 :1

0  . :1 .     .

3K

2K

////

$$$